Process 3: SLAM / VIO Navigation

Detailed Implementation Guide

Target Platform: NVIDIA Jetson Orin Nano · C++17 · CMake

# Table of Contents

1. Process Overview & Role in the Stack

2. Thread Architecture Diagram

3. Thread 1 – Visual Frontend

4. Thread 2 – IMU Reader / Pre-integrator

5. Thread 3 – Backend Optimiser (Graph SLAM)

6. Thread 4 – Loop Closure Detector

7. Thread 5 – Map Manager

8. Inter-Thread Communication (Lock-Free Queues)

9. IPC – Receiving Frames from Process 1

10. IPC – Publishing Pose to Process 4 & 5

11. Coordinate Frames & Transformations

12. Available SLAM/VIO Libraries & Algorithms

13. Sensor Configuration & Calibration

14. Performance Tuning for Jetson Orin Nano

15. C++ Concepts Reference

16. systemd Service Configuration

17. CMakeLists.txt

18. Implementation Roadmap

# 1. Process Overview & Role in the Stack

Process 3 (slam_vio_nav) is the spatial-intelligence core of the companion computer. It continuously estimates the drone's 6-DOF pose (position + orientation) in a world-fixed reference frame by fusing visual feature tracks from one or more cameras with inertial measurements from the IMU. The resulting pose is consumed by Process 4 (Mission Planner) for path planning and by Process 5 (Comms) for telemetry downlink to the GCS.

Unlike a pure VIO system that only performs odometry (open-loop integration), this process also maintains a sparse 3-D map, detects loop closures, and runs global graph optimisation to correct accumulated drift – making it a full Visual-Inertial SLAM pipeline.

## Responsibilities

Receive rectified/undistorted greyscale frames from Process 1 via shared memory

Read raw IMU data (accelerometer + gyroscope) at ≥200 Hz

Track visual features (ORB / FAST / SuperPoint) across consecutive frames

Pre-integrate IMU measurements between keyframes

Maintain a sliding-window or full pose graph and solve via nonlinear optimisation

Detect loop closures using bag-of-words or learned descriptors

Publish T_world_body (SE(3) pose) plus covariance to downstream processes

Persist and reload maps for re-localisation on revisit

## Input / Output Summary

| Direction | Data | Source / Sink | Transport |
| --- | --- | --- | --- |
| IN | Greyscale frames (640×480 @ 30 Hz) | Process 1 (Video/Capture) | Shared memory ring buffer |
| IN | IMU accel + gyro (≥200 Hz) | SPI/I²C driver or ROS2 topic | Direct read / SPSC queue |
| IN | Barometer altitude (optional) | I²C driver | SPSC queue |
| OUT | T_world_body (SE(3) + covariance) | Process 4 (Mission Planner) | Shared memory slot |
| OUT | T_world_body (SE(3)) | Process 5 (Comms → GCS) | Shared memory slot |
| OUT | Sparse 3-D map points (optional) | Process 4 for obstacle checks | Shared memory / file |
| OUT | Health / heartbeat | Process 7 (System Monitor) | sd_notify + shared flag |

# 2. Thread Architecture Diagram

Process 3 is decomposed into five dedicated threads, each pinned to a specific CPU core to maximise cache locality and minimise scheduling jitter. Threads communicate exclusively through lock-free SPSC queues.

┌─────────────────────────────────────────────────────────────────────┐

│                    Process 3: slam_vio_nav                         │

│                                                                     │

│  ┌──────────────┐    SPSC     ┌────────────────────┐               │

│  │  Thread 1    │───────────▶│   Thread 3          │               │

│  │  Visual      │  tracked   │   Backend           │               │

│  │  Frontend    │  features  │   Optimiser         │               │

│  └──────┬───────┘            │   (Graph SLAM)      │               │

│         │                    └─────────┬────────────┘               │

│         │ keyframe                     │ optimised                  │

│         │ flag                         │ poses                     │

│         ▼                              ▼                           │

│  ┌──────────────┐    SPSC     ┌────────────────────┐               │

│  │  Thread 2    │───────────▶│   Thread 5          │               │

│  │  IMU Reader  │  pre-int   │   Map Manager       │──▶ Pose OUT  │

│  │  / Pre-int   │  factors   │   (publish + persist)│              │

│  └──────────────┘            └─────────┬────────────┘               │

│                                        ▲                           │

│                              ┌─────────┴────────────┐               │

│                              │   Thread 4           │               │

│                              │   Loop Closure        │               │

│                              │   Detector            │               │

│                              └──────────────────────┘               │

│                                                                     │

│  ◄── Frames IN (shm from Process 1)                                │

│  ◄── IMU IN (SPI/I²C)                                              │

│  ──▶ Pose OUT (shm to Process 4 & 5)                               │

└─────────────────────────────────────────────────────────────────────┘

| Thread | Name | Core Affinity | Priority | Wake Source |
| --- | --- | --- | --- | --- |
| T1 | Visual Frontend | Core 2 | SCHED_FIFO 60 | Frame-ready semaphore (shm) |
| T2 | IMU Reader / Pre-integrator | Core 3 | SCHED_FIFO 70 | Timer (5 ms) / SPI IRQ |
| T3 | Backend Optimiser | Core 4 | SCHED_FIFO 50 | Keyframe insertion event |
| T4 | Loop Closure Detector | Core 5 | SCHED_OTHER (nice -5) | Keyframe insertion event |
| T5 | Map Manager | Core 2 (shared) | SCHED_FIFO 45 | Optimised graph event |

# 3. Thread 1 – Visual Frontend

## 3.1 Purpose

The Visual Frontend is the entry point for all camera data. It receives greyscale frames from the shared-memory ring buffer written by Process 1, detects feature keypoints, computes descriptors, and matches them against the previous frame to produce 2-D–2-D correspondences. When a keyframe criterion is met, it packages the tracked features and pushes them into the Backend Optimiser queue.

## 3.2 Pipeline Stages

1. Acquire frame pointer (zero-copy) from shared-memory ring buffer

2. (Optional) Apply CLAHE histogram equalisation for lighting robustness

3. Detect keypoints (ORB / FAST / Shi-Tomasi / SuperPoint)

4. Compute descriptors (ORB binary / SuperPoint float)

5. Match against previous-frame descriptors (BFMatcher or optical flow)

6. Geometric verification – RANSAC essential-matrix to reject outliers

7. Keyframe decision (parallax threshold, tracked-feature ratio, time)

8. If keyframe → push tracked features + pose prior to Backend queue

9. Always push latest relative-motion estimate to Map Manager for fast pose

## 3.3 Feature Detection Strategy

We partition the image into a grid (e.g., 8×6 cells) and enforce a minimum feature count per cell to ensure spatial distribution. This prevents features clustering on a single textured object while leaving other image regions untracked.

💡 C++ Concept #1: Strategy Pattern for Feature Detectors

Different environments (indoor/outdoor, textured/textureless) benefit from different feature detectors. The Strategy pattern lets you swap detectors at runtime without modifying the frontend pipeline. Each detector implements a common interface (IFeatureDetector), and the frontend holds a std::unique_ptr<IFeatureDetector> that is injected at construction. This is the same pattern used in Process 2 for DNN backends.

// ── Strategy interface ──

class IFeatureDetector {

public:

virtual ~IFeatureDetector() = default;

struct Result {

std::vector<cv::KeyPoint> keypoints;

cv::Mat descriptors;           // CV_8U for ORB, CV_32F for SuperPoint

};

virtual Result detect(const cv::Mat& grey_frame) = 0;

virtual std::string name() const = 0;

};

// ── ORB concrete strategy ──

class OrbDetector final : public IFeatureDetector {

cv::Ptr<cv::ORB> orb_;

int grid_rows_ = 6, grid_cols_ = 8;

public:

explicit OrbDetector(int max_features = 1500)

: orb_(cv::ORB::create(max_features)) {}

Result detect(const cv::Mat& grey) override {

Result res;

int cell_h = grey.rows / grid_rows_;

int cell_w = grey.cols / grid_cols_;

int per_cell = orb_->getMaxFeatures() / (grid_rows_ * grid_cols_);

for (int r = 0; r < grid_rows_; ++r) {

for (int c = 0; c < grid_cols_; ++c) {

cv::Rect roi(c * cell_w, r * cell_h, cell_w, cell_h);

std::vector<cv::KeyPoint> kps;

cv::Mat desc;

orb_->detectAndCompute(grey(roi), cv::noArray(), kps, desc);

// Offset keypoints back to full-image coordinates

for (auto& kp : kps) {

kp.pt.x += roi.x;

kp.pt.y += roi.y;

}

// Keep top-N per cell

if ((int)kps.size() > per_cell) {

std::partial_sort(kps.begin(), kps.begin() + per_cell,

kps.end(), [](auto& a, auto& b) {

return a.response > b.response;

});

kps.resize(per_cell);

desc = desc.rowRange(0, per_cell).clone();

}

res.keypoints.insert(res.keypoints.end(), kps.begin(), kps.end());

if (!desc.empty())

res.descriptors.push_back(desc);

}

}

return res;

}

std::string name() const override { return "ORB-Grid"; }

};

// ── Optical-flow strategy (KLT – no descriptors) ──

class KltTracker final : public IFeatureDetector {

cv::Mat prev_grey_;

std::vector<cv::Point2f> prev_pts_;

public:

Result detect(const cv::Mat& grey) override;  // uses calcOpticalFlowPyrLK

std::string name() const override { return "KLT"; }

};

## 3.4 Keyframe Decision Logic

A new keyframe is created when any of the following conditions is met:

Median parallax between current frame and last keyframe > 15 pixels (configurable)

Number of successfully tracked features drops below 60% of initial count

Time since last keyframe exceeds 0.5 s (ensures periodic updates for slow motion)

The backend optimiser queue is empty (opportunistic keyframe insertion)

struct KeyframePolicy {

double min_parallax_px    = 15.0;

double min_tracked_ratio  = 0.6;

double max_time_sec       = 0.5;

bool should_create_keyframe(

double median_parallax,

int tracked_count,

int initial_count,

double dt_since_last_kf,

bool backend_idle) const

{

if (median_parallax > min_parallax_px) return true;

double ratio = static_cast<double>(tracked_count) / initial_count;

if (ratio < min_tracked_ratio) return true;

if (dt_since_last_kf > max_time_sec) return true;

if (backend_idle && median_parallax > min_parallax_px * 0.5) return true;

return false;

}

};

## 3.5 Visual Frontend Thread Loop

void VisualFrontend::run() {

while (!stop_flag_.load(std::memory_order_relaxed)) {

// 1. Wait for new frame from shared memory

FrameHandle frame = shm_reader_.wait_and_acquire();  // blocks on semaphore

if (!frame.valid()) continue;

cv::Mat grey(frame.height(), frame.width(), CV_8UC1,

frame.data());  // zero-copy wrap

// 2. Detect & match features

auto result = detector_->detect(grey);

auto matches = matcher_.match(prev_result_, result);

// 3. Geometric verification (Essential matrix via RANSAC)

auto [inlier_matches, E] = geometric_verify(matches, camera_intrinsics_);

// 4. Compute relative pose R|t from Essential matrix

Sophus::SE3d T_cur_prev = decompose_essential(E, inlier_matches, camera_intrinsics_);

// 5. Accumulate relative motion since last keyframe

T_cur_kf_ = T_cur_prev * T_cur_kf_;

double parallax = compute_median_parallax(inlier_matches);

double dt = frame.timestamp() - last_kf_time_;

// 6. Keyframe decision

if (kf_policy_.should_create_keyframe(

parallax, inlier_matches.size(),

prev_result_.keypoints.size(), dt,

backend_queue_.was_empty())) {

KeyframeData kf{frame.timestamp(), grey.clone(), result,

inlier_matches, T_cur_kf_};

backend_queue_.push(std::move(kf));     // SPSC → Thread 3

T_cur_kf_ = Sophus::SE3d{};             // reset

last_kf_time_ = frame.timestamp();

}

// 7. Always publish fast odometry pose

pose_queue_.push(PoseStamped{frame.timestamp(), T_world_cur_});

prev_result_ = std::move(result);

shm_reader_.release(frame);

}

}

💡 C++ Concept #2: Sophus SE(3) Lie Group for Rigid Transforms

Sophus is a header-only C++ library that provides Lie group types (SO3, SE3) built on Eigen. Unlike using raw 4×4 matrices, Sophus::SE3d ensures that transformations always remain on the SE(3) manifold (valid rotation + translation). Composing poses is simply multiplication: T_world_cur = T_world_kf * T_kf_cur. Sophus also provides log/exp maps needed by optimisers (e.g., Ceres) to parameterise incremental updates in the tangent space.

#include <sophus/se3.hpp>

// Compose two transforms

Sophus::SE3d T_world_body = T_world_ref * T_ref_body;

// Inverse

Sophus::SE3d T_body_world = T_world_body.inverse();

// Extract translation & rotation

Eigen::Vector3d position = T_world_body.translation();

Eigen::Quaterniond orientation(T_world_body.rotationMatrix());

// Tangent-space perturbation (6-vector: [tx,ty,tz, rx,ry,rz])

Eigen::Matrix<double,6,1> delta;

delta << 0.01, 0.0, 0.0, 0.0, 0.0, 0.001;

Sophus::SE3d T_perturbed = Sophus::SE3d::exp(delta) * T_world_body;

# 4. Thread 2 – IMU Reader / Pre-integrator

## 4.1 Purpose

The IMU Reader thread acquires accelerometer and gyroscope measurements at high rate (typically 200–400 Hz) from the on-board IMU (e.g., BMI088, ICM-42688-P) via SPI or I²C. Between consecutive keyframes it pre-integrates these measurements into a single relative-motion factor that the backend optimiser can incorporate without re-integrating from scratch when the bias estimates change.

## 4.2 IMU Measurement Model

Raw IMU readings contain bias (slowly drifting) and noise:

// Measurement model

// ω_meas = ω_true + b_g + n_g        (gyroscope)

// a_meas = R_bw * (a_true - g) + b_a + n_a   (accelerometer)

//

// Where:

//   b_g, b_a = slowly varying biases (estimated by backend)

//   n_g, n_a = white noise

//   g        = gravity in world frame [0, 0, -9.81]

//   R_bw     = rotation from world to body

## 4.3 Pre-integration Theory

Pre-integration (Forster et al., 2017) accumulates N IMU measurements between two keyframes i and j into a single relative rotation ΔR_ij, velocity Δv_ij, and position Δp_ij. These "pre-integrated measurements" are functions of the bias estimates. When the optimiser updates the biases, instead of re-integrating all N samples, we apply a first-order correction using the pre-computed Jacobians ∂ΔR/∂b_g, ∂Δv/∂b_a, etc.

💡 C++ Concept #3: CRTP (Curiously Recurring Template Pattern) for Static Polymorphism

The IMU pre-integrator uses CRTP to provide a compile-time polymorphic interface for different IMU noise models (continuous vs. discrete-time) without virtual dispatch overhead. In a 200 Hz loop, eliminating vtable lookups matters. CRTP works by having the derived class pass itself as a template parameter to the base class, which then static_casts 'this' to the derived type to call the implementation.

template <typename Derived>

class ImuPreintegratorBase {

public:

void integrate(const ImuSample& sample) {

// Common bookkeeping

dt_total_ += sample.dt;

n_samples_++;

// Delegate noise model to derived (no virtual call!)

static_cast<Derived*>(this)->propagate_impl(sample);

}

PreintegratedImuFactor get_factor() const {

return {delta_R_, delta_v_, delta_p_,

jacobian_bias_gyro_, jacobian_bias_accel_,

covariance_, dt_total_};

}

protected:

Sophus::SO3d delta_R_ = Sophus::SO3d{};

Eigen::Vector3d delta_v_ = Eigen::Vector3d::Zero();

Eigen::Vector3d delta_p_ = Eigen::Vector3d::Zero();

Eigen::Matrix<double,9,6> jacobian_bias_gyro_;

Eigen::Matrix<double,9,6> jacobian_bias_accel_;

Eigen::Matrix<double,9,9> covariance_ = Eigen::Matrix<double,9,9>::Zero();

double dt_total_ = 0.0;

int n_samples_ = 0;

};

class ContinuousNoiseModel

: public ImuPreintegratorBase<ContinuousNoiseModel> {

friend class ImuPreintegratorBase<ContinuousNoiseModel>;

void propagate_impl(const ImuSample& s) {

// Continuous-time noise propagation

Eigen::Vector3d a_corrected = s.accel - bias_accel_;

Eigen::Vector3d w_corrected = s.gyro  - bias_gyro_;

Sophus::SO3d dR = Sophus::SO3d::exp(w_corrected * s.dt);

delta_p_ += delta_v_ * s.dt + 0.5 * delta_R_ * a_corrected * s.dt * s.dt;

delta_v_ += delta_R_ * a_corrected * s.dt;

delta_R_ = delta_R_ * dR;

// Update Jacobians & covariance (omitted for brevity)

}

Eigen::Vector3d bias_accel_, bias_gyro_;

};

## 4.4 Thread Loop

void ImuReaderThread::run() {

imu_driver_.configure(ImuConfig{

.sample_rate_hz = 200,

.accel_range    = AccelRange::PM_16G,

.gyro_range     = GyroRange::PM_2000DPS,

.dlpf_bandwidth = 92  // Hz

});

auto preint = std::make_unique<ContinuousNoiseModel>(

initial_bias_accel_, initial_bias_gyro_, noise_params_);

while (!stop_flag_.load(std::memory_order_relaxed)) {

ImuSample sample = imu_driver_.read_blocking();  // blocks ~5 ms

preint->integrate(sample);

// If the Visual Frontend signals a new keyframe, ship the factor

if (keyframe_event_.try_wait()) {

auto factor = preint->get_factor();

imu_factor_queue_.push(std::move(factor));   // SPSC → Thread 3

// Reset pre-integrator with latest bias from optimiser

auto latest_bias = bias_update_.load();      // atomic Bias struct

preint = std::make_unique<ContinuousNoiseModel>(

latest_bias.accel, latest_bias.gyro, noise_params_);

}

}

}

## 4.5 Hardware Interface (SPI)

On Jetson Orin Nano, the IMU is typically connected via SPI for maximum throughput (up to 10 MHz). We use Linux spidev with a memory-mapped DMA transfer for jitter-free acquisition.

class SpiImuDriver {

int fd_;

public:

explicit SpiImuDriver(const std::string& device = "/dev/spidev0.0",

uint32_t speed_hz = 8'000'000)

{

fd_ = ::open(device.c_str(), O_RDWR);

if (fd_ < 0) throw std::system_error(errno, std::system_category(), "SPI open");

::ioctl(fd_, SPI_IOC_WR_MAX_SPEED_HZ, &speed_hz);

uint8_t mode = SPI_MODE_3;

::ioctl(fd_, SPI_IOC_WR_MODE, &mode);

}

ImuSample read_blocking() {

uint8_t tx[14] = {0x80 | REG_ACCEL_XOUT_H};  // read burst

uint8_t rx[14] = {};

spi_ioc_transfer xfer{};

xfer.tx_buf = reinterpret_cast<__u64>(tx);

xfer.rx_buf = reinterpret_cast<__u64>(rx);

xfer.len = sizeof(tx);

xfer.speed_hz = 8'000'000;

::ioctl(fd_, SPI_IOC_MESSAGE(1), &xfer);

ImuSample s;

s.timestamp = steady_clock_now();

s.accel = parse_accel(&rx[1]);   // 3 × int16 → m/s²

s.gyro  = parse_gyro(&rx[7]);    // 3 × int16 → rad/s

return s;

}

~SpiImuDriver() { if (fd_ >= 0) ::close(fd_); }

};

💡 C++ Concept #4: RAII for Hardware Resources (File Descriptors)

The SPI driver above opens a file descriptor in the constructor and closes it in the destructor – the classic RAII (Resource Acquisition Is Initialization) pattern. This guarantees the FD is released even if an exception is thrown. For extra safety, delete copy operations and implement move semantics so the driver object can be transferred but never duplicated.

class SpiImuDriver {

int fd_ = -1;

public:

SpiImuDriver(const std::string& dev, uint32_t speed);

~SpiImuDriver() { if (fd_ >= 0) ::close(fd_); }

// Non-copyable

SpiImuDriver(const SpiImuDriver&) = delete;

SpiImuDriver& operator=(const SpiImuDriver&) = delete;

// Movable

SpiImuDriver(SpiImuDriver&& o) noexcept : fd_(std::exchange(o.fd_, -1)) {}

SpiImuDriver& operator=(SpiImuDriver&& o) noexcept {

if (this != &o) {

if (fd_ >= 0) ::close(fd_);

fd_ = std::exchange(o.fd_, -1);

}

return *this;

}

};

# 5. Thread 3 – Backend Optimiser (Graph SLAM)

## 5.1 Purpose

The Backend Optimiser maintains a factor graph where nodes represent keyframe poses and IMU biases, and edges represent visual re-projection factors, IMU pre-integration factors, and (when available) loop-closure factors. It solves the graph using iterative nonlinear least-squares (Gauss-Newton or Levenberg-Marquardt) to produce globally consistent poses.

## 5.2 Factor Graph Structure

Each keyframe i adds the following to the graph:

Pose node x_i ∈ SE(3) – the 6-DOF pose of the drone

Bias node b_i = [b_a, b_g] – IMU biases at keyframe i

Visual factor – re-projection error of tracked 3-D landmarks into keyframe i

IMU factor – connects x_i, v_i, b_i to x_{i+1}, v_{i+1}, b_{i+1}

Bias random-walk factor – soft constraint between b_i and b_{i+1}

(Optional) Loop-closure factor from Thread 4

## 5.3 Optimisation Libraries

| Library | Approach | Pros | Cons | License |
| --- | --- | --- | --- | --- |
| GTSAM | iSAM2 (incremental) | Best incremental solver, Bayes tree | Heavier dependency | BSD |
| Ceres Solver | Batch LM / Dogleg | Extremely flexible, Google-maintained | Batch-only (no incremental) | Apache-2.0 |
| g2o | Sparse LM | Lightweight, widely used in SLAM | API less ergonomic | BSD |

## 5.4 Using GTSAM iSAM2

GTSAM's iSAM2 performs incremental smoothing and mapping. Instead of re-solving the entire graph on each keyframe, it updates only the affected part of the Bayes tree – reducing per-keyframe optimisation time from O(n²) to near O(1) amortised.

#include <gtsam/navigation/CombinedImuFactor.h>

#include <gtsam/slam/SmartProjectionPoseFactor.h>

#include <gtsam/nonlinear/ISAM2.h>

#include <gtsam/nonlinear/Values.h>

#include <gtsam/nonlinear/NonlinearFactorGraph.h>

class BackendOptimiser {

gtsam::ISAM2 isam2_;

gtsam::NonlinearFactorGraph new_factors_;

gtsam::Values new_values_;

uint64_t kf_id_ = 0;

public:

BackendOptimiser() {

gtsam::ISAM2Params params;

params.relinearizeThreshold = 0.01;

params.relinearizeSkip = 1;

params.enableDetailedResults = false;

isam2_ = gtsam::ISAM2(params);

}

void add_keyframe(const KeyframeData& kf, const PreintegratedImuFactor& imu_factor) {

uint64_t prev = kf_id_;

kf_id_++;

// Pose initial estimate

gtsam::Pose3 pose_init = sophus_to_gtsam(kf.T_world_kf);

new_values_.insert(X(kf_id_), pose_init);

new_values_.insert(V(kf_id_), kf.velocity);

new_values_.insert(B(kf_id_), gtsam::imuBias::ConstantBias(

kf.bias_accel, kf.bias_gyro));

// IMU factor between prev and current keyframe

auto imu = boost::make_shared<gtsam::CombinedImuFactor>(

X(prev), V(prev), X(kf_id_), V(kf_id_),

B(prev), B(kf_id_), imu_factor.gtsam_pim);

new_factors_.add(imu);

// Visual re-projection factors (SmartFactor per landmark)

for (auto& [lm_id, obs] : kf.observations) {

smart_factors_[lm_id]->add(obs.uv, X(kf_id_));

}

// Incremental update

auto result = isam2_.update(new_factors_, new_values_);

new_factors_.resize(0);

new_values_.clear();

// Extract optimised pose

gtsam::Values estimate = isam2_.calculateEstimate();

publish_optimised_poses(estimate);

}

};

💡 C++ Concept #5: Type-Safe Graph Keys with Enum Symbols

GTSAM uses Symbol keys like X(i), V(i), B(i) to identify pose, velocity, and bias nodes in the graph. This pattern prevents accidentally connecting a pose factor to a bias node. In your own code, you can achieve similar type-safety using strong typedefs or enum class indices.

// GTSAM's approach – character + index:

gtsam::Key pose_key  = gtsam::Symbol('x', 42);  // X(42)

gtsam::Key vel_key   = gtsam::Symbol('v', 42);  // V(42)

gtsam::Key bias_key  = gtsam::Symbol('b', 42);  // B(42)

// Custom approach using strong typedefs:

enum class NodeType : char { Pose = 'x', Velocity = 'v', Bias = 'b' };

struct GraphKey {

NodeType type;

uint64_t index;

bool operator==(const GraphKey& o) const = default;

};

struct GraphKeyHash {

size_t operator()(const GraphKey& k) const {

return std::hash<uint64_t>{}(

(static_cast<uint64_t>(k.type) << 56) | k.index);

}

};

## 5.5 Thread Loop

void BackendOptimiser::run() {

while (!stop_flag_.load(std::memory_order_relaxed)) {

// Block until a keyframe arrives

KeyframeData kf;

if (!keyframe_queue_.pop_wait(kf, std::chrono::milliseconds(100)))

continue;

// Grab the corresponding IMU pre-integration factor

PreintegratedImuFactor imu_factor;

imu_factor_queue_.pop_wait(imu_factor, std::chrono::milliseconds(50));

add_keyframe(kf, imu_factor);

// Notify loop closure thread (new keyframe to check)

loop_closure_event_.signal();

// Update IMU thread with latest bias estimates

auto bias = isam2_.calculateEstimate<gtsam::imuBias::ConstantBias>(

B(kf_id_));

bias_update_.store({gtsam_to_eigen(bias.accelerometer()),

gtsam_to_eigen(bias.gyroscope())});

sd_notify(0, "WATCHDOG=1");  // heartbeat

}

}

# 6. Thread 4 – Loop Closure Detector

## 6.1 Purpose

Drift is inevitable in any odometry system. The Loop Closure thread detects when the drone revisits a previously seen location and provides a relative-pose constraint to the backend optimiser, enabling it to correct the accumulated drift across the entire trajectory.

## 6.2 Bag-of-Words Approach (DBoW2 / FBoW)

The classic approach converts each keyframe's feature descriptors into a compact bag-of-words (BoW) vector using a pre-trained visual vocabulary. Candidate loop closures are found by comparing BoW vectors (L1 score). Candidates are then verified geometrically (PnP RANSAC).

#include <DBoW2/DBoW2.h>

class LoopClosureDetector {

OrbVocabulary vocab_;             // Pre-trained ORB vocabulary

OrbDatabase db_;                  // Inverse-index database

std::map<DBoW2::EntryId, uint64_t> entry_to_kf_;

public:

explicit LoopClosureDetector(const std::string& vocab_path)

: vocab_(vocab_path), db_(vocab_, false, 0) {}

std::optional<LoopCandidate> detect(uint64_t kf_id,

const cv::Mat& descriptors)

{

// Convert descriptors to vector of cv::Mat rows

std::vector<cv::Mat> desc_vec;

for (int i = 0; i < descriptors.rows; ++i)

desc_vec.push_back(descriptors.row(i));

// Query database

DBoW2::QueryResults results;

db_.query(desc_vec, results, /*max_results=*/5);

// Add current keyframe to database

auto entry_id = db_.add(desc_vec);

entry_to_kf_[entry_id] = kf_id;

// Filter: ignore recent keyframes (within 30 KFs)

for (auto& r : results) {

uint64_t candidate_kf = entry_to_kf_[r.Id];

if (kf_id - candidate_kf < 30) continue;

if (r.Score < 0.03) continue;

return LoopCandidate{kf_id, candidate_kf, r.Score};

}

return std::nullopt;

}

};

💡 C++ Concept #6: std::optional for Nullable Return Values

The detect() method above returns std::optional<LoopCandidate>. This is cleaner than returning a bool + out-parameter, or using a sentinel value. The caller checks with has_value() or uses value_or(). std::optional avoids heap allocation (unlike std::unique_ptr) and communicates intent clearly: "this function might not produce a result."

auto candidate = loop_detector.detect(kf_id, descriptors);

if (candidate) {

// Use candidate->query_kf, candidate->match_kf, candidate->score

spdlog::info("Loop closure: KF {} → KF {} (score {:.3f})",

candidate->query_kf, candidate->match_kf, candidate->score);

backend.add_loop_closure(*candidate);

}

// No raw pointers, no sentinel values, no exceptions for control flow.

## 6.3 Geometric Verification

After a BoW candidate is found, we perform geometric verification:

1. Match features between query and candidate keyframes (BFMatcher + ratio test)

2. Compute relative pose via PnP-RANSAC (using 3-D map points of candidate)

3. If inlier count ≥ 30 and reprojection error < 2.0 px → accept loop closure

4. Construct loop-closure factor (SE(3) relative pose + covariance)

5. Push factor to backend optimiser queue → triggers global graph re-optimisation

## 6.4 Thread Loop

void LoopClosureThread::run() {

while (!stop_flag_.load(std::memory_order_relaxed)) {

loop_closure_event_.wait();  // signaled by backend on new keyframe

auto [kf_id, descriptors, keypoints, map_points] =

map_manager_.get_latest_keyframe_data();

auto candidate = detector_.detect(kf_id, descriptors);

if (!candidate) continue;

// Geometric verification

auto [match_kf_data] = map_manager_.get_keyframe_data(candidate->match_kf);

auto verification = verify_loop(keypoints, map_points,

match_kf_data, camera_intrinsics_);

if (!verification) continue;

// Package and send to backend

LoopClosureFactor factor{

.from_kf = candidate->query_kf,

.to_kf   = candidate->match_kf,

.T_from_to = verification->relative_pose,

.information = verification->information_matrix

};

loop_factor_queue_.push(std::move(factor));

spdlog::info("Loop closure accepted: KF{} → KF{}",

factor.from_kf, factor.to_kf);

}

}

# 7. Thread 5 – Map Manager

## 7.1 Purpose

The Map Manager is the single source of truth for the current SLAM state. It receives optimised poses from the backend, maintains the sparse 3-D map of landmarks, publishes the latest T_world_body to downstream processes (Mission Planner, Comms), and handles map serialisation (save/load) for persistent re-localisation.

## 7.2 Responsibilities

Receive optimised keyframe poses from backend optimiser (SPSC queue)

Maintain a spatial index (kd-tree) of 3-D map points

Publish latest T_world_body + covariance to shared-memory slots

Cull redundant keyframes and map points (memory management)

Serialise map to disk on landing / on command

Deserialise and re-localise on startup if map file exists

## 7.3 Pose Publishing (Zero-Copy Shared Memory)

// Shared-memory layout for pose publication

struct alignas(64) ShmPoseSlot {

std::atomic<uint64_t> sequence;     // monotonic, odd = writing

double timestamp;

double translation[3];              // x, y, z in world frame

double quaternion[4];               // w, x, y, z

double covariance[36];              // 6×6 upper-triangle

uint8_t padding[64 - (8+8+24+32+288) % 64];

};

class PosePublisher {

ShmPoseSlot* slot_;  // mmap'd shared memory

public:

void publish(double ts, const Sophus::SE3d& T, const Eigen::Matrix6d& cov) {

uint64_t seq = slot_->sequence.load(std::memory_order_relaxed);

slot_->sequence.store(seq + 1, std::memory_order_release);  // odd = writing

slot_->timestamp = ts;

Eigen::Map<Eigen::Vector3d>(slot_->translation) = T.translation();

Eigen::Quaterniond q(T.rotationMatrix());

slot_->quaternion[0] = q.w();

slot_->quaternion[1] = q.x();

slot_->quaternion[2] = q.y();

slot_->quaternion[3] = q.z();

Eigen::Map<Eigen::Matrix<double,6,6>>(slot_->covariance) = cov;

slot_->sequence.store(seq + 2, std::memory_order_release);  // even = valid

}

};

💡 C++ Concept #7: SeqLock Pattern for Single-Writer / Multi-Reader Shared Memory

The ShmPoseSlot uses a SeqLock – a sequence number that is odd during writes and even when stable. Readers spin-read the sequence, copy the data, then verify the sequence hasn't changed. This is lock-free, wait-free for writers, and allows multiple readers (Process 4 and Process 5) without any mutex. It works because there is exactly one writer (Map Manager thread).

// Reader side (in Process 4 or Process 5):

bool read_pose(const ShmPoseSlot* slot, PoseStamped& out) {

for (int attempt = 0; attempt < 3; ++attempt) {

uint64_t seq1 = slot->sequence.load(std::memory_order_acquire);

if (seq1 & 1) continue;  // writer is mid-update, retry

// Copy data

out.timestamp = slot->timestamp;

out.position  = Eigen::Map<const Eigen::Vector3d>(slot->translation);

out.orientation = Eigen::Quaterniond(

slot->quaternion[0], slot->quaternion[1],

slot->quaternion[2], slot->quaternion[3]);

std::atomic_thread_fence(std::memory_order_acquire);

uint64_t seq2 = slot->sequence.load(std::memory_order_relaxed);

if (seq1 == seq2) return true;  // consistent read

}

return false;  // failed after retries (write was in progress)

}

## 7.4 Map Serialisation

For persistent re-localisation, the map must be saved to disk. We use a compact binary format (or optionally cereal/protobuf) containing keyframe poses, 3-D landmarks, BoW vectors, and vocabulary reference.

#include <cereal/archives/binary.hpp>

#include <cereal/types/vector.hpp>

#include <cereal/types/map.hpp>

struct SerializableMap {

std::vector<KeyframePose> keyframes;

std::vector<Landmark3D> landmarks;

std::vector<BowVector> bow_vectors;

std::string vocabulary_hash;  // to verify vocabulary compatibility

template <class Archive>

void serialize(Archive& ar) {

ar(keyframes, landmarks, bow_vectors, vocabulary_hash);

}

};

void MapManager::save_map(const std::string& path) {

SerializableMap smap = build_serializable();

std::ofstream ofs(path, std::ios::binary);

cereal::BinaryOutputArchive archive(ofs);

archive(smap);

spdlog::info("Map saved: {} KFs, {} landmarks → {}",

smap.keyframes.size(), smap.landmarks.size(), path);

}

## 7.5 Map Point Culling

To bound memory usage, the Map Manager periodically culls:

Map points observed by fewer than 3 keyframes

Map points with high reprojection error (> 5 px average)

Keyframes that are too close to neighbours (< 5 cm translation)

Keyframes older than a sliding window (e.g., 200 KFs) unless they have loop-closure edges

# 8. Inter-Thread Communication (Lock-Free Queues)

All inter-thread data flow uses lock-free SPSC (Single-Producer Single-Consumer) bounded ring buffers. This is the same SPSC design used in Process 1 and Process 2. Each queue is templated on the message type and has a compile-time capacity.

## 8.1 Queue Inventory

| Queue | Producer | Consumer | Message Type | Capacity |
| --- | --- | --- | --- | --- |
| keyframe_queue | T1 Visual Frontend | T3 Backend Optimiser | KeyframeData | 8 |
| imu_factor_queue | T2 IMU Reader | T3 Backend Optimiser | PreintegratedImuFactor | 8 |
| pose_queue | T1 Visual Frontend | T5 Map Manager | PoseStamped | 16 |
| optimised_queue | T3 Backend Optimiser | T5 Map Manager | OptimisedPoses | 4 |
| loop_factor_queue | T4 Loop Closure | T3 Backend Optimiser | LoopClosureFactor | 4 |

💡 C++ Concept #8: Lock-Free SPSC Ring Buffer (Recap)

This is the same SPSC queue used across the entire drone stack. The key invariant is: only one thread writes head_, only one thread reads tail_. std::memory_order_acquire/release fences ensure correct visibility across cores without any mutex. The buffer is cache-line padded to prevent false sharing between producer and consumer cache lines.

template <typename T, size_t Capacity>

class SpscQueue {

static_assert((Capacity & (Capacity - 1)) == 0, "Capacity must be power of 2");

struct alignas(64) {

std::atomic<size_t> head{0};

};

struct alignas(64) {

std::atomic<size_t> tail{0};

};

std::array<T, Capacity> buffer_;

public:

bool push(T&& item) {

size_t h = head.load(std::memory_order_relaxed);

size_t next = (h + 1) & (Capacity - 1);

if (next == tail.load(std::memory_order_acquire)) return false; // full

buffer_[h] = std::move(item);

head.store(next, std::memory_order_release);

return true;

}

bool pop(T& item) {

size_t t = tail.load(std::memory_order_relaxed);

if (t == head.load(std::memory_order_acquire)) return false; // empty

item = std::move(buffer_[t]);

tail.store((t + 1) & (Capacity - 1), std::memory_order_release);

return true;

}

};

# 9. IPC – Receiving Frames from Process 1

Process 3 reads greyscale camera frames that were written by Process 1 into a POSIX shared-memory ring buffer. The mechanism uses shm_open + mmap for zero-copy access, and a named POSIX semaphore for signalling.

## 9.1 Shared Memory Reader

class ShmFrameReader {

int shm_fd_;

void* base_ptr_;

size_t ring_size_;

sem_t* sem_;

size_t read_idx_ = 0;

struct FrameHeader {

uint64_t sequence;

uint64_t timestamp_ns;

uint32_t width, height, stride;

uint32_t format;  // GREY8

};

public:

ShmFrameReader(const std::string& shm_name, const std::string& sem_name,

size_t num_slots)

{

shm_fd_ = shm_open(shm_name.c_str(), O_RDONLY, 0);

if (shm_fd_ < 0) throw std::system_error(errno, std::system_category());

struct stat sb;

fstat(shm_fd_, &sb);

ring_size_ = sb.st_size;

base_ptr_ = mmap(nullptr, ring_size_, PROT_READ, MAP_SHARED, shm_fd_, 0);

sem_ = sem_open(sem_name.c_str(), 0);

}

FrameHandle wait_and_acquire() {

sem_wait(sem_);  // block until new frame

auto* header = reinterpret_cast<const FrameHeader*>(

static_cast<char*>(base_ptr_) + read_idx_ * slot_size_);

read_idx_ = (read_idx_ + 1) % num_slots_;

return FrameHandle{header};

}

~ShmFrameReader() {

munmap(base_ptr_, ring_size_);

close(shm_fd_);

sem_close(sem_);

}

};

# 10. IPC – Publishing Pose to Process 4 & 5

The Map Manager (Thread 5) publishes the optimised pose via a shared-memory SeqLock slot (see Section 7.3). Two downstream processes read this slot:

Process 4 (Mission Planner): reads pose for path planning and obstacle avoidance

Process 5 (Comms): reads pose for MAVLink telemetry and GCS display

## 10.1 Shared Memory Layout

// /dev/shm/slam_pose – created by Process 3 at startup

// Layout: single ShmPoseSlot (see Section 7.3)

void MapManager::create_pose_shm() {

int fd = shm_open("/slam_pose", O_CREAT | O_RDWR, 0666);

ftruncate(fd, sizeof(ShmPoseSlot));

auto* slot = static_cast<ShmPoseSlot*>(

mmap(nullptr, sizeof(ShmPoseSlot), PROT_READ | PROT_WRITE,

MAP_SHARED, fd, 0));

slot->sequence.store(0, std::memory_order_relaxed);

pose_publisher_ = std::make_unique<PosePublisher>(slot);

close(fd);  // mapping persists after close

}

## 10.2 Pose Message Format

| Field | Type | Bytes | Description |
| --- | --- | --- | --- |
| sequence | uint64_t (atomic) | 8 | SeqLock counter (odd=writing) |
| timestamp | double | 8 | CLOCK_MONOTONIC seconds |
| translation | double[3] | 24 | x,y,z in NED world frame (metres) |
| quaternion | double[4] | 32 | w,x,y,z (Hamilton convention) |
| covariance | double[36] | 288 | 6×6 pose covariance (row-major) |

# 11. Coordinate Frames & Transformations

Consistent coordinate frame conventions are critical in a SLAM system. A single sign error in a rotation can cause the drone to fly into the ground.

## 11.1 Frame Definitions

| Frame | Name | Origin | Axes | Convention |
| --- | --- | --- | --- | --- |
| W | World / Map | Take-off point | X=North, Y=East, Z=Down | NED |
| B | Body (IMU) | IMU centre | X=Forward, Y=Right, Z=Down | FRD |
| C | Camera | Optical centre | X=Right, Y=Down, Z=Forward | OpenCV |
| L | LiDAR | LiDAR centre | Manufacturer-defined | Varies |

## 11.2 Key Transforms

// Notation: T_target_source transforms a point FROM source TO target.

// p_target = T_target_source * p_source

// The SLAM output is T_world_body (T_WB)

// Camera is rigidly attached to body: T_body_camera (T_BC) from calibration

// To project a world point into the camera:

// p_camera = T_CB * T_BW * p_world

//          = T_camera_world * p_world

//          = (T_WB * T_BC).inverse() * p_world

// Gravity in body frame:

// g_body = R_BW * g_world = R_BW * [0, 0, 9.81]^T  (NED: gravity = +Z)

// T_BC is obtained from camera-IMU extrinsic calibration (Kalibr)

Sophus::SE3d T_body_camera;  // loaded from calibration YAML

## 11.3 Quaternion Convention

We use Hamilton quaternions (q = w + xi + yj + zk) throughout the stack. Eigen::Quaterniond stores coefficients as [x, y, z, w] internally but constructs as Quaterniond(w, x, y, z). Be careful with this ordering. MAVLink ATTITUDE_QUATERNION also uses [w, x, y, z] ordering.

# 12. Available SLAM/VIO Libraries & Algorithms

## 12.1 Complete VIO/SLAM Systems

| System | Type | Sensors | Backend | Loop Closure | License | GPU |
| --- | --- | --- | --- | --- | --- | --- |
| ORB-SLAM3 | V-SLAM + VIO | Mono/Stereo/RGBD + IMU | g2o bundle adj. | DBoW2 | GPL-3.0 | No |
| VINS-Fusion | VIO + SLAM | Mono/Stereo + IMU + GPS | Ceres | DBoW2 | GPL-3.0 | No |
| Kimera-VIO | VIO + Mesh | Stereo + IMU | GTSAM iSAM2 | DBoW2 (Kimera-LCD) | BSD-2 | No |
| OKVIS2 | VIO | Mono/Multi + IMU | Custom Gauss-Newton | Optional | BSD-3 | No |
| SVO Pro | Semi-direct VO+VIO | Mono/Stereo + IMU | Ceres / iSAM2 | Optional | GPL-3.0 | No |
| Basalt | VIO | Stereo + IMU | Custom square-root BA | No | BSD-3 | No |

## 12.2 Learned / Hybrid Approaches

| System | Approach | GPU Required | FPS (Orin Nano est.) | License |
| --- | --- | --- | --- | --- |
| SuperPoint + SuperGlue | CNN feature detect + match | Yes (TensorRT) | ~15-20 | Non-commercial |
| DROID-SLAM | Dense learned SLAM | Yes (heavy) | ~3-5 | BSD-3 |
| DPVO | Dense patch VO | Yes | ~10 | MIT |
| TartanVO | Learning-based VO | Yes | ~8-12 | BSD-3 |

## 12.3 Optimisation Libraries

| Library | Purpose | Key Feature | License |
| --- | --- | --- | --- |
| GTSAM | Factor-graph optimisation | iSAM2 incremental | BSD |
| Ceres Solver | General nonlinear least-squares | Auto-differentiation | Apache-2.0 |
| g2o | Graph optimisation | Lightweight, sparse | BSD |
| Sophus | Lie group types | SE(3)/SO(3) on Eigen | MIT |
| Eigen | Linear algebra | Expression templates | MPL-2.0 |

## 12.4 Recommendation

For the Jetson Orin Nano with limited GPU headroom (most GPU is used by Process 2 DNN inference), we recommend starting with a classical approach:

Primary: ORB-SLAM3 (well-tested, visual-inertial, loop closure) – but GPL-3.0

Alternative: Kimera-VIO (BSD-2 license, iSAM2 backend, actively maintained)

Minimalist: Basalt + custom loop closure (BSD-3, very efficient)

For a from-scratch implementation following this document, the thread architecture maps cleanly to Kimera-VIO's pipeline stages.

# 13. Sensor Configuration & Calibration

## 13.1 Camera Intrinsic Calibration

Use a checkerboard or AprilGrid pattern with OpenCV's calibrateCamera() or Kalibr. Store intrinsics in a YAML file:

# camera_intrinsics.yaml

camera_model: pinhole

distortion_model: radtan

resolution: [640, 480]

intrinsics: [458.654, 457.296, 367.215, 248.375]  # fx, fy, cx, cy

distortion_coefficients: [-0.28340, 0.07395, 0.00019, 0.00001]  # k1, k2, p1, p2

## 13.2 Camera-IMU Extrinsic Calibration

Kalibr (kalibr_calibrate_imu_camera) estimates T_body_camera and the temporal offset t_cam_imu. This is critical – even 1 ms time offset causes significant drift at high speeds.

# camera_imu_extrinsics.yaml (Kalibr output)

T_body_camera:

- [0.0148655, -0.999880,  0.00414, -0.0216401]

- [0.999557,   0.014964,  0.02571, -0.064677]

- [-0.025774, 0.003760,   0.99966,  0.009811]

- [0.0,       0.0,        0.0,      1.0]

time_offset_imu_camera: -0.00781  # seconds (cam trails IMU)

## 13.3 IMU Noise Parameters

Obtain from IMU datasheet or Allan variance analysis. These parameters feed directly into the pre-integrator noise model.

| Parameter | Symbol | Typical BMI088 Value | Unit |
| --- | --- | --- | --- |
| Gyro noise density | σ_g | 0.004 | rad/s/√Hz |
| Gyro random walk | σ_bg | 2.2e-5 | rad/s²/√Hz |
| Accel noise density | σ_a | 0.012 | m/s²/√Hz |
| Accel random walk | σ_ba | 8.0e-5 | m/s³/√Hz |

struct ImuNoiseParams {

double gyro_noise_density  = 0.004;    // rad/s/√Hz

double gyro_random_walk    = 2.2e-5;   // rad/s²/√Hz

double accel_noise_density = 0.012;    // m/s²/√Hz

double accel_random_walk   = 8.0e-5;   // m/s³/√Hz

Eigen::Matrix3d gyro_cov()  const { return Eigen::Matrix3d::Identity() * gyro_noise_density * gyro_noise_density; }

Eigen::Matrix3d accel_cov() const { return Eigen::Matrix3d::Identity() * accel_noise_density * accel_noise_density; }

};

# 14. Performance Tuning for Jetson Orin Nano

## 14.1 CPU Budget

The Jetson Orin Nano has 6× Arm Cortex-A78AE cores. Process 3 uses cores 2–5. Profile with perf and ensure no thread exceeds its frame budget.

| Thread | Target Latency | Budget | Typical Measured |
| --- | --- | --- | --- |
| T1 Visual Frontend | < 20 ms (50 Hz frame budget) | 33 ms @ 30 Hz | 8–15 ms |
| T2 IMU Reader | < 4 ms (250 Hz) | 5 ms | 0.5–1 ms |
| T3 Backend Optimiser | < 100 ms per keyframe | 200 ms (keyframe rate) | 30–80 ms (iSAM2) |
| T4 Loop Closure | < 200 ms (not real-time critical) | 500 ms | 50–150 ms |
| T5 Map Manager | < 5 ms per update | 33 ms | 1–3 ms |

## 14.2 Memory Budget

Total cgroup limit for Process 3: 512 MB (configurable)

Map points: ~50 KB per 1000 landmarks

Keyframes (images not stored, only features): ~20 KB each

GTSAM Bayes tree: ~200 KB for 500 keyframes

BoW database: ~100 KB for 1000 entries with 10K vocabulary

Total estimate at 500 KFs: ~80 MB

## 14.3 NVPMODEL & Jetson Clocks

# Set maximum performance mode

sudo nvpmodel -m 0          # MAXN (all cores at max)

sudo jetson_clocks           # Lock CPU/GPU/EMC clocks at max

# Pin process to specific cores

taskset -c 2-5 ./slam_vio_nav

# Set real-time priority

chrt -f 60 ./slam_vio_nav

## 14.4 Compile-Time Optimisations

Build with -O3 -march=armv8.2-a+fp16+dotprod

Enable Eigen vectorisation: -DEIGEN_DONT_ALIGN_STATICALLY=OFF

Use LTO (Link-Time Optimisation): -flto

Enable NEON intrinsics for feature matching: -mfpu=neon (auto for aarch64)

Profile-Guided Optimisation (PGO): build with -fprofile-generate, run benchmark, rebuild with -fprofile-use

# 15. C++ Concepts Reference

This section consolidates the C++ design patterns and idioms used in Process 3, explaining why each is chosen and how it benefits the SLAM pipeline.

• Strategy Pattern (§3.3)

Swap feature detectors at runtime (ORB → KLT → SuperPoint) without modifying the frontend pipeline. Adheres to the Open/Closed Principle.

• Sophus Lie Groups (§3.5)

Ensures all pose transforms remain on the SE(3) manifold. Provides log/exp maps for optimiser parameterisation in tangent space.

• CRTP – Static Polymorphism (§4.3)

Eliminates virtual dispatch in the 200 Hz IMU loop. Compile-time polymorphism with zero overhead vs. hand-written code.

• RAII for Hardware (§4.5)

File descriptors, shared-memory mappings, and SPI handles are released automatically by destructors. Non-copyable + movable prevents double-close.

• std::optional (§6.2)

Loop closure detection returns std::optional<LoopCandidate> – clearly communicates that "no loop found" is a normal outcome, not an error.

• SeqLock for Shared Memory (§7.3)

Single-writer multi-reader lock-free pattern for publishing pose at high frequency. No mutex required; readers are wait-free.

• Lock-Free SPSC Queue (§8)

The inter-thread communication backbone. No locks, no contention, cache-line padded to prevent false sharing.

• Move Semantics (throughout)

Keyframe data, feature vectors, and map points are std::move'd into queues – avoiding expensive deep copies of cv::Mat and Eigen matrices.

• std::variant + std::visit (for sensor fusion)

When integrating optional sensors (barometer, GPS), std::variant cleanly represents "one of several measurement types" without inheritance.

• constexpr + strong types (§5.4)

Graph keys use enum class and constexpr hashing to prevent node-type mix-ups at compile time.

💡 C++ Concept #9: std::variant + std::visit for Multi-Sensor Measurements

When the SLAM system accepts measurements from different sensor types (camera, IMU, barometer, GPS), std::variant provides a type-safe union. std::visit with a visitor struct ensures exhaustive handling of all types at compile time – if you add a new sensor type but forget to handle it, the code won't compile.

struct CameraMeasurement { double timestamp; std::vector<cv::KeyPoint> keypoints; };

struct ImuMeasurement     { double timestamp; Eigen::Vector3d accel, gyro; };

struct BaroMeasurement    { double timestamp; double altitude_m; };

struct GpsMeasurement     { double timestamp; double lat, lon, alt; double hdop; };

using SensorMeasurement = std::variant<

CameraMeasurement, ImuMeasurement, BaroMeasurement, GpsMeasurement>;

// Visitor pattern with std::visit

struct MeasurementHandler {

BackendOptimiser& backend;

void operator()(const CameraMeasurement& m) { backend.add_visual(m); }

void operator()(const ImuMeasurement& m)    { backend.add_imu(m); }

void operator()(const BaroMeasurement& m)   { backend.add_baro(m); }

void operator()(const GpsMeasurement& m)    { backend.add_gps(m); }

};

// Usage:

SensorMeasurement meas = imu_queue.pop();

std::visit(MeasurementHandler{backend}, meas);

💡 C++ Concept #10: Eigen::Map for Zero-Copy Views

Eigen::Map wraps raw memory as an Eigen vector/matrix without copying data. This is used extensively when reading from shared memory (Section 7.3) and when converting between C arrays and Eigen types for GTSAM/Ceres.

// Wrap raw double[3] in shared memory as Vector3d (zero-copy)

double raw[3] = {1.0, 2.0, 3.0};

Eigen::Map<Eigen::Vector3d> v(raw);

v.normalize();  // modifies raw[] directly

// Const map for read-only access:

const double* pose_data = shm_slot->translation;

Eigen::Map<const Eigen::Vector3d> pos(pose_data);

// Map a contiguous buffer as a matrix:

double cov_data[36];

Eigen::Map<Eigen::Matrix<double,6,6,Eigen::RowMajor>> cov(cov_data);

# 16. systemd Service Configuration

# /etc/systemd/system/slam-vio-nav.service

[Unit]

Description=Process 3 – SLAM / VIO Navigation

After=video-capture.service

Requires=video-capture.service

StartLimitBurst=5

StartLimitIntervalSec=60

[Service]

Type=notify

ExecStart=/opt/drone/bin/slam_vio_nav \

--config /opt/drone/config/slam_config.yaml

WatchdogSec=5

Restart=on-failure

RestartSec=2

# Resource limits (cgroups v2)

MemoryMax=512M

MemoryHigh=400M

CPUAffinity=2 3 4 5

IOWeight=50

# Security hardening

ProtectSystem=strict

ProtectHome=yes

NoNewPrivileges=yes

ReadWritePaths=/dev/shm /opt/drone/maps

# Real-time scheduling

LimitRTPRIO=70

LimitMEMLOCK=infinity

[Install]

WantedBy=drone.target

## 16.1 Startup Sequence

1. systemd starts slam-vio-nav after video-capture.service is ready

2. Process opens shared-memory ring buffer from Process 1 (read-only)

3. Process creates /dev/shm/slam_pose (write) for outgoing pose

4. Process loads calibration files (intrinsics, extrinsics, IMU params)

5. Process optionally loads saved map for re-localisation

6. All 5 threads are spawned with core affinity and RT priority

7. Process calls sd_notify("READY=1") to signal systemd

8. WatchdogSec=5 → backend must call sd_notify("WATCHDOG=1") every ~2.5 s

// In main()

int main(int argc, char* argv[]) {

auto config = load_config(argv);

// Open inputs

auto shm_reader = ShmFrameReader("/video_capture_ring", "/video_sem", 4);

auto imu_driver = SpiImuDriver("/dev/spidev0.0", 8'000'000);

// Create output pose shared memory

create_pose_shm("/slam_pose");

// Load calibration

auto cam_params = load_camera_params(config.camera_yaml);

auto imu_params = load_imu_params(config.imu_yaml);

auto T_BC = load_extrinsics(config.extrinsics_yaml);

// Optionally load saved map

std::optional<Map> prior_map;

if (std::filesystem::exists(config.map_path))

prior_map = load_map(config.map_path);

// Create and start threads

SlamSystem slam(shm_reader, imu_driver, cam_params, imu_params, T_BC, prior_map);

slam.start();

// Signal systemd: we are ready

sd_notify(0, "READY=1");

// Wait for shutdown signal

sigset_t sigs;

sigemptyset(&sigs);

sigaddset(&sigs, SIGTERM);

sigaddset(&sigs, SIGINT);

sigprocmask(SIG_BLOCK, &sigs, nullptr);

int sig;

sigwait(&sigs, &sig);

slam.stop();

save_map(config.map_path, slam.get_map());

sd_notify(0, "STOPPING=1");

return 0;

}

# 17. CMakeLists.txt

cmake_minimum_required(VERSION 3.18)

project(slam_vio_nav LANGUAGES CXX)

set(CMAKE_CXX_STANDARD 17)

set(CMAKE_CXX_STANDARD_REQUIRED ON)

set(CMAKE_CXX_FLAGS_RELEASE "-O3 -march=armv8.2-a+fp16+dotprod -flto -DNDEBUG")

# ── Dependencies ──

find_package(OpenCV 4.5 REQUIRED COMPONENTS core imgproc features2d calib3d)

find_package(Eigen3 3.3 REQUIRED NO_MODULE)

find_package(GTSAM REQUIRED)

find_package(Sophus REQUIRED)

find_package(spdlog REQUIRED)

# DBoW2 (build from source or find_package)

find_package(DBoW2 QUIET)

if(NOT DBoW2_FOUND)

add_subdirectory(third_party/DBoW2)

endif()

# cereal (header-only)

find_package(cereal REQUIRED)

# systemd for sd_notify

find_package(PkgConfig REQUIRED)

pkg_check_modules(SYSTEMD REQUIRED libsystemd)

# ── Sources ──

set(SOURCES

src/main.cpp

src/visual_frontend.cpp

src/imu_reader.cpp

src/backend_optimiser.cpp

src/loop_closure.cpp

src/map_manager.cpp

src/feature_detectors/orb_detector.cpp

src/feature_detectors/klt_tracker.cpp

src/ipc/shm_frame_reader.cpp

src/ipc/pose_publisher.cpp

src/calibration/calib_loader.cpp

src/utils/spsc_queue.hpp

)

add_executable(${PROJECT_NAME} ${SOURCES})

target_include_directories(${PROJECT_NAME} PRIVATE

${CMAKE_CURRENT_SOURCE_DIR}/include

${SYSTEMD_INCLUDE_DIRS}

)

target_link_libraries(${PROJECT_NAME} PRIVATE

OpenCV::core OpenCV::imgproc OpenCV::features2d OpenCV::calib3d

Eigen3::Eigen

gtsam

Sophus::Sophus

spdlog::spdlog

DBoW2

cereal::cereal

${SYSTEMD_LIBRARIES}

pthread rt

)

# ── Install ──

install(TARGETS ${PROJECT_NAME} DESTINATION /opt/drone/bin)

install(DIRECTORY config/ DESTINATION /opt/drone/config

FILES_MATCHING PATTERN "*.yaml")

# 18. Implementation Roadmap

## Phase 1: IMU Reader & Pre-integration (Week 1–2)

Implement SPI IMU driver (RAII, read_blocking)

Implement pre-integration class with CRTP noise model

Unit test: compare pre-integrated output vs. numerical integration

Verify 200 Hz read rate with jitter < 0.5 ms

## Phase 2: Visual Frontend (Week 2–4)

Implement shared-memory frame reader (connect to Process 1)

Implement ORB grid detector (Strategy pattern)

Implement BFMatcher + RANSAC geometric verification

Implement keyframe decision logic

End-to-end test: visualise tracked features on live frames

## Phase 3: Backend Optimiser (Week 4–6)

Integrate GTSAM with iSAM2

Add CombinedImuFactor + SmartProjectionPoseFactor

Wire keyframe + IMU factor queues (SPSC)

Test with EuRoC MAV dataset – verify ATE < 0.1 m

Benchmark: ensure < 100 ms per keyframe update

## Phase 4: Pose Publication & IPC (Week 6–7)

Implement SeqLock ShmPoseSlot publisher

Implement reader side for Process 4 & 5

Verify pose publication rate ≥ 30 Hz

Verify multi-reader correctness with stress test

## Phase 5: Loop Closure (Week 7–9)

Integrate DBoW2 vocabulary & database

Implement geometric verification (PnP RANSAC)

Wire loop-closure factor to backend

Test on datasets with revisits – verify drift correction

## Phase 6: Map Persistence & Hardening (Week 9–11)

Implement map serialisation (cereal binary)

Implement re-localisation on startup

systemd integration: sd_notify, watchdog, cgroup limits

Flight test: verify < 0.5 m position error over 5 min flight

Stress test: long-duration (30 min) indoor/outdoor flights

Document configuration parameters and tuning guide