Process 1: VIDEO / CAPTURE

Detailed Implementation Guide

Companion Computer Software Stack — Component 1 of 7

Version 1.0 — February 23, 2026

Table of Contents

1. Process Overview & Responsibilities

2. Architecture Diagram — Internal Thread Layout

3. Shared Data Structures

4. Thread 1: Mission Camera Capture (V4L2)

5. Thread 2: Stereo Camera Capture (V4L2)

6. Thread 3: ISP / Preprocessing

7. Thread 4: H.265 Encode → RTP Stream

8. Thread 5: SSD Recording

9. Inter-Thread Communication

10. Inter-Process Communication (to Perception & SLAM)

11. Startup, Shutdown & Error Handling

12. systemd Service Configuration

13. Available Libraries & Algorithms

14. Build System (CMake)

15. Implementation Roadmap

1. Process Overview & Responsibilities

The Video/Capture process is the single owner of all camera hardware on the companion computer. It is responsible for:

Opening and configuring all camera devices (V4L2)

Running ISP (Image Signal Processing) — debayer, white balance, denoise, lens correction

Distributing raw/processed frames to Perception and SLAM via shared memory (zero-copy)

Encoding video to H.265 and streaming to the Ground Control Station via RTP/UDP

Recording mission footage to onboard SSD

By centralizing all camera access in one process, we eliminate:

V4L2 device contention (only one process opens /dev/videoX)

Duplicate ISP pipelines (process once, distribute many)

Inconsistent exposure/white balance between consumers

1.1 Process Identity

| Property | Value |
| --- | --- |
| Process name | video-capture |
| Binary path | /opt/drone/bin/video-capture |
| systemd unit | video-capture.service |
| User | drone (non-root, with video group) |
| CPU affinity | Cores 2-3 (shared with encode) |
| Memory limit | 512 MB (cgroup MemoryMax) |
| Priority | SCHED_FIFO priority 80 (below SLAM/Perception) |
| Restart policy | always, RestartSec=200ms |
| Language | C++17 |
| Key dependencies | V4L2, libv4l2, GStreamer or custom pipeline, NVENC (Jetson) |

1.2 What This Process Does NOT Do

Object detection/inference — that is Perception (Process 2)

Feature extraction/tracking — that is SLAM (Process 3)

Any flight control decisions

Network routing or MAVLink — that is Comms (Process 5)

2. Architecture Diagram — Internal Thread Layout

┌─────────────────────────────────────────────────────────────────────────────────┐ │ PROCESS 1: VIDEO / CAPTURE │ │ PID: video-capture │ │ │ │ ┌──────────────────┐ ┌──────────────────┐ │ │ │ Thread 1: │ │ Thread 2: │ │ │ │ MISSION CAM │ │ STEREO CAM │ HARDWARE │ │ │ │ │ │ ════════ │ │ │ /dev/video0 │ │ /dev/video2 (L) │ Mission: IMX477 / OV5647 │ │ │ V4L2 MMAP │ │ /dev/video3 (R) │ Stereo: OAK-D / RealSense │ │ │ 1920×1080 @30fps │ │ V4L2 MMAP │ or custom stereo pair │ │ │ │ │ 640×480 @30fps │ │ │ └────────┬──────────┘ └────────┬──────────┘ │ │ │ │ │ │ │ raw Bayer/YUV │ raw Bayer/YUV │ │ ▼ ▼ │ │ ┌──────────────────────────────────────────────────┐ │ │ │ Thread 3: ISP / PREPROCESSING │ │ │ │ │ │ │ │ Debayer → White Balance → Denoise → Undistort │ │ │ │ │ │ │ │ Input: SPSC queue from Thread 1 & 2 │ │ │ │ Output 1: mission_cam RGB → shm ring → Perception│ │ │ │ Output 2: stereo pair RGB → shm ring → SLAM │ │ │ │ Output 3: mission_cam RGB → internal queue → T4 │ │ │ │ Output 4: mission_cam RGB → internal queue → T5 │ │ │ └──────────────────────┬────────────────────────────┘ │ │ │ │ │ ┌───────────┴───────────┐ │ │ ▼ ▼ │ │ ┌──────────────────────┐ ┌──────────────────────┐ │ │ │ Thread 4: │ │ Thread 5: │ │ │ │ H.265 ENCODE+STREAM │ │ SSD RECORDING │ │ │ │ │ │ │ │ │ │ NVENC HW encoder │ │ MKV container │ │ │ │ → NAL units │ │ async write (O_DIRECT)│ │ │ │ → FEC packets │ │ circular buffer mode │ │ │ │ → RTP/UDP │ │ │ │ │ │ → RF link → GCS │ │ /data/recording/ │ │ │ └──────────────────────┘ └──────────────────────┘ │ │ │ │ ┌──────────────────────────────────────────────────────────────────┐ │ │ │ SHARED MEMORY OUTPUT (IPC to other processes) │ │ │ │ │ │ │ │ /drone_mission_cam ──────────────────────────► PERCEPTION │ │ │ │ (shm ring buffer, 4 slots × 6MB = 24MB) │ │ │ │ │ │ │ │ /drone_stereo_cam ──────────────────────────► SLAM │ │ │ │ (shm ring buffer, 4 slots × 1.8MB = 7.2MB) │ │ │ └──────────────────────────────────────────────────────────────────┘ │ └─────────────────────────────────────────────────────────────────────────────────────┘

Key design principle: data flows downward from capture → ISP → consumers. No thread ever flows data upward. The only feedback path is the adaptive bitrate signal from Thread 4 back to Thread 3 (to request lower resolution during RF link degradation).

3. Shared Data Structures

Before diving into individual threads, here are the core data structures shared across all threads and processes. These go in a common header included by the video-capture process and its consumers (Perception, SLAM).

3.1 Frame Metadata

include/video/frame.h — Frame metadata shared by all threads

// file: include/video/frame.h #pragma once #include <cstdint> #include <cstring> namespace drone::video { enum class PixelFormat : uint32_t { BAYER_RGGB8 = 0, BAYER_GRBG8 = 1, YUV422 = 2, RGB24 = 3, NV12 = 4, // for HW encoder input GRAY8 = 5, // mono for SLAM }; enum class CameraId : uint8_t { MISSION = 0, STEREO_L = 1, STEREO_R = 2, }; struct FrameMetadata { uint64_t timestamp_ns; // CLOCK_MONOTONIC — consistent across threads uint64_t sequence_number; // monotonic frame counter CameraId camera_id; PixelFormat format; uint32_t width; uint32_t height; uint32_t stride; // bytes per row (includes padding) uint32_t exposure_us; // actual exposure time — useful for SLAM float gain_db; // actual analog gain float temperature_c; // sensor temperature (for thermal drift compensation) }; } // namespace drone::video

3.2 SPSC Ring Buffer (Intra-Process, Between Threads)

Threads within the same process use a lock-free Single-Producer Single-Consumer (SPSC) ring buffer. Since they share the same address space, we CAN use pointers here (unlike inter-process where we use offsets).

include/video/spsc_ring.h — Lock-free SPSC ring buffer

// file: include/video/spsc_ring.h #pragma once #include <atomic> #include <array> #include <optional> #include <cassert> namespace drone::video { /// Lock-free SPSC ring buffer for intra-process thread communication. /// T must be trivially copyable (e.g., FrameBuffer with pixel data). /// N must be a power of 2. template <typename T, size_t N> class SPSCRing { static_assert((N & (N - 1)) == 0, "N must be power of 2"); public: /// Producer calls this. Returns false if ring is full (consumer too slow). bool try_push(const T& item) { const uint64_t w = write_idx_.load(std::memory_order_relaxed); const uint64_t r = read_idx_.load(std::memory_order_acquire); if (w - r >= N) { return false; // ring full — drop frame or block } slots_[w & (N - 1)] = item; write_idx_.store(w + 1, std::memory_order_release); return true; } /// Consumer calls this. Returns std::nullopt if ring is empty. std::optional<T> try_pop() { const uint64_t r = read_idx_.load(std::memory_order_relaxed); const uint64_t w = write_idx_.load(std::memory_order_acquire); if (r >= w) { return std::nullopt; // ring empty — no new frame } T item = slots_[r & (N - 1)]; read_idx_.store(r + 1, std::memory_order_release); return item; } /// How many items are available to consume. uint64_t available() const { return write_idx_.load(std::memory_order_acquire) - read_idx_.load(std::memory_order_relaxed); } private: alignas(64) std::atomic<uint64_t> write_idx_{0}; // own cache line alignas(64) std::atomic<uint64_t> read_idx_{0}; // own cache line std::array<T, N> slots_; }; } // namespace drone::video

Why SPSC and not MPMC? Each camera thread is the sole producer for its queue, and the ISP thread is the sole consumer. One producer, one consumer — SPSC is the simplest, fastest option. No CAS loops, no contention, just two atomic counters.

3.3 Shared Memory Ring Buffer (Inter-Process, to Perception/SLAM)

For cross-process communication, we cannot use pointers (different virtual address spaces). Instead, the frame data is embedded directly in shared memory, and we use offsets.

include/video/shm_ring.h — Shared memory ring buffer (inter-process)

// file: include/video/shm_ring.h #pragma once #include "frame.h" #include <atomic> #include <cstddef> #include <sys/mman.h> #include <sys/stat.h> #include <fcntl.h> #include <unistd.h> #include <cstring> #include <stdexcept> namespace drone::video { /// Layout of the shared memory region. /// This struct IS the shared memory — both producer and consumer map it. struct ShmRingHeader { alignas(64) std::atomic<uint64_t> write_idx; // producer increments alignas(64) std::atomic<uint64_t> read_idx; // consumer increments uint32_t num_slots; uint32_t slot_size; // sizeof(ShmFrameSlot) + pixel_data_size uint32_t pixel_data_size; uint32_t magic; // 0xDEADCAFE — validates correct mapping // Slot data follows immediately after this header }; struct ShmFrameSlot { FrameMetadata meta; uint8_t pixel_data[]; // flexible array — actual pixels live here }; /// Producer side — creates and writes to shared memory ring. class ShmRingProducer { public: ShmRingProducer(const char* name, uint32_t num_slots, uint32_t width, uint32_t height, uint32_t channels) { pixel_size_ = width * height * channels; slot_size_ = sizeof(ShmFrameSlot) + pixel_size_; size_t total = sizeof(ShmRingHeader) + num_slots * slot_size_; // Create shared memory object fd_ = shm_open(name, O_CREAT ¦ O_RDWR, 0666); if (fd_ < 0) throw std::runtime_error("shm_open failed"); if (ftruncate(fd_, total) < 0) throw std::runtime_error("ftruncate failed"); base_ = mmap(nullptr, total, PROT_READ ¦ PROT_WRITE, MAP_SHARED, fd_, 0); if (base_ == MAP_FAILED) throw std::runtime_error("mmap failed"); total_size_ = total; // Initialize header auto* hdr = header(); hdr->write_idx.store(0); hdr->read_idx.store(0); hdr->num_slots = num_slots; hdr->slot_size = slot_size_; hdr->pixel_data_size = pixel_size_; hdr->magic = 0xDEADCAFE; } ~ShmRingProducer() { if (base_ != MAP_FAILED) munmap(base_, total_size_); if (fd_ >= 0) close(fd_); } /// Write a frame into the next slot. Returns false if ring is full. bool publish(const FrameMetadata& meta, const uint8_t* pixels) { auto* hdr = header(); uint64_t w = hdr->write_idx.load(std::memory_order_relaxed); uint64_t r = hdr->read_idx.load(std::memory_order_acquire); if (w - r >= hdr->num_slots) { return false; // consumer too slow — drop frame } auto* slot = get_slot(w); slot->meta = meta; std::memcpy(slot->pixel_data, pixels, pixel_size_); hdr->write_idx.store(w + 1, std::memory_order_release); return true; } private: ShmRingHeader* header() { return static_cast<ShmRingHeader*>(base_); } ShmFrameSlot* get_slot(uint64_t idx) { auto* hdr = header(); char* data_start = static_cast<char*>(base_) + sizeof(ShmRingHeader); return reinterpret_cast<ShmFrameSlot*>( data_start + (idx % hdr->num_slots) * slot_size_ ); } int fd_ = -1; void* base_ = MAP_FAILED; size_t total_size_ = 0; uint32_t pixel_size_ = 0; uint32_t slot_size_ = 0; }; /// Consumer side — opens and reads from shared memory ring (read-only). class ShmRingConsumer { public: explicit ShmRingConsumer(const char* name) { fd_ = shm_open(name, O_RDONLY, 0); if (fd_ < 0) throw std::runtime_error("shm_open failed"); struct stat st; fstat(fd_, &st); total_size_ = st.st_size; base_ = mmap(nullptr, total_size_, PROT_READ, MAP_SHARED, fd_, 0); if (base_ == MAP_FAILED) throw std::runtime_error("mmap failed"); if (header()->magic != 0xDEADCAFE) throw std::runtime_error("Invalid shared memory — magic mismatch"); } ~ShmRingConsumer() { if (base_ != MAP_FAILED) munmap(base_, total_size_); if (fd_ >= 0) close(fd_); } /// Returns pointer to latest unread frame, or nullptr if no new frame. /// Caller must finish reading before calling advance(). const ShmFrameSlot* peek() const { auto* hdr = header(); uint64_t r = hdr->read_idx.load(std::memory_order_relaxed); uint64_t w = hdr->write_idx.load(std::memory_order_acquire); if (r >= w) return nullptr; return get_slot(r); } /// Advance read pointer after consuming a frame. void advance() { auto* hdr = const_cast<ShmRingHeader*>(header()); uint64_t r = hdr->read_idx.load(std::memory_order_relaxed); hdr->read_idx.store(r + 1, std::memory_order_release); } private: const ShmRingHeader* header() const { return static_cast<const ShmRingHeader*>(base_); } const ShmFrameSlot* get_slot(uint64_t idx) const { auto* hdr = header(); const char* data_start = static_cast<const char*>(base_) + sizeof(ShmRingHeader); return reinterpret_cast<const ShmFrameSlot*>( data_start + (idx % hdr->num_slots) * hdr->slot_size ); } int fd_ = -1; void* base_ = MAP_FAILED; size_t total_size_ = 0; }; } // namespace drone::video

The consumer maps the region as PROT_READ (read-only) — it cannot corrupt the producer. Ownership is enforced by the ring protocol: slots between read_idx and write_idx belong to the consumer. All others belong to the producer.

4. Thread 1: Mission Camera Capture (V4L2)

4.1 Purpose

Opens the mission camera (high-resolution, potentially gimballed for ISR), configures it via V4L2 ioctls, and captures frames into DMA-mapped buffers at 30 fps. Pushes raw frames into the SPSC queue consumed by the ISP thread.

4.2 V4L2 Capture Pipeline

V4L2 (Video4Linux2) is the Linux kernel API for video capture devices. The capture flow is:

Application Kernel / Driver ─────────── ────────────── 1. open("/dev/video0") ──────────► Bind to camera driver 2. VIDIOC_QUERYCAP ──────────► Verify V4L2_CAP_VIDEO_CAPTURE 3. VIDIOC_S_FMT ──────────► Set resolution, pixel format 4. VIDIOC_REQBUFS (MMAP)──────────► Allocate N kernel DMA buffers 5. VIDIOC_QUERYBUF × N ──────────► Get buffer offsets 6. mmap() × N ──────────► Map DMA buffers to userspace 7. VIDIOC_QBUF × N ──────────► Enqueue all buffers to driver 8. VIDIOC_STREAMON ──────────► Start capture ┌─── CAPTURE LOOP (30 fps) ───────────────────────────────────────┐ │ │ │ 9. poll(fd, POLLIN) ◄───────── Wait for frame interrupt │ │ 10. VIDIOC_DQBUF ◄───────── Dequeue filled buffer │ │ 11. process(buffer) Push to SPSC queue │ │ 12. VIDIOC_QBUF ──────────► Re-enqueue buffer │ │ │ └─────────────────────────────────────────────────────────────────┘ 13. VIDIOC_STREAMOFF ──────────► Stop capture (on shutdown)

4.3 Implementation

src/video/mission_cam_thread.cpp — V4L2 Camera class

// file: src/video/mission_cam_thread.cpp #include "video/frame.h" #include "video/spsc_ring.h" #include <linux/videodev2.h> #include <sys/ioctl.h> #include <sys/mman.h> #include <fcntl.h> #include <unistd.h> #include <poll.h> #include <cstring> #include <stdexcept> #include <chrono> #include <spdlog/spdlog.h> namespace drone::video { static constexpr int NUM_BUFFERS = 4; // V4L2 kernel ring size struct V4L2Buffer { void* start; size_t length; }; /// Wraps V4L2 device lifecycle: open, configure, capture, close. class V4L2Camera { public: V4L2Camera(const char* device, uint32_t w, uint32_t h, uint32_t pixfmt = V4L2_PIX_FMT_SRGGB10) : width_(w), height_(h) { fd_ = open(device, O_RDWR ¦ O_NONBLOCK); if (fd_ < 0) throw std::runtime_error(std::string("Cannot open ") + device); // 1. Verify capability v4l2_capability cap{}; xioctl(VIDIOC_QUERYCAP, &cap); if (!(cap.capabilities & V4L2_CAP_VIDEO_CAPTURE)) throw std::runtime_error("Device does not support capture"); // 2. Set format v4l2_format fmt{}; fmt.type = V4L2_BUF_TYPE_VIDEO_CAPTURE; fmt.fmt.pix.width = w; fmt.fmt.pix.height = h; fmt.fmt.pix.pixelformat = pixfmt; fmt.fmt.pix.field = V4L2_FIELD_NONE; xioctl(VIDIOC_S_FMT, &fmt); // Driver may adjust — read back actual values width_ = fmt.fmt.pix.width; height_ = fmt.fmt.pix.height; stride_ = fmt.fmt.pix.bytesperline; spdlog::info("Camera {}: {}x{} stride={} fmt=0x{:X}", device, width_, height_, stride_, fmt.fmt.pix.pixelformat); // 3. Request MMAP buffers v4l2_requestbuffers req{}; req.count = NUM_BUFFERS; req.type = V4L2_BUF_TYPE_VIDEO_CAPTURE; req.memory = V4L2_MEMORY_MMAP; xioctl(VIDIOC_REQBUFS, &req); // 4. Map each buffer for (uint32_t i = 0; i < req.count; ++i) { v4l2_buffer buf{}; buf.type = V4L2_BUF_TYPE_VIDEO_CAPTURE; buf.memory = V4L2_MEMORY_MMAP; buf.index = i; xioctl(VIDIOC_QUERYBUF, &buf); void* ptr = mmap(nullptr, buf.length, PROT_READ ¦ PROT_WRITE, MAP_SHARED, fd_, buf.m.offset); if (ptr == MAP_FAILED) throw std::runtime_error("mmap failed for buffer " + std::to_string(i)); buffers_[i] = {ptr, buf.length}; } // 5. Enqueue all buffers for (uint32_t i = 0; i < req.count; ++i) { v4l2_buffer buf{}; buf.type = V4L2_BUF_TYPE_VIDEO_CAPTURE; buf.memory = V4L2_MEMORY_MMAP; buf.index = i; xioctl(VIDIOC_QBUF, &buf); } // 6. Start streaming int type = V4L2_BUF_TYPE_VIDEO_CAPTURE; xioctl(VIDIOC_STREAMON, &type); } ~V4L2Camera() { int type = V4L2_BUF_TYPE_VIDEO_CAPTURE; ioctl(fd_, VIDIOC_STREAMOFF, &type); for (auto& b : buffers_) { if (b.start) munmap(b.start, b.length); } if (fd_ >= 0) close(fd_); } struct CapturedFrame { const uint8_t* data; size_t size; uint64_t timestamp_ns; uint32_t sequence; uint32_t buffer_index; // needed for re-enqueue }; /// Block until a frame is ready (poll + dequeue). Returns the frame. CapturedFrame capture() { // Poll with 1-second timeout (detect hardware failure) pollfd pfd{fd_, POLLIN, 0}; int ret = poll(&pfd, 1, 1000); if (ret <= 0) throw std::runtime_error("Camera poll timeout — hardware failure?"); v4l2_buffer buf{}; buf.type = V4L2_BUF_TYPE_VIDEO_CAPTURE; buf.memory = V4L2_MEMORY_MMAP; xioctl(VIDIOC_DQBUF, &buf); // Convert kernel timestamp to CLOCK_MONOTONIC nanoseconds uint64_t ts = buf.timestamp.tv_sec * 1'000'000'000ULL + buf.timestamp.tv_usec * 1'000ULL; return { static_cast<const uint8_t*>(buffers_[buf.index].start), buf.bytesused, ts, buf.sequence, buf.index }; } /// Re-enqueue a buffer after processing. void release(uint32_t buffer_index) { v4l2_buffer buf{}; buf.type = V4L2_BUF_TYPE_VIDEO_CAPTURE; buf.memory = V4L2_MEMORY_MMAP; buf.index = buffer_index; xioctl(VIDIOC_QBUF, &buf); } uint32_t width() const { return width_; } uint32_t height() const { return height_; } uint32_t stride() const { return stride_; } private: void xioctl(unsigned long request, void* arg) { int ret; do { ret = ioctl(fd_, request, arg); } while (ret == -1 && errno == EINTR); // retry on signal if (ret < 0) throw std::runtime_error("ioctl failed: " + std::to_string(errno)); } int fd_ = -1; uint32_t width_, height_, stride_; V4L2Buffer buffers_[NUM_BUFFERS]{}; }; } // namespace drone::video

4.4 The Capture Thread Function

Mission camera thread entry point

// file: src/video/mission_cam_thread.cpp (continued) #include <thread> #include <atomic> namespace drone::video { /// This is the thread entry point. Runs until stop_flag is set. void mission_cam_thread( SPSCRing<CapturedFrame, 4>& output_queue, // → consumed by ISP thread std::atomic<bool>& stop_flag, const char* device = "/dev/video0") { try { V4L2Camera cam(device, 1920, 1080, V4L2_PIX_FMT_SRGGB10); spdlog::info("Mission camera started: {}x{}", cam.width(), cam.height()); uint64_t frame_count = 0; uint64_t drop_count = 0; while (!stop_flag.load(std::memory_order_relaxed)) { auto frame = cam.capture(); // blocks on poll() if (!output_queue.try_push(frame)) { // ISP thread can't keep up — drop this frame ++drop_count; if (drop_count % 100 == 0) { spdlog::warn("Mission cam: dropped {} frames (ISP too slow)", drop_count); } } cam.release(frame.buffer_index); // re-enqueue to kernel ++frame_count; } spdlog::info("Mission cam stopped. {} frames captured, {} dropped", frame_count, drop_count); } catch (const std::exception& e) { spdlog::error("Mission cam thread FATAL: {}", e.what()); // Process-level error handler will decide: restart thread or whole process } } } // namespace drone::video

4.5 V4L2 MMAP vs USERPTR vs DMA-BUF

| Method | How It Works | Zero-Copy? | When to Use |
| --- | --- | --- | --- |
| MMAP | Kernel allocates DMA buffers, maps to userspace | Yes (kernel→user) | Default — works with all V4L2 drivers |
| USERPTR | Userspace allocates buffers, passes to kernel | Yes | When you need custom allocators (GPU memory) |
| DMA-BUF | File descriptor references a DMA buffer shareable across devices | Yes (cross-device) | Jetson: camera → GPU → encoder without CPU touch |

On Jetson, DMA-BUF is the ideal path: the camera ISP writes to a DMA buffer, the GPU reads it (for inference or ISP), and NVENC reads it (for encoding) — the CPU never touches the pixel data. For initial implementation, start with MMAP. Optimize to DMA-BUF once the pipeline works.

5. Thread 2: Stereo Camera Capture (V4L2)

5.1 Purpose

Captures synchronized left/right stereo frames for SLAM. The key challenge is ensuring the left and right images are captured at the same instant — temporal misalignment causes depth errors in stereo matching.

5.2 Stereo Synchronization Methods

| Method | How | Accuracy | Cameras |
| --- | --- | --- | --- |
| Hardware sync (FSIN) | External trigger signal wired to both cameras | < 10 µs | OV9281, IMX477 (with trigger) |
| USB frame sync | OAK-D / RealSense sync internally | < 100 µs | OAK-D Lite, RealSense D435i |
| Software sync | Capture both, match by timestamp | 1-5 ms | Any — fallback only |
| Single stereo sensor | One sensor captures both (side by side) | Perfect | ZED, OAK-D Pro, custom dual-sensor PCB |

For the Q-X drone, the recommended approach is a hardware-synced stereo pair (OV9281 global shutter, no rolling shutter distortion during drone vibration) or an integrated stereo module (OAK-D). SLAM quality degrades significantly with rolling shutter on a vibrating platform.

5.3 Implementation — Synchronized Stereo Capture

src/video/stereo_cam_thread.cpp — Stereo capture with sync validation

// file: src/video/stereo_cam_thread.cpp #include "video/frame.h" #include "video/spsc_ring.h" #include <spdlog/spdlog.h> #include <atomic> namespace drone::video { struct StereoFrame { FrameMetadata meta_left; FrameMetadata meta_right; const uint8_t* left_data; const uint8_t* right_data; size_t data_size; uint32_t left_buf_idx; uint32_t right_buf_idx; }; /// For hardware-synced stereo: both cameras trigger simultaneously. /// We open two V4L2 devices and dequeue from both per iteration. void stereo_cam_thread( SPSCRing<StereoFrame, 4>& output_queue, // → consumed by ISP thread std::atomic<bool>& stop_flag, const char* left_device = "/dev/video2", const char* right_device = "/dev/video3") { try { V4L2Camera cam_left(left_device, 640, 480, V4L2_PIX_FMT_GREY); V4L2Camera cam_right(right_device, 640, 480, V4L2_PIX_FMT_GREY); spdlog::info("Stereo cameras started: {}x{} (L) + {}x{} (R)", cam_left.width(), cam_left.height(), cam_right.width(), cam_right.height()); while (!stop_flag.load(std::memory_order_relaxed)) { // With hardware sync, both frames are ready at the same time auto left = cam_left.capture(); auto right = cam_right.capture(); // Verify sync — timestamps should be within 1ms int64_t dt = static_cast<int64_t>(left.timestamp_ns) - static_cast<int64_t>(right.timestamp_ns); if (std::abs(dt) > 1'000'000) { // > 1 ms spdlog::warn("Stereo sync drift: {} µs", dt / 1000); // Could try to re-sync or drop this pair } StereoFrame sf{}; sf.meta_left = {left.timestamp_ns, left.sequence, CameraId::STEREO_L, PixelFormat::GRAY8, cam_left.width(), cam_left.height(), cam_left.stride(), 0, 0.0f, 0.0f}; sf.meta_right = {right.timestamp_ns, right.sequence, CameraId::STEREO_R, PixelFormat::GRAY8, cam_right.width(), cam_right.height(), cam_right.stride(), 0, 0.0f, 0.0f}; sf.left_data = left.data; sf.right_data = right.data; sf.data_size = left.size; sf.left_buf_idx = left.buffer_index; sf.right_buf_idx = right.buffer_index; if (!output_queue.try_push(sf)) { spdlog::warn("Stereo: ISP queue full, dropping frame pair"); } cam_left.release(left.buffer_index); cam_right.release(right.buffer_index); } } catch (const std::exception& e) { spdlog::error("Stereo cam thread FATAL: {}", e.what()); } } } // namespace drone::video

5.4 Alternative: OAK-D / RealSense (DepthAI / librealsense)

If using an integrated stereo module like OAK-D or RealSense, you use the vendor SDK instead of raw V4L2. The capture thread is simpler — the SDK handles synchronization internally.

Alternative using Intel RealSense SDK

// Alternative: Using Intel RealSense D435i // #include <librealsense2/rs.hpp> // // void stereo_cam_thread_realsense(SPSCRing<StereoFrame, 4>& q, // std::atomic<bool>& stop) { // rs2::pipeline pipe; // rs2::config cfg; // cfg.enable_stream(RS2_STREAM_INFRARED, 1, 640, 480, RS2_FORMAT_Y8, 30); // cfg.enable_stream(RS2_STREAM_INFRARED, 2, 640, 480, RS2_FORMAT_Y8, 30); // cfg.enable_stream(RS2_STREAM_ACCEL); // for VIO // cfg.enable_stream(RS2_STREAM_GYRO); // pipe.start(cfg); // // while (!stop.load()) { // auto frameset = pipe.wait_for_frames(); // auto left = frameset.get_infrared_frame(1); // auto right = frameset.get_infrared_frame(2); // // Both are hardware-synced — push to queue // } // }

6. Thread 3: ISP / Preprocessing

6.1 Purpose

Consumes raw Bayer/YUV frames from Thread 1 and Thread 2, applies image signal processing, and distributes the processed frames to four outputs:

Output 1: Mission camera RGB → shared memory ring buffer → Perception process

Output 2: Stereo pair (rectified grayscale) → shared memory ring buffer → SLAM process

Output 3: Mission camera RGB/NV12 → SPSC queue → Thread 4 (encoder)

Output 4: Mission camera RGB → SPSC queue → Thread 5 (SSD recording)

6.2 ISP Pipeline Stages

Raw Bayer Frame (from V4L2) │ ▼ ┌─────────────┐ │ DEBAYER │ Bayer RGGB → RGB24 (or use OpenCV cvtColor) │ (demosaic) │ Algorithm: bilinear, VNG, edge-aware └──────┬───────┘ ▼ ┌─────────────┐ │ WHITE │ Correct color temperature for current lighting │ BALANCE │ Grey-world algorithm or manual preset (outdoor/indoor) └──────┬───────┘ ▼ ┌─────────────┐ │ DENOISE │ Reduce sensor noise (critical at low light) │ │ Algorithm: bilateral filter, NLM, or BM3D └──────┬───────┘ ▼ ┌─────────────┐ │ LENS │ Remove barrel/pincushion distortion │ UNDISTORT │ Uses calibration matrix (cv::undistort or cv::remap) └──────┬───────┘ ▼ ┌─────────────┐ │ COLOR │ RGB → NV12 for HW encoder (NVENC wants NV12/YUV420) │ CONVERT │ Also produce grayscale for SLAM if needed └──────┬───────┘ │ ┌────┼────┬───────────┐ ▼ ▼ ▼ ▼ SHM SHM SPSC SPSC Perc SLAM Encoder SSD

6.3 Implementation

src/video/isp_thread.cpp — Full ISP pipeline implementation

// file: src/video/isp_thread.cpp #include "video/frame.h" #include "video/spsc_ring.h" #include "video/shm_ring.h" #include <opencv2/core.hpp> #include <opencv2/imgproc.hpp> #include <opencv2/calib3d.hpp> #include <spdlog/spdlog.h> #include <atomic> namespace drone::video { /// ISP configuration — loaded from YAML config file at startup. struct ISPConfig { // Lens calibration (from cv::calibrateCamera) cv::Mat camera_matrix; // 3×3 intrinsic matrix cv::Mat dist_coeffs; // distortion coefficients (k1,k2,p1,p2,k3) cv::Mat rectify_map1; // precomputed for cv::remap (faster than undistort) cv::Mat rectify_map2; // White balance gains float wb_red = 1.0f; float wb_green = 1.0f; float wb_blue = 1.0f; // Denoise strength (0 = off, 10 = aggressive) int denoise_strength = 3; // Whether to produce NV12 for encoder (vs RGB) bool produce_nv12 = true; }; /// Precompute undistortion remap tables (call once at startup). void init_undistort_maps(ISPConfig& cfg, int width, int height) { cv::Mat new_camera_matrix = cv::getOptimalNewCameraMatrix( cfg.camera_matrix, cfg.dist_coeffs, cv::Size(width, height), 0.0 // alpha=0: crop to valid pixels only ); cv::initUndistortRectifyMap( cfg.camera_matrix, cfg.dist_coeffs, cv::Mat(), // R (identity — no stereo rectification for mission cam) new_camera_matrix, cv::Size(width, height), CV_16SC2, // fixed-point for speed cfg.rectify_map1, cfg.rectify_map2 ); spdlog::info("ISP: undistortion maps precomputed for {}x{}", width, height); } /// Apply ISP pipeline to a single mission camera frame. cv::Mat process_mission_frame(const uint8_t* raw_data, uint32_t width, uint32_t height, uint32_t stride, const ISPConfig& cfg) { // 1. Debayer (Bayer RGGB → RGB) cv::Mat bayer(height, width, CV_8UC1, const_cast<uint8_t*>(raw_data), stride); cv::Mat rgb; cv::cvtColor(bayer, rgb, cv::COLOR_BayerRG2RGB); // 2. White balance if (cfg.wb_red != 1.0f ¦¦ cfg.wb_blue != 1.0f) { std::vector<cv::Mat> channels; cv::split(rgb, channels); channels[0] *= cfg.wb_blue; // B channels[2] *= cfg.wb_red; // R cv::merge(channels, rgb); } // 3. Denoise (bilateral filter — preserves edges) if (cfg.denoise_strength > 0) { cv::Mat denoised; cv::bilateralFilter(rgb, denoised, cfg.denoise_strength, // d (neighborhood diameter) cfg.denoise_strength * 10.0, // sigmaColor cfg.denoise_strength * 10.0); // sigmaSpace rgb = denoised; } // 4. Lens undistortion (using precomputed remap — ~2ms vs ~8ms for undistort) cv::Mat undistorted; cv::remap(rgb, undistorted, cfg.rectify_map1, cfg.rectify_map2, cv::INTER_LINEAR); return undistorted; } struct EncoderFrame { cv::Mat image; // NV12 or RGB FrameMetadata meta; }; /// ISP thread entry point — consumes from both camera queues, /// distributes to all four outputs. void isp_thread( // Inputs SPSCRing<V4L2Camera::CapturedFrame, 4>& mission_queue, SPSCRing<StereoFrame, 4>& stereo_queue, // Outputs — inter-process ShmRingProducer& shm_perception, // → Perception process ShmRingProducer& shm_slam, // → SLAM process // Outputs — intra-process SPSCRing<EncoderFrame, 4>& encoder_queue, // → Thread 4 SPSCRing<EncoderFrame, 4>& recorder_queue, // → Thread 5 // Control std::atomic<bool>& stop_flag, const ISPConfig& cfg) { spdlog::info("ISP thread started"); while (!stop_flag.load(std::memory_order_relaxed)) { // ── Process mission camera frames ── if (auto frame = mission_queue.try_pop()) { auto rgb = process_mission_frame( frame->data, 1920, 1080, 1920, cfg); // Output 1: To Perception (via shared memory) FrameMetadata meta{ frame->timestamp_ns, frame->sequence, CameraId::MISSION, PixelFormat::RGB24, static_cast<uint32_t>(rgb.cols), static_cast<uint32_t>(rgb.rows), static_cast<uint32_t>(rgb.step), 0, 0.0f, 0.0f }; shm_perception.publish(meta, rgb.data); // Output 3: To encoder EncoderFrame ef; if (cfg.produce_nv12) { cv::cvtColor(rgb, ef.image, cv::COLOR_RGB2YUV_I420); } else { ef.image = rgb; } ef.meta = meta; encoder_queue.try_push(ef); // Output 4: To SSD recorder EncoderFrame rf; rf.image = rgb.clone(); // recorder may be slower — needs own copy rf.meta = meta; recorder_queue.try_push(rf); } // ── Process stereo frames ── if (auto stereo = stereo_queue.try_pop()) { // For SLAM: rectify both images using stereo calibration // (Stereo rectification aligns epipolar lines — required for // dense stereo matching and VIO) // Simplified: just undistort each. Full stereo rectification // needs R, T from stereo calibration. // Output 2: To SLAM (left image via shared memory) // SLAM typically only needs the left image + IMU data FrameMetadata meta_l = stereo->meta_left; meta_l.format = PixelFormat::GRAY8; shm_slam.publish(meta_l, stereo->left_data); } // Prevent busy-spin if both queues are empty // (In practice, at 30fps each camera produces a frame every 33ms // so we only spin briefly between frames) if (mission_queue.available() == 0 && stereo_queue.available() == 0) { std::this_thread::sleep_for(std::chrono::microseconds(100)); } } spdlog::info("ISP thread stopped"); } } // namespace drone::video

6.4 Jetson Hardware ISP Alternative

On NVIDIA Jetson (Orin Nano), you can bypass software ISP entirely using the hardware ISP via libargus / nvargus-daemon:

| Approach | CPU Cost | Latency | Quality | When to Use |
| --- | --- | --- | --- | --- |
| OpenCV software ISP | 15-20% of 1 core | ~5ms @ 1080p | Good (tunable) | Prototyping, non-Jetson platforms |
| Jetson HW ISP (libargus) | ~0% | <1ms | Excellent | Production on Jetson (preferred) |
| GStreamer nvarguscamerasrc | ~0% | <1ms | Excellent | Quick prototyping on Jetson |

Jetson hardware ISP via GStreamer (prototype shortcut)

// GStreamer pipeline for Jetson hardware ISP + capture: // gst-launch-1.0 nvarguscamerasrc sensor-id=0 \ // ! "video/x-raw(memory:NVMM), width=1920, height=1080, framerate=30/1" \ // ! nvvidconv \ // ! "video/x-raw, format=BGRx" \ // ! videoconvert \ // ! appsink // In code, use cv::VideoCapture with GStreamer backend: // cv::VideoCapture cap( // "nvarguscamerasrc sensor-id=0 ! " // "video/x-raw(memory:NVMM), width=1920, height=1080, framerate=30/1 ! " // "nvvidconv ! video/x-raw, format=BGRx ! videoconvert ! appsink", // cv::CAP_GSTREAMER // );

7. Thread 4: H.265 Encode → RTP Stream

7.1 Purpose

Encodes processed frames into H.265/HEVC video, adds forward error correction (FEC), and streams to the Ground Control Station via RTP/UDP over the RF data link. This is the operator's live video feed — latency target is < 100ms glass-to-glass.

7.2 Encoding Pipeline

RGB/NV12 frame (from ISP) │ ▼ ┌───────────────────┐ │ H.265 ENCODER │ NVENC (Jetson HW) or x265 (software fallback) │ │ Bitrate: 2-15 Mbps (adaptive) │ Produces NAL │ Keyframe interval: 1 per second (or on demand) │ units (compressed) │ Encoding latency: < 5ms (HW), ~20ms (SW) └──────┬─────────────┘ ▼ ┌───────────────────┐ │ FEC ENCODER │ Reed-Solomon or XOR-based │ │ Adds K redundancy packets per N data packets │ e.g., 20 data │ Receiver can reconstruct any K losses from N+K │ + 5 FEC = 25 │ 25% overhead, tolerates 20% packet loss └──────┬─────────────┘ ▼ ┌───────────────────┐ │ RTP PACKETIZER │ Split NAL units into RTP packets (≤ 1400 bytes) │ │ Add RTP header (sequence number, timestamp, SSRC) │ RFC 7798 │ H.265 RTP payload format └──────┬─────────────┘ ▼ ┌───────────────────┐ │ UDP SOCKET │ sendmmsg() for batch sending │ │ Destination: GCS IP (or broadcast) │ Non-blocking │ If send buffer full → drop oldest, not newest └──────┬─────────────┘ ▼ RF DATA LINK → GCS

7.3 Adaptive Bitrate Control

The RF link quality varies during flight (range, interference, obstacles). The encoder must adapt:

| Link Quality | RSSI (dBm) | Bitrate | Resolution | FPS | FEC Overhead |
| --- | --- | --- | --- | --- | --- |
| Excellent | > -60 | 15 Mbps | 1920×1080 | 30 | 10% |
| Good | -60 to -75 | 8 Mbps | 1920×1080 | 30 | 15% |
| Fair | -75 to -85 | 4 Mbps | 1280×720 | 30 | 20% |
| Poor | -85 to -90 | 2 Mbps | 1280×720 | 15 | 25% |
| Critical | < -90 | 500 Kbps | 640×360 | 10 | 30% |

The link quality signal comes from the Comms process (Process 5) via the shared data bus. The encoder thread subscribes to link-quality updates and adjusts on-the-fly using the encoder's dynamic bitrate API.

7.4 Implementation

src/video/encoder_thread.cpp — H.265 encode + RTP streaming

// file: src/video/encoder_thread.cpp #include "video/frame.h" #include "video/spsc_ring.h" #include <spdlog/spdlog.h> #include <atomic> #include <chrono> #include <sys/socket.h> #include <netinet/in.h> #include <arpa/inet.h> #include <cstring> namespace drone::video { /// Simplified RTP packet header (RFC 3550) struct RTPHeader { uint8_t version_flags; // V=2, P=0, X=0, CC=0 → 0x80 uint8_t payload_type; // 96 for dynamic H.265 uint16_t sequence; // monotonic, wraps at 65535 uint32_t timestamp; // 90kHz clock uint32_t ssrc; // source identifier }; /// Abstraction over NVENC / x265 / FFmpeg encoder class H265Encoder { public: struct Config { uint32_t width; uint32_t height; uint32_t fps; uint32_t bitrate_kbps; uint32_t keyframe_interval; // frames between I-frames bool use_hw; // true = NVENC, false = x265 }; explicit H265Encoder(const Config& cfg) : cfg_(cfg) { // In real implementation: // If use_hw: initialize NVENC via NvEncodeAPI // Else: initialize x265_encoder_open() spdlog::info("H.265 encoder: {}x{} @ {} kbps ({} encoder)", cfg.width, cfg.height, cfg.bitrate_kbps, cfg.use_hw ? "NVENC HW" : "x265 SW"); } struct EncodedNAL { const uint8_t* data; size_t size; bool is_keyframe; }; /// Encode one frame. Returns encoded NAL units. /// In production, this calls NvEncEncodePicture() or x265_encoder_encode() std::vector<EncodedNAL> encode(const uint8_t* frame_data) { // Placeholder — real implementation feeds frame to encoder // and collects output NAL units std::vector<EncodedNAL> nals; // ... encoder produces NAL units ... return nals; } /// Change bitrate on-the-fly (for adaptive streaming) void set_bitrate(uint32_t new_kbps) { if (new_kbps != cfg_.bitrate_kbps) { cfg_.bitrate_kbps = new_kbps; // Real implementation: NvEncReconfigureEncoder() or x265 reconfig spdlog::info("Encoder bitrate changed to {} kbps", new_kbps); } } private: Config cfg_; }; /// Simple UDP sender for RTP packets class RTPSender { public: RTPSender(const char* dest_ip, uint16_t dest_port) { sock_ = socket(AF_INET, SOCK_DGRAM ¦ SOCK_NONBLOCK, 0); if (sock_ < 0) throw std::runtime_error("socket() failed"); memset(&dest_, 0, sizeof(dest_)); dest_.sin_family = AF_INET; dest_.sin_port = htons(dest_port); inet_pton(AF_INET, dest_ip, &dest_.sin_addr); // Increase send buffer to handle bursts int buf_size = 2 * 1024 * 1024; // 2MB setsockopt(sock_, SOL_SOCKET, SO_SNDBUF, &buf_size, sizeof(buf_size)); } ~RTPSender() { if (sock_ >= 0) close(sock_); } void send_nal(const uint8_t* nal_data, size_t nal_size, uint32_t rtp_ts) { // Fragment NAL into RTP packets (max 1400 bytes payload) constexpr size_t MAX_PAYLOAD = 1400; if (nal_size <= MAX_PAYLOAD) { // Single NAL unit packet send_rtp_packet(nal_data, nal_size, rtp_ts, true); } else { // Fragmentation Unit (FU) — RFC 7798 Section 4.4.3 size_t offset = 2; // skip NAL header (2 bytes for H.265) bool first = true; while (offset < nal_size) { size_t chunk = std::min(MAX_PAYLOAD - 3, nal_size - offset); bool last = (offset + chunk >= nal_size); // Build FU header: FU indicator + FU header uint8_t fu_packet[MAX_PAYLOAD + 3]; // ... construct FU packet per RFC 7798 ... send_rtp_packet(fu_packet, chunk + 3, rtp_ts, last); offset += chunk; first = false; } } } private: void send_rtp_packet(const uint8_t* payload, size_t len, uint32_t timestamp, bool marker) { uint8_t packet[1500]; RTPHeader hdr{}; hdr.version_flags = 0x80; hdr.payload_type = 96 ¦ (marker ? 0x80 : 0); hdr.sequence = htons(seq_++); hdr.timestamp = htonl(timestamp); hdr.ssrc = htonl(0x44524F4E); // "DRON" memcpy(packet, &hdr, sizeof(hdr)); memcpy(packet + sizeof(hdr), payload, len); sendto(sock_, packet, sizeof(hdr) + len, 0, (sockaddr*)&dest_, sizeof(dest_)); } int sock_ = -1; sockaddr_in dest_{}; uint16_t seq_ = 0; }; /// Encoder thread entry point void encoder_thread( SPSCRing<EncoderFrame, 4>& input_queue, // from ISP thread std::atomic<bool>& stop_flag, std::atomic<uint32_t>& target_bitrate_kbps, // set by adaptive controller const char* gcs_ip = "192.168.1.100", uint16_t gcs_port = 5600) { H265Encoder encoder(H265Encoder::Config{ 1920, 1080, 30, 8000, 30, true // NVENC }); RTPSender sender(gcs_ip, gcs_port); auto last_bitrate_check = std::chrono::steady_clock::now(); uint64_t frames_encoded = 0; while (!stop_flag.load(std::memory_order_relaxed)) { auto frame_opt = input_queue.try_pop(); if (!frame_opt) { std::this_thread::sleep_for(std::chrono::microseconds(500)); continue; } auto& ef = *frame_opt; // Encode frame auto nals = encoder.encode(ef.image.data); // RTP timestamp: 90kHz clock = (frame_ns / 1000000) * 90 uint32_t rtp_ts = static_cast<uint32_t>( (ef.meta.timestamp_ns / 1'000'000) * 90); // Send each NAL unit for (auto& nal : nals) { sender.send_nal(nal.data, nal.size, rtp_ts); } ++frames_encoded; // Check if bitrate needs adjusting (every 500ms) auto now = std::chrono::steady_clock::now(); if (now - last_bitrate_check > std::chrono::milliseconds(500)) { uint32_t target = target_bitrate_kbps.load(std::memory_order_relaxed); encoder.set_bitrate(target); last_bitrate_check = now; } } spdlog::info("Encoder stopped. {} frames encoded", frames_encoded); } } // namespace drone::video

7.5 Available Encoding Libraries

| Library | Type | License | Latency | Notes |
| --- | --- | --- | --- | --- |
| NVENC (NvEncodeAPI) | Hardware | NVIDIA proprietary | < 5ms | Best for Jetson. Zero CPU. Requires CUDA toolkit. |
| x265 (libx265) | Software | GPL v2 | 15-25ms | Best SW encoder. Good for non-NVIDIA. GPL = license risk. |
| OpenH264 | Software | BSD | 10-20ms | H.264 only (not H.265). Cisco-backed, BSD licensed. |
| FFmpeg/libavcodec | Both | LGPL/GPL | Varies | Wraps NVENC + x265 + others. Most flexible. |
| GStreamer | Framework | LGPL | Varies | Pipeline-based. nvv4l2h265enc for Jetson HW. |
| VA-API (Intel) | Hardware | MIT | < 5ms | For Intel GPUs (e.g., NUC-based companion). |

8. Thread 5: SSD Recording

8.1 Purpose

Records mission footage to onboard NVMe SSD for post-mission analysis, evidence archive, and ML training data collection. This thread must never block the live pipeline — it runs at lower priority and drops frames if disk I/O stalls.

8.2 Design Decisions

| Decision | Choice | Why |
| --- | --- | --- |
| Container format | MKV (Matroska) | Resilient to truncation (crash recovery), supports H.265 + metadata |
| Write method | O_DIRECT + aligned buffers | Bypass page cache — no memory pressure on other processes |
| Write thread | Dedicated (this thread) | Disk write stalls (up to 50ms on NVMe flush) never block capture |
| Circular buffer | Optional — overwrite oldest segment | For long missions on limited SSD capacity |
| Timestamps | Embedded per-frame + sidecar .csv | Frame-accurate sync with telemetry for post-processing |

8.3 Implementation

src/video/recorder_thread.cpp — SSD recording thread

// file: src/video/recorder_thread.cpp #include "video/frame.h" #include "video/spsc_ring.h" #include <spdlog/spdlog.h> #include <atomic> #include <fstream> #include <filesystem> #include <chrono> namespace drone::video { /// Simple recording session — writes encoded frames to MKV container. /// In production, use libavformat (FFmpeg) for proper MKV muxing. class MissionRecorder { public: explicit MissionRecorder(const std::string& output_dir) { namespace fs = std::filesystem; fs::create_directories(output_dir); // Generate filename from timestamp auto now = std::chrono::system_clock::now(); auto t = std::chrono::system_clock::to_time_t(now); char buf[64]; std::strftime(buf, sizeof(buf), "%Y%m%d_%H%M%S", std::localtime(&t)); video_path_ = output_dir + "/" + buf + "_mission.mkv"; metadata_path_ = output_dir + "/" + buf + "_metadata.csv"; // Open metadata sidecar metadata_file_.open(metadata_path_); metadata_file_ << "frame_seq,timestamp_ns,exposure_us,gain_db\n"; // In production: open MKV via avformat_alloc_output_context2() spdlog::info("Recording to: {}", video_path_); } ~MissionRecorder() { // Flush and finalize container metadata_file_.close(); spdlog::info("Recording finalized: {} frames, {:.1f} MB", frame_count_, bytes_written_ / (1024.0 * 1024.0)); } void write_frame(const EncoderFrame& ef) { // In production: use av_write_frame() to mux into MKV // Here showing the metadata sidecar write: metadata_file_ << ef.meta.sequence_number << "," << ef.meta.timestamp_ns << "," << ef.meta.exposure_us << "," << ef.meta.gain_db << "\n"; // Track stats bytes_written_ += ef.image.total() * ef.image.elemSize(); ++frame_count_; // Flush metadata every 100 frames (not every frame — too slow) if (frame_count_ % 100 == 0) { metadata_file_.flush(); } } /// Check if disk space is running low bool disk_space_ok() const { namespace fs = std::filesystem; auto info = fs::space("/data"); size_t free_mb = info.available / (1024 * 1024); return free_mb > 500; // keep at least 500MB free } private: std::string video_path_; std::string metadata_path_; std::ofstream metadata_file_; uint64_t frame_count_ = 0; uint64_t bytes_written_ = 0; }; /// Recorder thread entry point void recorder_thread( SPSCRing<EncoderFrame, 4>& input_queue, // from ISP thread std::atomic<bool>& stop_flag, const std::string& recording_dir = "/data/recordings") { MissionRecorder recorder(recording_dir); while (!stop_flag.load(std::memory_order_relaxed)) { auto frame_opt = input_queue.try_pop(); if (!frame_opt) { std::this_thread::sleep_for(std::chrono::milliseconds(1)); continue; } // Check disk space periodically if (!recorder.disk_space_ok()) { spdlog::warn("SSD low on space — dropping recording frames"); // Could also implement circular buffer: delete oldest segment continue; } recorder.write_frame(*frame_opt); } // Destructor will finalize the recording spdlog::info("Recorder thread stopped"); } } // namespace drone::video

9. Inter-Thread Communication

All threads within Process 1 communicate via lock-free SPSC queues. No mutexes, no condition variables, no blocking. If a consumer is slow, the producer drops the frame rather than blocking the pipeline.

Thread 1 (Mission Cam) ──SPSC──► Thread 3 (ISP) ──SPSC──► Thread 4 (Encoder) │ Thread 2 (Stereo Cam) ──SPSC──► │ ──SPSC──► Thread 5 (Recorder) │ ──SHM Ring──► PERCEPTION (Process 2) │ ──SHM Ring──► SLAM (Process 3)

9.1 Queue Sizing

| Queue | Size | Frame Size | Memory | Drop Policy |
| --- | --- | --- | --- | --- |
| Mission Cam → ISP | 4 slots | ~6 MB (raw) | 24 MB | Drop newest (producer drops) |
| Stereo → ISP | 4 slots | ~600 KB (gray pair) | 2.4 MB | Drop newest |
| ISP → Encoder | 4 slots | ~3 MB (NV12 1080p) | 12 MB | Drop newest |
| ISP → Recorder | 4 slots | ~6 MB (RGB 1080p) | 24 MB | Drop newest (recording is best-effort) |

4 slots is chosen because it gives 4/30 ≈ 133ms of buffer at 30fps. If any consumer thread stalls for more than 133ms, frames are dropped. This is intentional — we never want backpressure to stall the camera capture.

9.2 Adaptive Bitrate Feedback (The One Exception)

There is one "reverse" data flow: the Comms process reports RF link quality, which the encoder thread reads to adjust bitrate. This is done via a simple std::atomic<uint32_t> target_bitrate_kbps — no queue needed, just the latest value.

10. Inter-Process Communication (to Perception & SLAM)

10.1 Shared Memory Regions

| SHM Name | Purpose | Slots | Slot Size | Total | Consumer |
| --- | --- | --- | --- | --- | --- |
| /drone_mission_cam | Mission camera RGB frames | 4 | ~6.2 MB (1920×1080×3 + meta) | ~25 MB | Perception |
| /drone_stereo_cam | Stereo left frame (grayscale) | 4 | ~307 KB (640×480×1 + meta) | ~1.2 MB | SLAM |

10.2 Lifecycle

The Video/Capture process CREATES the shared memory regions on startup (shm_open with O_CREAT). Consumers OPEN them (shm_open without O_CREAT). If consumers start before the producer, they retry with exponential backoff until the SHM region appears.

Consumer startup pattern with retry

// Consumer startup pattern (in Perception or SLAM process): ShmRingConsumer* connect_to_video(const char* shm_name, int max_retries = 50) { for (int i = 0; i < max_retries; ++i) { try { return new ShmRingConsumer(shm_name); } catch (const std::exception& e) { spdlog::warn("Waiting for {} (attempt {}/{}): {}", shm_name, i+1, max_retries, e.what()); std::this_thread::sleep_for( std::chrono::milliseconds(100 * std::min(i + 1, 10))); } } throw std::runtime_error(std::string("Cannot connect to ") + shm_name); }

10.3 What Happens When Video/Capture Restarts

If this process crashes and systemd restarts it:

shm_unlink() is called in the destructor (or by the new instance before re-creating)

Consumers detect stale data via magic number check + write_idx not advancing

Consumer reconnects by closing old mapping and re-opening the new SHM region

Frame sequence numbers will have a gap — consumers handle this gracefully

11. Startup, Shutdown & Error Handling

11.1 Process Main — Thread Orchestration

src/video/main.cpp — Process entry point, thread orchestration

// file: src/video/main.cpp #include "video/frame.h" #include "video/spsc_ring.h" #include "video/shm_ring.h" #include <thread> #include <atomic> #include <signal.h> #include <spdlog/spdlog.h> #include <systemd/sd-daemon.h> // for sd_notify namespace drone::video { // Forward declarations (defined in individual thread source files) void mission_cam_thread(SPSCRing<V4L2Camera::CapturedFrame, 4>&, std::atomic<bool>&, const char*); void stereo_cam_thread(SPSCRing<StereoFrame, 4>&, std::atomic<bool>&, const char*, const char*); void isp_thread(SPSCRing<V4L2Camera::CapturedFrame, 4>&, SPSCRing<StereoFrame, 4>&, ShmRingProducer&, ShmRingProducer&, SPSCRing<EncoderFrame, 4>&, SPSCRing<EncoderFrame, 4>&, std::atomic<bool>&, const ISPConfig&); void encoder_thread(SPSCRing<EncoderFrame, 4>&, std::atomic<bool>&, std::atomic<uint32_t>&, const char*, uint16_t); void recorder_thread(SPSCRing<EncoderFrame, 4>&, std::atomic<bool>&, const std::string&); } // namespace drone::video static std::atomic<bool> g_stop{false}; void signal_handler(int) { g_stop.store(true, std::memory_order_relaxed); } int main(int argc, char* argv[]) { spdlog::set_level(spdlog::level::info); spdlog::info("video-capture process starting (PID {})", getpid()); // Register signal handlers for graceful shutdown signal(SIGTERM, signal_handler); signal(SIGINT, signal_handler); using namespace drone::video; // ── 1. Create shared data structures ── // Intra-process queues (thread ↔ thread) SPSCRing<V4L2Camera::CapturedFrame, 4> mission_cam_queue; SPSCRing<StereoFrame, 4> stereo_cam_queue; SPSCRing<EncoderFrame, 4> encoder_queue; SPSCRing<EncoderFrame, 4> recorder_queue; // Inter-process shared memory (this process → Perception / SLAM) ShmRingProducer shm_perception("/drone_mission_cam", 4, 1920, 1080, 3); ShmRingProducer shm_slam("/drone_stereo_cam", 4, 640, 480, 1); // Adaptive bitrate target (set by comms process via data bus) std::atomic<uint32_t> target_bitrate_kbps{8000}; // Load ISP calibration config ISPConfig isp_cfg; // isp_cfg = load_config("/opt/drone/config/isp.yaml"); // ── 2. Launch threads (order matters — start consumers before producers) ── std::thread t_encoder(encoder_thread, std::ref(encoder_queue), std::ref(g_stop), std::ref(target_bitrate_kbps), "192.168.1.100", 5600); std::thread t_recorder(recorder_thread, std::ref(recorder_queue), std::ref(g_stop), "/data/recordings"); std::thread t_isp(isp_thread, std::ref(mission_cam_queue), std::ref(stereo_cam_queue), std::ref(shm_perception), std::ref(shm_slam), std::ref(encoder_queue), std::ref(recorder_queue), std::ref(g_stop), std::cref(isp_cfg)); std::thread t_mission_cam(mission_cam_thread, std::ref(mission_cam_queue), std::ref(g_stop), "/dev/video0"); std::thread t_stereo_cam(stereo_cam_thread, std::ref(stereo_cam_queue), std::ref(g_stop), "/dev/video2", "/dev/video3"); // ── 3. Notify systemd we're ready ── sd_notify(0, "READY=1"); spdlog::info("All threads started — notified systemd READY"); // ── 4. Main thread: watchdog loop ── while (!g_stop.load(std::memory_order_relaxed)) { // Send watchdog keepalive to systemd sd_notify(0, "WATCHDOG=1"); // Monitor thread health (check if queues are flowing) // ... log stats every 10 seconds ... std::this_thread::sleep_for(std::chrono::seconds(2)); } // ── 5. Graceful shutdown ── spdlog::info("Shutting down..."); sd_notify(0, "STOPPING=1"); // Threads check g_stop and exit their loops t_mission_cam.join(); t_stereo_cam.join(); t_isp.join(); t_encoder.join(); t_recorder.join(); // Clean up shared memory shm_unlink("/drone_mission_cam"); shm_unlink("/drone_stereo_cam"); spdlog::info("video-capture process stopped cleanly"); return 0; }

11.2 Error Handling Strategy

| Error | Severity | Response |
| --- | --- | --- |
| Camera device not found | FATAL | Log error, exit process. systemd restarts. |
| Camera poll timeout (> 1s) | CRITICAL | Assume hardware failure. Close and re-open device. If fails 3x, exit. |
| SPSC queue full (ISP slow) | WARNING | Drop frame, increment counter, log every 100th drop. |
| SHM publish fails (consumer slow) | WARNING | Drop frame. Consumer will see gap in sequence numbers. |
| Encoder error | ERROR | Skip frame. If persistent (10 consecutive), restart encoder. |
| Disk write error | WARNING | Stop recording, continue streaming. Recording is best-effort. |
| SSD full | WARNING | Switch to circular mode (delete oldest) or stop recording. |
| Signal SIGTERM | NORMAL | Set stop flag, join all threads, clean up SHM, exit 0. |

12. systemd Service Configuration

systemd service unit file

# /etc/systemd/system/video-capture.service [Unit] Description=Drone Video Capture & Streaming After=network.target # Start after hardware is ready After=dev-video0.device Wants=dev-video0.device [Service] Type=notify # Process calls sd_notify(READY=1) ExecStart=/opt/drone/bin/video-capture User=drone Group=video # Access to /dev/videoX without root # Resource limits CPUAffinity=2 3 # Pin to cores 2-3 MemoryMax=512M MemoryHigh=384M CPUQuota=200% # Max 2 cores worth of CPU # Restart policy Restart=always RestartSec=200ms # Restart within 200ms StartLimitIntervalSec=60 StartLimitBurst=10 # Max 10 restarts per minute # Watchdog: process must send WATCHDOG=1 every 5s or it's killed WatchdogSec=5 # Security hardening ProtectSystem=strict PrivateTmp=true ReadWritePaths=/data/recordings # Only writable path (for SSD recording) NoNewPrivileges=true # Shared memory permissions ReadWritePaths=/dev/shm # For shm_open() # Namespace for device access PrivateDevices=false # Need access to /dev/videoX and /dev/nvhost-* DeviceAllow=/dev/video0 rw DeviceAllow=/dev/video2 rw DeviceAllow=/dev/video3 rw [Install] WantedBy=multi-user.target

Key points:

Type=notify means systemd waits for READY=1 before considering the service started. Dependent services (Perception, SLAM) won't start until video-capture is ready.

WatchdogSec=5 means the main thread must call sd_notify("WATCHDOG=1") every 5 seconds or systemd kills and restarts the process.

CPUAffinity=2 3 pins this process to cores 2-3, leaving cores 0-1 for SLAM (which is CPU-intensive during loop closure).

13. Available Libraries & Algorithms

13.1 Camera Capture

| Library | License | Language | Platform | Notes |
| --- | --- | --- | --- | --- |
| V4L2 (kernel API) | GPL (kernel) | C | Any Linux | Direct ioctl — lowest latency, full control |
| libv4l2 | LGPL | C | Any Linux | Wraps V4L2 with format conversion (useful for prototyping) |
| libargus (NVIDIA) | Proprietary | C++ | Jetson only | Hardware ISP, zero-copy to GPU. Best for production on Jetson. |
| librealsense2 | Apache 2.0 | C++ | Cross-platform | Intel RealSense cameras. Built-in stereo sync. |
| DepthAI (Luxonis) | MIT | C++/Python | Cross-platform | OAK-D cameras. On-chip stereo + neural inference. |
| OpenCV VideoCapture | Apache 2.0 | C++ | Cross-platform | Wraps V4L2/GStreamer. Easy but less control. |

13.2 Image Signal Processing (ISP)

| Algorithm | Library | License | Notes |
| --- | --- | --- | --- |
| Debayer (demosaic) | OpenCV cvtColor(COLOR_BayerRG2RGB) | Apache 2.0 | Bilinear by default. Use EA for better quality. |
| White balance | OpenCV (manual) or libcamera | Apache 2.0 / LGPL | Grey-world or custom gains |
| Denoise — Bilateral | OpenCV bilateralFilter() | Apache 2.0 | Edge-preserving. Good balance of speed/quality. |
| Denoise — NLM | OpenCV fastNlMeansDenoising() | Apache 2.0 | Better quality, slower (~15ms @ 1080p) |
| Denoise — BM3D | bm3d library | Various | Best quality, too slow for real-time at 1080p |
| Lens undistortion | OpenCV undistort() / remap() | Apache 2.0 | remap() with precomputed maps is ~4x faster |
| Jetson HW ISP | libargus | Proprietary | All ISP steps in hardware. ~0% CPU. |

13.3 Video Encoding

| Library | License | H.265? | HW Accel | Notes |
| --- | --- | --- | --- | --- |
| NVENC (NvEncodeAPI) | NVIDIA proprietary | Yes | NVIDIA GPU | Best for Jetson. < 5ms encode latency. |
| x265 | GPL v2 | Yes | CPU only | Best SW encoder. GPL = license concern for proprietary drone. |
| Kvazaar | LGPL v2.1 | Yes | CPU only | LGPL-friendly H.265. Slightly lower quality than x265. |
| FFmpeg | LGPL/GPL | Via backends | All | Wraps NVENC, x265, etc. Most flexible. |
| GStreamer | LGPL | Via plugins | All | Pipeline-based. nvv4l2h265enc on Jetson. |
| OpenH264 | BSD | No (H.264) | CPU | BSD license. H.264 only — use if H.265 is not needed. |

13.4 RTP / Streaming

| Library | License | Notes |
| --- | --- | --- |
| live555 | LGPL | Full RTSP/RTP stack. Used in VLC. Mature but complex API. |
| libsrtp | BSD | Secure RTP (SRTP). Add encryption if needed. |
| GStreamer rtpbin | LGPL | High-level RTP handling with jitter buffers. |
| Custom (raw UDP) | N/A | Smallest footprint. What we showed in code above. |

13.5 Forward Error Correction (FEC)

| Library | License | Type | Notes |
| --- | --- | --- | --- |
| libfec | LGPL | Reed-Solomon | Classic RS codes. Well-tested. |
| OpenFEC | CeCILL-C (LGPL-like) | LDPC / Reed-Solomon | More algorithms. Used in DVB. |
| Wirehair | BSD | Fountain code | Rateless — can generate unlimited repair symbols. |
| CM256 | BSD | Cauchy RS | Optimized for small blocks. Good for RTP. |
| Custom XOR | N/A | Simple XOR interleave | Simplest. Recovers 1 lost packet per group of N. |

13.6 Recording / Container

| Library | License | Format | Notes |
| --- | --- | --- | --- |
| libavformat (FFmpeg) | LGPL | MKV, MP4, etc. | Most complete. Handles muxing + timestamps. |
| libmatroska | LGPL | MKV | MKV-specific. Lower level than FFmpeg. |
| GStreamer matroskamux | LGPL | MKV | Pipeline element. Easy if using GStreamer already. |

14. Build System (CMake)

CMakeLists.txt for Process 1

# file: process1_video_capture/CMakeLists.txt cmake_minimum_required(VERSION 3.16) project(video-capture LANGUAGES CXX) set(CMAKE_CXX_STANDARD 17) set(CMAKE_CXX_STANDARD_REQUIRED ON) set(CMAKE_EXPORT_COMPILE_COMMANDS ON) # For clangd/IDE support # ── Dependencies ── find_package(OpenCV REQUIRED COMPONENTS core imgproc calib3d) find_package(PkgConfig REQUIRED) pkg_check_modules(SPDLOG REQUIRED spdlog) # Optional: V4L2 (usually part of linux-headers, not a separate package) # Optional: NVENC (NVIDIA Video Codec SDK) # ── Library: video-capture core ── add_library(video_capture_lib STATIC src/video/mission_cam_thread.cpp src/video/stereo_cam_thread.cpp src/video/isp_thread.cpp src/video/encoder_thread.cpp src/video/recorder_thread.cpp ) target_include_directories(video_capture_lib PUBLIC ${CMAKE_CURRENT_SOURCE_DIR}/include ${OpenCV_INCLUDE_DIRS} ${SPDLOG_INCLUDE_DIRS} ) target_link_libraries(video_capture_lib PUBLIC ${OpenCV_LIBS} ${SPDLOG_LIBRARIES} rt # for shm_open, shm_unlink pthread # for std::thread systemd # for sd_notify ) # ── Executable: video-capture ── add_executable(video-capture src/video/main.cpp) target_link_libraries(video-capture PRIVATE video_capture_lib) # ── Install ── install(TARGETS video-capture DESTINATION /opt/drone/bin) install(FILES config/isp.yaml DESTINATION /opt/drone/config) # ── Tests ── enable_testing() add_executable(test_spsc_ring tests/test_spsc_ring.cpp) target_include_directories(test_spsc_ring PRIVATE include) target_link_libraries(test_spsc_ring pthread) add_test(NAME SPSCRingTest COMMAND test_spsc_ring) add_executable(test_shm_ring tests/test_shm_ring.cpp) target_include_directories(test_shm_ring PRIVATE include) target_link_libraries(test_shm_ring pthread rt) add_test(NAME ShmRingTest COMMAND test_shm_ring)

15. Implementation Roadmap

Suggested order for implementing Process 1, starting from the simplest testable component and building up:

Phase 1: Data Structures (Week 1)

| Task | Files | Test |
| --- | --- | --- |
| 1.1 Implement FrameMetadata | include/video/frame.h | Unit test: serialize/deserialize |
| 1.2 Implement SPSC ring buffer | include/video/spsc_ring.h | Unit test: push/pop, full, empty, wrap-around |
| 1.3 Stress test SPSC | tests/test_spsc_ring.cpp | Two threads, 10M frames, verify no corruption |
| 1.4 Implement SHM ring buffer | include/video/shm_ring.h | Unit test: producer/consumer in two processes |
| 1.5 Cross-process SHM test | tests/test_shm_ring.cpp | fork(), producer in parent, consumer in child |

Phase 2: Camera Capture (Week 2)

| Task | Files | Test |
| --- | --- | --- |
| 2.1 V4L2Camera class | src/video/mission_cam_thread.cpp | Capture 100 frames from /dev/video0, verify timestamps |
| 2.2 Mission cam thread | src/video/mission_cam_thread.cpp | Run thread, verify SPSC queue receives frames |
| 2.3 Stereo cam thread | src/video/stereo_cam_thread.cpp | Verify left/right sync < 1ms |
| 2.4 Camera failure handling |  | Unplug USB camera during capture, verify clean recovery |

Phase 3: ISP Pipeline (Week 3)

| Task | Files | Test |
| --- | --- | --- |
| 3.1 Camera calibration | scripts/calibrate_camera.py | Print checkerboard, run OpenCV calibration, save YAML |
| 3.2 Debayer + white balance | src/video/isp_thread.cpp | Visual: compare raw vs processed image |
| 3.3 Lens undistortion | src/video/isp_thread.cpp | Verify straight lines are straight after undistort |
| 3.4 Full ISP → SHM output |  | Read frames from SHM in a test consumer process |
| 3.5 Performance profiling |  | Measure ISP latency: target < 5ms @ 1080p |

Phase 4: Encoding & Streaming (Week 4)

| Task | Files | Test |
| --- | --- | --- |
| 4.1 H.265 encoder wrapper | src/video/encoder_thread.cpp | Encode test frames, verify output is valid H.265 |
| 4.2 RTP packetizer | src/video/encoder_thread.cpp | Send to localhost, capture with Wireshark, verify RTP headers |
| 4.3 GCS receiver test |  | Play stream in VLC: vlc rtp://@:5600 |
| 4.4 Adaptive bitrate |  | Simulate link quality changes, verify bitrate adapts |
| 4.5 FEC implementation |  | Drop 20% of UDP packets, verify video reconstructs |

Phase 5: Recording & Integration (Week 5)

| Task | Files | Test |
| --- | --- | --- |
| 5.1 MKV recorder | src/video/recorder_thread.cpp | Record 60s, play back in VLC, verify timestamps |
| 5.2 Metadata sidecar |  | Verify CSV matches frame count and timestamps |
| 5.3 Disk space monitoring |  | Fill disk to 500MB free, verify recording stops gracefully |
| 5.4 Process main + thread orchestration | src/video/main.cpp | Start all 5 threads, verify frame flow end-to-end |
| 5.5 systemd integration |  | Install service, test start/stop/restart/watchdog |

Phase 6: Hardening (Week 6)

| Task | Files | Test |
| --- | --- | --- |
| 6.1 Camera hot-unplug test |  | Unplug during capture, verify restart within 200ms |
| 6.2 Consumer crash test |  | Kill Perception process, verify Video continues, SHM re-created |
| 6.3 Memory leak check |  | Run under Valgrind for 1 hour, verify 0 leaks |
| 6.4 CPU profiling |  | Run under perf, optimize hot paths |
| 6.5 24-hour soak test |  | Run for 24 hours, verify no drift, no leaks, stable FPS |

After completing Phase 6, Process 1 (Video/Capture) is production-ready. You can then move to Process 2 (Perception) which consumes frames from the /drone_mission_cam shared memory ring buffer.

16. File Tree Summary

Complete file tree for Process 1

process1_video_capture/ ├── CMakeLists.txt ├── config/ │ └── isp.yaml # Camera calibration + ISP parameters ├── include/ │ └── video/ │ ├── frame.h # FrameMetadata, PixelFormat, CameraId │ ├── spsc_ring.h # Lock-free SPSC ring (intra-process) │ └── shm_ring.h # Shared memory ring (inter-process) ├── src/ │ └── video/ │ ├── main.cpp # Process entry point, thread orchestration │ ├── mission_cam_thread.cpp # Thread 1: Mission camera V4L2 capture │ ├── stereo_cam_thread.cpp # Thread 2: Stereo camera V4L2 capture │ ├── isp_thread.cpp # Thread 3: ISP (debayer, WB, undistort) │ ├── encoder_thread.cpp # Thread 4: H.265 encode + RTP stream │ └── recorder_thread.cpp # Thread 5: SSD recording (MKV) ├── tests/ │ ├── test_spsc_ring.cpp # SPSC ring buffer unit tests │ ├── test_shm_ring.cpp # Shared memory ring buffer tests │ └── test_isp_pipeline.cpp # ISP pipeline visual verification └── scripts/ └── calibrate_camera.py # OpenCV camera calibration script