Process 3: SLAM / VIO Navigation

Detailed Implementation Guide

Target Platform: NVIDIA Jetson Orin Nano · C++17 · CMake

# Table of Contents

- 1. Process Overview & Role in the Stack

- 2. Thread Architecture Diagram

- 3. Thread 1 – Visual Frontend

- 4. Thread 2 – IMU Reader / Pre-integrator

- 5. Thread 3 – Backend Optimiser (Graph SLAM)

- 6. Thread 4 – Loop Closure Detector

- 7. Thread 5 – Map Manager

- 8. Inter-Thread Communication (Lock-Free Queues)

- 9. IPC – Receiving Frames from Process 1

- 10. IPC – Publishing Pose to Process 4 & 5

- 11. Coordinate Frames & Transformations

- 12. Available SLAM/VIO Libraries & Algorithms

- 13. Sensor Configuration & Calibration

- 14. Performance Tuning for Jetson Orin Nano

- 15. C++ Concepts Reference

- 16. systemd Service Configuration

- 17. CMakeLists.txt

- 18. Implementation Roadmap

# 1. Process Overview & Role in the Stack

Process 3 (slam_vio_nav) is the spatial-intelligence core of the companion computer. It continuously estimates the drone's 6-DOF pose (position + orientation) in a world-fixed reference frame by fusing visual feature tracks from one or more cameras with inertial measurements from the IMU. The resulting pose is consumed by Process 4 (Mission Planner) for path planning and by Process 5 (Comms) for telemetry downlink to the GCS.

Unlike a pure VIO system that only performs odometry (open-loop integration), this process also maintains a sparse 3-D map, detects loop closures, and runs global graph optimisation to correct accumulated drift – making it a full Visual-Inertial SLAM pipeline.

## Responsibilities

- Receive rectified/undistorted greyscale frames from Process 1 via shared memory

- Read raw IMU data (accelerometer + gyroscope) at ≥200 Hz

- Track visual features (ORB / FAST / SuperPoint) across consecutive frames

- Pre-integrate IMU measurements between keyframes

- Maintain a sliding-window or full pose graph and solve via nonlinear optimisation

- Detect loop closures using bag-of-words or learned descriptors

- Publish T_world_body (SE(3) pose) plus covariance to downstream processes

- Persist and reload maps for re-localisation on revisit

## Input / Output Summary

| Direction | Data | Source / Sink | Transport |
| --- | --- | --- | --- |
| IN | Greyscale frames (640×480 @ 30 Hz) | Process 1 (Video/Capture) | Shared memory ring buffer |
| IN | IMU accel + gyro (≥200 Hz) | SPI/I²C driver or ROS2 topic | Direct read / SPSC queue |
| IN | Barometer altitude (optional) | I²C driver | SPSC queue |
| OUT | T_world_body (SE(3) + covariance) | Process 4 (Mission Planner) | Shared memory slot |
| OUT | T_world_body (SE(3)) | Process 5 (Comms → GCS) | Shared memory slot |
| OUT | Sparse 3-D map points (optional) | Process 4 for obstacle checks | Shared memory / file |
| OUT | Health / heartbeat | Process 7 (System Monitor) | sd_notify + shared flag |

# 2. Thread Architecture Diagram

Process 3 is decomposed into five dedicated threads, each pinned to a specific CPU core to maximise cache locality and minimise scheduling jitter. Threads communicate exclusively through lock-free SPSC queues.

┌─────────────────────────────────────────────────────────────────────┐

│                    Process 3: slam_vio_nav                         │

│                                                                     │

│  ┌──────────────┐    SPSC     ┌────────────────────┐               │

│  │  Thread 1    │───────────▶│   Thread 3          │               │

│  │  Visual      │  tracked   │   Backend           │               │

│  │  Frontend    │  features  │   Optimiser         │               │

│  └──────┬───────┘            │   (Graph SLAM)      │               │

│         │                    └─────────┬────────────┘               │

│         │ keyframe                     │ optimised                  │

│         │ flag                         │ poses                     │

│         ▼                              ▼                           │

│  ┌──────────────┐    SPSC     ┌────────────────────┐               │

│  │  Thread 2    │───────────▶│   Thread 5          │               │

│  │  IMU Reader  │  pre-int   │   Map Manager       │──▶ Pose OUT  │

│  │  / Pre-int   │  factors   │   (publish + persist)│              │

│  └──────────────┘            └─────────┬────────────┘               │

│                                        ▲                           │

│                              ┌─────────┴────────────┐               │

│                              │   Thread 4           │               │

│                              │   Loop Closure        │               │

│                              │   Detector            │               │

│                              └──────────────────────┘               │

│                                                                     │

│  ◄── Frames IN (shm from Process 1)                                │

│  ◄── IMU IN (SPI/I²C)                                              │

│  ──▶ Pose OUT (shm to Process 4 & 5)                               │

└─────────────────────────────────────────────────────────────────────┘

| Thread | Name | Core Affinity | Priority | Wake Source |
| --- | --- | --- | --- | --- |
| T1 | Visual Frontend | Core 2 | SCHED_FIFO 60 | Frame-ready semaphore (shm) |
| T2 | IMU Reader / Pre-integrator | Core 3 | SCHED_FIFO 70 | Timer (5 ms) / SPI IRQ |
| T3 | Backend Optimiser | Core 4 | SCHED_FIFO 50 | Keyframe insertion event |
| T4 | Loop Closure Detector | Core 5 | SCHED_OTHER (nice -5) | Keyframe insertion event |
| T5 | Map Manager | Core 2 (shared) | SCHED_FIFO 45 | Optimised graph event |

# 3. Thread 1 – Visual Frontend

## 3.1 Purpose

The Visual Frontend is the entry point for all camera data. It receives greyscale frames from the shared-memory ring buffer written by Process 1, detects feature keypoints, computes descriptors, and matches them against the previous frame to produce 2-D–2-D correspondences. When a keyframe criterion is met, it packages the tracked features and pushes them into the Backend Optimiser queue.

## 3.2 Pipeline Stages

- 1. Acquire frame pointer (zero-copy) from shared-memory ring buffer

- 2. (Optional) Apply CLAHE histogram equalisation for lighting robustness

- 3. Detect keypoints (ORB / FAST / Shi-Tomasi / SuperPoint)

- 4. Compute descriptors (ORB binary / SuperPoint float)

- 5. Match against previous-frame descriptors (BFMatcher or optical flow)

- 6. Geometric verification – RANSAC essential-matrix to reject outliers

- 7. Keyframe decision (parallax threshold, tracked-feature ratio, time)

- 8. If keyframe → push tracked features + pose prior to Backend queue

- 9. Always push latest relative-motion estimate to Map Manager for fast pose

## 3.3 Feature Detection Strategy

We partition the image into a grid (e.g., 8×6 cells) and enforce a minimum feature count per cell to ensure spatial distribution. This prevents features clustering on a single textured object while leaving other image regions untracked.

💡 C++ Concept #1: Strategy Pattern for Feature Detectors

Different environments (indoor/outdoor, textured/textureless) benefit from different feature detectors. The Strategy pattern lets you swap detectors at runtime without modifying the frontend pipeline. Each detector implements a common interface (IFeatureDetector), and the frontend holds a std::unique_ptr<IFeatureDetector> that is injected at construction. This is the same pattern used in Process 2 for DNN backends.

## 3.4 Keyframe Decision Logic

A new keyframe is created when any of the following conditions is met:

- Median parallax between current frame and last keyframe > 15 pixels (configurable)

- Number of successfully tracked features drops below 60% of initial count

- Time since last keyframe exceeds 0.5 s (ensures periodic updates for slow motion)

- The backend optimiser queue is empty (opportunistic keyframe insertion)

## 3.5 Visual Frontend Thread Loop

💡 C++ Concept #2: Sophus SE(3) Lie Group for Rigid Transforms

Sophus is a header-only C++ library that provides Lie group types (SO3, SE3) built on Eigen. Unlike using raw 4×4 matrices, Sophus::SE3d ensures that transformations always remain on the SE(3) manifold (valid rotation + translation). Composing poses is simply multiplication: T_world_cur = T_world_kf * T_kf_cur. Sophus also provides log/exp maps needed by optimisers (e.g., Ceres) to parameterise incremental updates in the tangent space.

# 4. Thread 2 – IMU Reader / Pre-integrator

## 4.1 Purpose

The IMU Reader thread acquires accelerometer and gyroscope measurements at high rate (typically 200–400 Hz) from the on-board IMU (e.g., BMI088, ICM-42688-P) via SPI or I²C. Between consecutive keyframes it pre-integrates these measurements into a single relative-motion factor that the backend optimiser can incorporate without re-integrating from scratch when the bias estimates change.

## 4.2 IMU Measurement Model

Raw IMU readings contain bias (slowly drifting) and noise:

// Measurement model

// ω_meas = ω_true + b_g + n_g        (gyroscope)

// a_meas = R_bw * (a_true - g) + b_a + n_a   (accelerometer)

//

// Where:

//   b_g, b_a = slowly varying biases (estimated by backend)

//   n_g, n_a = white noise

//   g        = gravity in world frame [0, 0, -9.81]

//   R_bw     = rotation from world to body

## 4.3 Pre-integration Theory

Pre-integration (Forster et al., 2017) accumulates N IMU measurements between two keyframes i and j into a single relative rotation ΔR_ij, velocity Δv_ij, and position Δp_ij. These "pre-integrated measurements" are functions of the bias estimates. When the optimiser updates the biases, instead of re-integrating all N samples, we apply a first-order correction using the pre-computed Jacobians ∂ΔR/∂b_g, ∂Δv/∂b_a, etc.

💡 C++ Concept #3: CRTP (Curiously Recurring Template Pattern) for Static Polymorphism

The IMU pre-integrator uses CRTP to provide a compile-time polymorphic interface for different IMU noise models (continuous vs. discrete-time) without virtual dispatch overhead. In a 200 Hz loop, eliminating vtable lookups matters. CRTP works by having the derived class pass itself as a template parameter to the base class, which then static_casts 'this' to the derived type to call the implementation.

## 4.4 Thread Loop

## 4.5 Hardware Interface (SPI)

On Jetson Orin Nano, the IMU is typically connected via SPI for maximum throughput (up to 10 MHz). We use Linux spidev with a memory-mapped DMA transfer for jitter-free acquisition.

💡 C++ Concept #4: RAII for Hardware Resources (File Descriptors)

The SPI driver above opens a file descriptor in the constructor and closes it in the destructor – the classic RAII (Resource Acquisition Is Initialization) pattern. This guarantees the FD is released even if an exception is thrown. For extra safety, delete copy operations and implement move semantics so the driver object can be transferred but never duplicated.

class SpiImuDriver {

int fd_ = -1;

public:

SpiImuDriver(const std::string& dev, uint32_t speed);

~SpiImuDriver() { if (fd_ >= 0) ::close(fd_); }

// Non-copyable

SpiImuDriver(const SpiImuDriver&) = delete;

SpiImuDriver& operator=(const SpiImuDriver&) = delete;

// Movable

SpiImuDriver(SpiImuDriver&& o) noexcept : fd_(std::exchange(o.fd_, -1)) {}

SpiImuDriver& operator=(SpiImuDriver&& o) noexcept {

if (this != &o) {

if (fd_ >= 0) ::close(fd_);

fd_ = std::exchange(o.fd_, -1);

}

return *this;

}

};

# 5. Thread 3 – Backend Optimiser (Graph SLAM)

## 5.1 Purpose

The Backend Optimiser maintains a factor graph where nodes represent keyframe poses and IMU biases, and edges represent visual re-projection factors, IMU pre-integration factors, and (when available) loop-closure factors. It solves the graph using iterative nonlinear least-squares (Gauss-Newton or Levenberg-Marquardt) to produce globally consistent poses.

## 5.2 Factor Graph Structure

Each keyframe i adds the following to the graph:

- Pose node x_i ∈ SE(3) – the 6-DOF pose of the drone

- Bias node b_i = [b_a, b_g] – IMU biases at keyframe i

- Visual factor – re-projection error of tracked 3-D landmarks into keyframe i

- IMU factor – connects x_i, v_i, b_i to x_{i+1}, v_{i+1}, b_{i+1}

- Bias random-walk factor – soft constraint between b_i and b_{i+1}

- (Optional) Loop-closure factor from Thread 4

## 5.3 Optimisation Libraries

| Library | Approach | Pros | Cons | License |
| --- | --- | --- | --- | --- |
| GTSAM | iSAM2 (incremental) | Best incremental solver, Bayes tree | Heavier dependency | BSD |
| Ceres Solver | Batch LM / Dogleg | Extremely flexible, Google-maintained | Batch-only (no incremental) | Apache-2.0 |
| g2o | Sparse LM | Lightweight, widely used in SLAM | API less ergonomic | BSD |

## 5.4 Using GTSAM iSAM2

GTSAM's iSAM2 performs incremental smoothing and mapping. Instead of re-solving the entire graph on each keyframe, it updates only the affected part of the Bayes tree – reducing per-keyframe optimisation time from O(n²) to near O(1) amortised.

};

💡 C++ Concept #5: Type-Safe Graph Keys with Enum Symbols

GTSAM uses Symbol keys like X(i), V(i), B(i) to identify pose, velocity, and bias nodes in the graph. This pattern prevents accidentally connecting a pose factor to a bias node. In your own code, you can achieve similar type-safety using strong typedefs or enum class indices.

## 5.5 Thread Loop

# 6. Thread 4 – Loop Closure Detector

## 6.1 Purpose

Drift is inevitable in any odometry system. The Loop Closure thread detects when the drone revisits a previously seen location and provides a relative-pose constraint to the backend optimiser, enabling it to correct the accumulated drift across the entire trajectory.

## 6.2 Bag-of-Words Approach (DBoW2 / FBoW)

The classic approach converts each keyframe's feature descriptors into a compact bag-of-words (BoW) vector using a pre-trained visual vocabulary. Candidate loop closures are found by comparing BoW vectors (L1 score). Candidates are then verified geometrically (PnP RANSAC).

💡 C++ Concept #6: std::optional for Nullable Return Values

The detect() method above returns std::optional<LoopCandidate>. This is cleaner than returning a bool + out-parameter, or using a sentinel value. The caller checks with has_value() or uses value_or(). std::optional avoids heap allocation (unlike std::unique_ptr) and communicates intent clearly: "this function might not produce a result."

auto candidate = loop_detector.detect(kf_id, descriptors);

if (candidate) {

// Use candidate->query_kf, candidate->match_kf, candidate->score

spdlog::info("Loop closure: KF {} → KF {} (score {:.3f})",

candidate->query_kf, candidate->match_kf, candidate->score);

backend.add_loop_closure(*candidate);

}

// No raw pointers, no sentinel values, no exceptions for control flow.

## 6.3 Geometric Verification

After a BoW candidate is found, we perform geometric verification:

- 1. Match features between query and candidate keyframes (BFMatcher + ratio test)

- 2. Compute relative pose via PnP-RANSAC (using 3-D map points of candidate)

- 3. If inlier count ≥ 30 and reprojection error < 2.0 px → accept loop closure

- 4. Construct loop-closure factor (SE(3) relative pose + covariance)

- 5. Push factor to backend optimiser queue → triggers global graph re-optimisation

## 6.4 Thread Loop

# 7. Thread 5 – Map Manager

## 7.1 Purpose

The Map Manager is the single source of truth for the current SLAM state. It receives optimised poses from the backend, maintains the sparse 3-D map of landmarks, publishes the latest T_world_body to downstream processes (Mission Planner, Comms), and handles map serialisation (save/load) for persistent re-localisation.

## 7.2 Responsibilities

- Receive optimised keyframe poses from backend optimiser (SPSC queue)

- Maintain a spatial index (kd-tree) of 3-D map points

- Publish latest T_world_body + covariance to shared-memory slots

- Cull redundant keyframes and map points (memory management)

- Serialise map to disk on landing / on command

- Deserialise and re-localise on startup if map file exists

## 7.3 Pose Publishing (Zero-Copy Shared Memory)

💡 C++ Concept #7: SeqLock Pattern for Single-Writer / Multi-Reader Shared Memory

The ShmPoseSlot uses a SeqLock – a sequence number that is odd during writes and even when stable. Readers spin-read the sequence, copy the data, then verify the sequence hasn't changed. This is lock-free, wait-free for writers, and allows multiple readers (Process 4 and Process 5) without any mutex. It works because there is exactly one writer (Map Manager thread).

// Reader side (in Process 4 or Process 5):

bool read_pose(const ShmPoseSlot* slot, PoseStamped& out) {

for (int attempt = 0; attempt < 3; ++attempt) {

uint64_t seq1 = slot->sequence.load(std::memory_order_acquire);

if (seq1 & 1) continue;  // writer is mid-update, retry

// Copy data

out.timestamp = slot->timestamp;

out.position  = Eigen::Map<const Eigen::Vector3d>(slot->translation);

out.orientation = Eigen::Quaterniond(

slot->quaternion[0], slot->quaternion[1],

slot->quaternion[2], slot->quaternion[3]);

std::atomic_thread_fence(std::memory_order_acquire);

uint64_t seq2 = slot->sequence.load(std::memory_order_relaxed);

if (seq1 == seq2) return true;  // consistent read

}

return false;  // failed after retries (write was in progress)

}

## 7.4 Map Serialisation

For persistent re-localisation, the map must be saved to disk. We use a compact binary format (or optionally cereal/protobuf) containing keyframe poses, 3-D landmarks, BoW vectors, and vocabulary reference.

## 7.5 Map Point Culling

To bound memory usage, the Map Manager periodically culls:

- Map points observed by fewer than 3 keyframes

- Map points with high reprojection error (> 5 px average)

- Keyframes that are too close to neighbours (< 5 cm translation)

- Keyframes older than a sliding window (e.g., 200 KFs) unless they have loop-closure edges

# 8. Inter-Thread Communication (Lock-Free Queues)

All inter-thread data flow uses lock-free SPSC (Single-Producer Single-Consumer) bounded ring buffers. This is the same SPSC design used in Process 1 and Process 2. Each queue is templated on the message type and has a compile-time capacity.

## 8.1 Queue Inventory

| Queue | Producer | Consumer | Message Type | Capacity |
| --- | --- | --- | --- | --- |
| keyframe_queue | T1 Visual Frontend | T3 Backend Optimiser | KeyframeData | 8 |
| imu_factor_queue | T2 IMU Reader | T3 Backend Optimiser | PreintegratedImuFactor | 8 |
| pose_queue | T1 Visual Frontend | T5 Map Manager | PoseStamped | 16 |
| optimised_queue | T3 Backend Optimiser | T5 Map Manager | OptimisedPoses | 4 |
| loop_factor_queue | T4 Loop Closure | T3 Backend Optimiser | LoopClosureFactor | 4 |

💡 C++ Concept #8: Lock-Free SPSC Ring Buffer (Recap)

This is the same SPSC queue used across the entire drone stack. The key invariant is: only one thread writes head_, only one thread reads tail_. std::memory_order_acquire/release fences ensure correct visibility across cores without any mutex. The buffer is cache-line padded to prevent false sharing between producer and consumer cache lines.

# 9. IPC – Receiving Frames from Process 1

Process 3 reads greyscale camera frames that were written by Process 1 into a POSIX shared-memory ring buffer. The mechanism uses shm_open + mmap for zero-copy access, and a named POSIX semaphore for signalling.

## 9.1 Shared Memory Reader

# 10. IPC – Publishing Pose to Process 4 & 5

The Map Manager (Thread 5) publishes the optimised pose via a shared-memory SeqLock slot (see Section 7.3). Two downstream processes read this slot:

- Process 4 (Mission Planner): reads pose for path planning and obstacle avoidance

- Process 5 (Comms): reads pose for MAVLink telemetry and GCS display

## 10.1 Shared Memory Layout

// /dev/shm/slam_pose – created by Process 3 at startup

// Layout: single ShmPoseSlot (see Section 7.3)

void MapManager::create_pose_shm() {

int fd = shm_open("/slam_pose", O_CREAT | O_RDWR, 0666);

ftruncate(fd, sizeof(ShmPoseSlot));

auto* slot = static_cast<ShmPoseSlot*>(

mmap(nullptr, sizeof(ShmPoseSlot), PROT_READ | PROT_WRITE,

MAP_SHARED, fd, 0));

slot->sequence.store(0, std::memory_order_relaxed);

pose_publisher_ = std::make_unique<PosePublisher>(slot);

close(fd);  // mapping persists after close

}

## 10.2 Pose Message Format

| Field | Type | Bytes | Description |
| --- | --- | --- | --- |
| sequence | uint64_t (atomic) | 8 | SeqLock counter (odd=writing) |
| timestamp | double | 8 | CLOCK_MONOTONIC seconds |
| translation | double[3] | 24 | x,y,z in NED world frame (metres) |
| quaternion | double[4] | 32 | w,x,y,z (Hamilton convention) |
| covariance | double[36] | 288 | 6×6 pose covariance (row-major) |

# 11. Coordinate Frames & Transformations

Consistent coordinate frame conventions are critical in a SLAM system. A single sign error in a rotation can cause the drone to fly into the ground.

## 11.1 Frame Definitions

| Frame | Name | Origin | Axes | Convention |
| --- | --- | --- | --- | --- |
| W | World / Map | Take-off point | X=North, Y=East, Z=Down | NED |
| B | Body (IMU) | IMU centre | X=Forward, Y=Right, Z=Down | FRD |
| C | Camera | Optical centre | X=Right, Y=Down, Z=Forward | OpenCV |
| L | LiDAR | LiDAR centre | Manufacturer-defined | Varies |

## 11.2 Key Transforms

// Notation: T_target_source transforms a point FROM source TO target.

// p_target = T_target_source * p_source

// The SLAM output is T_world_body (T_WB)

// Camera is rigidly attached to body: T_body_camera (T_BC) from calibration

// To project a world point into the camera:

// p_camera = T_CB * T_BW * p_world

//          = T_camera_world * p_world

//          = (T_WB * T_BC).inverse() * p_world

// Gravity in body frame:

// g_body = R_BW * g_world = R_BW * [0, 0, 9.81]^T  (NED: gravity = +Z)

// T_BC is obtained from camera-IMU extrinsic calibration (Kalibr)

Sophus::SE3d T_body_camera;  // loaded from calibration YAML

## 11.3 Quaternion Convention

We use Hamilton quaternions (q = w + xi + yj + zk) throughout the stack. Eigen::Quaterniond stores coefficients as [x, y, z, w] internally but constructs as Quaterniond(w, x, y, z). Be careful with this ordering. MAVLink ATTITUDE_QUATERNION also uses [w, x, y, z] ordering.

# 12. Available SLAM/VIO Libraries & Algorithms

## 12.1 Complete VIO/SLAM Systems

| System | Type | Sensors | Backend | Loop Closure | License | GPU |
| --- | --- | --- | --- | --- | --- | --- |
| ORB-SLAM3 | V-SLAM + VIO | Mono/Stereo/RGBD + IMU | g2o bundle adj. | DBoW2 | GPL-3.0 | No |
| VINS-Fusion | VIO + SLAM | Mono/Stereo + IMU + GPS | Ceres | DBoW2 | GPL-3.0 | No |
| Kimera-VIO | VIO + Mesh | Stereo + IMU | GTSAM iSAM2 | DBoW2 (Kimera-LCD) | BSD-2 | No |
| OKVIS2 | VIO | Mono/Multi + IMU | Custom Gauss-Newton | Optional | BSD-3 | No |
| SVO Pro | Semi-direct VO+VIO | Mono/Stereo + IMU | Ceres / iSAM2 | Optional | GPL-3.0 | No |
| Basalt | VIO | Stereo + IMU | Custom square-root BA | No | BSD-3 | No |

## 12.2 Learned / Hybrid Approaches

| System | Approach | GPU Required | FPS (Orin Nano est.) | License |
| --- | --- | --- | --- | --- |
| SuperPoint + SuperGlue | CNN feature detect + match | Yes (TensorRT) | ~15-20 | Non-commercial |
| DROID-SLAM | Dense learned SLAM | Yes (heavy) | ~3-5 | BSD-3 |
| DPVO | Dense patch VO | Yes | ~10 | MIT |
| TartanVO | Learning-based VO | Yes | ~8-12 | BSD-3 |

## 12.3 Optimisation Libraries

| Library | Purpose | Key Feature | License |
| --- | --- | --- | --- |
| GTSAM | Factor-graph optimisation | iSAM2 incremental | BSD |
| Ceres Solver | General nonlinear least-squares | Auto-differentiation | Apache-2.0 |
| g2o | Graph optimisation | Lightweight, sparse | BSD |
| Sophus | Lie group types | SE(3)/SO(3) on Eigen | MIT |
| Eigen | Linear algebra | Expression templates | MPL-2.0 |

## 12.4 Recommendation

For the Jetson Orin Nano with limited GPU headroom (most GPU is used by Process 2 DNN inference), we recommend starting with a classical approach:

- Primary: ORB-SLAM3 (well-tested, visual-inertial, loop closure) – but GPL-3.0

- Alternative: Kimera-VIO (BSD-2 license, iSAM2 backend, actively maintained)

- Minimalist: Basalt + custom loop closure (BSD-3, very efficient)

For a from-scratch implementation following this document, the thread architecture maps cleanly to Kimera-VIO's pipeline stages.

# 13. Sensor Configuration & Calibration

## 13.1 Camera Intrinsic Calibration

Use a checkerboard or AprilGrid pattern with OpenCV's calibrateCamera() or Kalibr. Store intrinsics in a YAML file:

# camera_intrinsics.yaml

camera_model: pinhole

distortion_model: radtan

resolution: [640, 480]

intrinsics: [458.654, 457.296, 367.215, 248.375]  # fx, fy, cx, cy

distortion_coefficients: [-0.28340, 0.07395, 0.00019, 0.00001]  # k1, k2, p1, p2

## 13.2 Camera-IMU Extrinsic Calibration

Kalibr (kalibr_calibrate_imu_camera) estimates T_body_camera and the temporal offset t_cam_imu. This is critical – even 1 ms time offset causes significant drift at high speeds.

# camera_imu_extrinsics.yaml (Kalibr output)

T_body_camera:

- [0.0148655, -0.999880,  0.00414, -0.0216401]

- [0.999557,   0.014964,  0.02571, -0.064677]

- [-0.025774, 0.003760,   0.99966,  0.009811]

- [0.0,       0.0,        0.0,      1.0]

time_offset_imu_camera: -0.00781  # seconds (cam trails IMU)

## 13.3 IMU Noise Parameters

Obtain from IMU datasheet or Allan variance analysis. These parameters feed directly into the pre-integrator noise model.

| Parameter | Symbol | Typical BMI088 Value | Unit |
| --- | --- | --- | --- |
| Gyro noise density | σ_g | 0.004 | rad/s/√Hz |
| Gyro random walk | σ_bg | 2.2e-5 | rad/s²/√Hz |
| Accel noise density | σ_a | 0.012 | m/s²/√Hz |
| Accel random walk | σ_ba | 8.0e-5 | m/s³/√Hz |

# 14. Performance Tuning for Jetson Orin Nano

## 14.1 CPU Budget

The Jetson Orin Nano has 6× Arm Cortex-A78AE cores. Process 3 uses cores 2–5. Profile with perf and ensure no thread exceeds its frame budget.

| Thread | Target Latency | Budget | Typical Measured |
| --- | --- | --- | --- |
| T1 Visual Frontend | < 20 ms (50 Hz frame budget) | 33 ms @ 30 Hz | 8–15 ms |
| T2 IMU Reader | < 4 ms (250 Hz) | 5 ms | 0.5–1 ms |
| T3 Backend Optimiser | < 100 ms per keyframe | 200 ms (keyframe rate) | 30–80 ms (iSAM2) |
| T4 Loop Closure | < 200 ms (not real-time critical) | 500 ms | 50–150 ms |
| T5 Map Manager | < 5 ms per update | 33 ms | 1–3 ms |

## 14.2 Memory Budget

- Total cgroup limit for Process 3: 512 MB (configurable)

- Map points: ~50 KB per 1000 landmarks

- Keyframes (images not stored, only features): ~20 KB each

- GTSAM Bayes tree: ~200 KB for 500 keyframes

- BoW database: ~100 KB for 1000 entries with 10K vocabulary

- Total estimate at 500 KFs: ~80 MB

## 14.3 NVPMODEL & Jetson Clocks

## 14.4 Compile-Time Optimisations

- Build with -O3 -march=armv8.2-a+fp16+dotprod

- Enable Eigen vectorisation: -DEIGEN_DONT_ALIGN_STATICALLY=OFF

- Use LTO (Link-Time Optimisation): -flto

- Enable NEON intrinsics for feature matching: -mfpu=neon (auto for aarch64)

- Profile-Guided Optimisation (PGO): build with -fprofile-generate, run benchmark, rebuild with -fprofile-use

# 15. C++ Concepts Reference

This section consolidates the C++ design patterns and idioms used in Process 3, explaining why each is chosen and how it benefits the SLAM pipeline.

• Strategy Pattern (§3.3)

Swap feature detectors at runtime (ORB → KLT → SuperPoint) without modifying the frontend pipeline. Adheres to the Open/Closed Principle.

• Sophus Lie Groups (§3.5)

Ensures all pose transforms remain on the SE(3) manifold. Provides log/exp maps for optimiser parameterisation in tangent space.

• CRTP – Static Polymorphism (§4.3)

Eliminates virtual dispatch in the 200 Hz IMU loop. Compile-time polymorphism with zero overhead vs. hand-written code.

• RAII for Hardware (§4.5)

File descriptors, shared-memory mappings, and SPI handles are released automatically by destructors. Non-copyable + movable prevents double-close.

• std::optional (§6.2)

Loop closure detection returns std::optional<LoopCandidate> – clearly communicates that "no loop found" is a normal outcome, not an error.

• SeqLock for Shared Memory (§7.3)

Single-writer multi-reader lock-free pattern for publishing pose at high frequency. No mutex required; readers are wait-free.

• Lock-Free SPSC Queue (§8)

The inter-thread communication backbone. No locks, no contention, cache-line padded to prevent false sharing.

• Move Semantics (throughout)

Keyframe data, feature vectors, and map points are std::move'd into queues – avoiding expensive deep copies of cv::Mat and Eigen matrices.

• std::variant + std::visit (for sensor fusion)

When integrating optional sensors (barometer, GPS), std::variant cleanly represents "one of several measurement types" without inheritance.

• constexpr + strong types (§5.4)

Graph keys use enum class and constexpr hashing to prevent node-type mix-ups at compile time.

💡 C++ Concept #9: std::variant + std::visit for Multi-Sensor Measurements

When the SLAM system accepts measurements from different sensor types (camera, IMU, barometer, GPS), std::variant provides a type-safe union. std::visit with a visitor struct ensures exhaustive handling of all types at compile time – if you add a new sensor type but forget to handle it, the code won't compile.

💡 C++ Concept #10: Eigen::Map for Zero-Copy Views

Eigen::Map wraps raw memory as an Eigen vector/matrix without copying data. This is used extensively when reading from shared memory (Section 7.3) and when converting between C arrays and Eigen types for GTSAM/Ceres.

# 16. systemd Service Configuration

## 16.1 Startup Sequence

- 1. systemd starts slam-vio-nav after video-capture.service is ready

- 2. Process opens shared-memory ring buffer from Process 1 (read-only)

- 3. Process creates /dev/shm/slam_pose (write) for outgoing pose

- 4. Process loads calibration files (intrinsics, extrinsics, IMU params)

- 5. Process optionally loads saved map for re-localisation

- 6. All 5 threads are spawned with core affinity and RT priority

- 7. Process calls sd_notify("READY=1") to signal systemd

- 8. WatchdogSec=5 → backend must call sd_notify("WATCHDOG=1") every 18. Implementation Roadmap

## Phase 1: IMU Reader & Pre-integration (Week 1–2)

- Implement SPI IMU driver (RAII, read_blocking)

- Implement pre-integration class with CRTP noise model

- Unit test: compare pre-integrated output vs. numerical integration

- Verify 200 Hz read rate with jitter < 0.5 ms

## Phase 2: Visual Frontend (Week 2–4)

- Implement shared-memory frame reader (connect to Process 1)

- Implement ORB grid detector (Strategy pattern)

- Implement BFMatcher + RANSAC geometric verification

- Implement keyframe decision logic

- End-to-end test: visualise tracked features on live frames

## Phase 3: Backend Optimiser (Week 4–6)

- Integrate GTSAM with iSAM2

- Add CombinedImuFactor + SmartProjectionPoseFactor

- Wire keyframe + IMU factor queues (SPSC)

- Test with EuRoC MAV dataset – verify ATE < 0.1 m

- Benchmark: ensure < 100 ms per keyframe update

## Phase 4: Pose Publication & IPC (Week 6–7)

- Implement SeqLock ShmPoseSlot publisher

- Implement reader side for Process 4 & 5

- Verify pose publication rate ≥ 30 Hz

- Verify multi-reader correctness with stress test

## Phase 5: Loop Closure (Week 7–9)

- Integrate DBoW2 vocabulary & database

- Implement geometric verification (PnP RANSAC)

- Wire loop-closure factor to backend

- Test on datasets with revisits – verify drift correction

## Phase 6: Map Persistence & Hardening (Week 9–11)

- Implement map serialisation (cereal binary)

- Implement re-localisation on startup

- systemd integration: sd_notify, watchdog, cgroup limits

- Flight test: verify < 0.5 m position error over 5 min flight

- Stress test: long-duration (30 min) indoor/outdoor flights

- Document configuration parameters and tuning guide