# Interview Preparation Guide

## Head of Software — Twentyfour Industries

European Defense Drones | Munich | Early-Stage Startup

Prepared: February 18, 2026

# Part 1: Know the Company

## Company Profile

Twentyfour Industries is an early-stage, VC-backed startup based in Munich, Germany. They design, manufacture, and deploy small unmanned aerial systems (sUAS) for European and allied defense & security partners.

### Key Product: Q-X Quadcopter

| Specification | Value |
| --- | --- |
| Type | 10-inch modular quadcopter |
| Max Payload | 2.5 kg |
| Max Speed | 140 km/h |
| Operational Radius | 12 km |
| Endurance | 20 min (optimized for short, precise missions) |
| Weather Availability | 98% year-round in Central Europe |

Key selling points: European-made (sovereignty play), modular multi-mission payload concept, cost-effective at scale, already deployed across multiple European countries for civil operators and military units.

### What This Tells You About the Software Challenges

• 140 km/h + 20 min endurance → latency-critical, power-constrained code

• 12 km radius → reliable RF link, possible GNSS-denied scenarios

• Multi-mission modular payloads → hardware-abstraction layers, plugin architecture

• "At scale" manufacturing → reproducible software, OTA updates, fleet management

• Defense/security → safety-critical, possibly export-controlled, reliability > features

• Early stage (~5 engineers → 30+) → you're building everything from ground up

### Software Domains (From Job Posting + Inference)

1. Flight controller software (autopilot, PX4/ArduPilot or custom)

2. Companion computer software (perception, autonomous control, mission planning)

3. Ground Control Station (GCS) software

4. RF/communication link management

5. Low-latency video pipeline

6. GNSS-denied navigation

7. System integration (FC ↔ companion ↔ payloads ↔ GCS ↔ RF)

### Their R&D Team (Current Openings)

• Head of Software (YOUR role)

• Autonomy & Robotics Software Engineer

• Senior Computer Vision Engineer (Perception & Autonomy)

• Senior Controls Engineer (GNC & Autonomy)

• Junior Embedded Linux Engineer

→ They're building a full autonomy stack: perception + controls + planning. As Head of Software, you'd be the technical leader across ALL of these.

# Part 2: C++ Technical Deep Dive

The JD says "deep hands-on expertise in C++, embedded Linux, and real-time systems." Expect the CTO to probe deeply. They want a senior IC first, not a pure manager.

## 2.1 Memory Management & Ownership

Q: Explain the difference between unique_ptr, shared_ptr, and raw pointers. When would you use each in a drone's flight software?

Model Answer:

unique_ptr: exclusive ownership, zero overhead over raw pointers. Use for resources with clear single owners — sensor driver instances, command queue entries, mission way-points.

shared_ptr: reference-counted shared ownership. Use when multiple subsystems need read access — e.g., a fused world model shared between perception, planning, and logging. Has atomic ref-count overhead (~2x raw pointer size + cache line contention).

raw pointers: non-owning observers only. Use for optional back-references ('parent' pointers in a tree), or when passing to C APIs (MAVLink, hardware drivers). Never for ownership. In flight-critical code, I'd prefer unique_ptr as default, shared_ptr only when truly shared, and raw pointers limited to C API boundaries with clear ownership documentation.

Likely Follow-ups:

- What's the overhead of shared_ptr vs unique_ptr?

- How does weak_ptr break circular references?

- When would you use raw new/delete in production drone code?

1. Camera frame buffer

// Sensor thread produces, processing thread consumes — zero-copy transfer

struct CameraFrame {

uint64_t timestamp_ns;

int width, height;

std::vector<uint8_t> pixels;  // moved along with the unique_ptr

};

// Sensor thread

auto frame = std::make_unique<CameraFrame>();

frame->timestamp_ns = get_monotonic_ns();

frame->width = 1280;

frame->height = 720;

frame->pixels.resize(1280 * 720 * 3);

capture_from_v4l2(frame->pixels.data());

processing_queue.push(std::move(frame));  // ownership → queue

// frame is nullptr here — sensor thread can't accidentally modify it

2. Mission plan object

struct Waypoint {

double latitude, longitude, altitude;

float loiter_time_s;

};

struct MissionPlan {

std::string mission_id;

std::vector<Waypoint> waypoints;

// ...

};

class MissionManager {

std::unique_ptr<MissionPlan> active_mission_;

public:

void loadMission(const std::string& filepath) {

// Old mission is automatically destroyed when we reset

active_mission_ = std::make_unique<MissionPlan>();

active_mission_->mission_id = "MSN-042";

active_mission_->waypoints = parseWaypointsFromFile(filepath);

// No memory leak even if we load 100 missions in a row

}

void abortMission() {

active_mission_.reset();  // destructor runs, memory freed

// active_mission_ is now nullptr

}

bool hasMission() const { return active_mission_ != nullptr; }

};

3. Log file writer

class LogWriter {

std::ofstream file_;

public:

LogWriter(const std::string& path) : file_(path, std::ios::app) {

if (!file_.is_open()) throw std::runtime_error("Cannot open log: " + path);

}

~LogWriter() {

file_.flush();

file_.close();  // RAII: file always closed, even on crash/exception

}

void write(const std::string& msg) {

file_ << get_timestamp() << " | " << msg << "\n";

}

};

class FlightRecorder {

std::unique_ptr<LogWriter> logger_;

public:

void startRecording(const std::string& flight_id) {

// If already recording, old log is cleanly closed first

logger_ = std::make_unique<LogWriter>("/data/logs/" + flight_id + ".log");

logger_->write("Recording started");

}

void stopRecording() {

if (logger_) {

logger_->write("Recording stopped");

logger_.reset();  // destructor flushes and closes the file

}

}

void log(const std::string& msg) {

if (logger_) logger_->write(msg);

}

};

4. Payload driver instance (factory pattern)

// Abstract interface — all payloads implement this

class PayloadDriver {

public:

virtual ~PayloadDriver() = default;  // virtual destructor — critical!

virtual bool initialize() = 0;

virtual void shutdown() = 0;

virtual std::string getName() const = 0;

};

class ThermalCameraDriver : public PayloadDriver {

int serial_fd_ = -1;

public:

bool initialize() override {

serial_fd_ = open("/dev/ttyUSB0", O_RDWR);

return serial_fd_ >= 0;

}

void shutdown() override {

if (serial_fd_ >= 0) { close(serial_fd_); serial_fd_ = -1; }

}

~ThermalCameraDriver() override { shutdown(); }  // RAII cleanup

std::string getName() const override { return "ThermalCam-640"; }

};

class LidarDriver : public PayloadDriver {

void* dma_buffer_ = nullptr;

public:

bool initialize() override {

dma_buffer_ = mmap(/* ... DMA buffer setup ... */);

return dma_buffer_ != MAP_FAILED;

}

void shutdown() override {

if (dma_buffer_) { munmap(dma_buffer_, 4096); dma_buffer_ = nullptr; }

}

~LidarDriver() override { shutdown(); }

std::string getName() const override { return "VLP-16"; }

};

// Factory — reads hardware ID, returns the right driver

std::unique_ptr<PayloadDriver> createPayloadDriver(int payload_id) {

switch (payload_id) {

case 0x01: return std::make_unique<ThermalCameraDriver>();

case 0x02: return std::make_unique<LidarDriver>();

default:   return nullptr;  // unknown payload

}

}

// Usage in the payload manager

class PayloadManager {

std::unique_ptr<PayloadDriver> active_payload_;

public:

void onPayloadPlugged(int payload_id) {

active_payload_ = createPayloadDriver(payload_id);

if (active_payload_ && active_payload_->initialize()) {

std::cout << "Payload online: " << active_payload_->getName() << "\n";

}

}

void onPayloadUnplugged() {

active_payload_.reset();  // destructor calls shutdown() → closes serial/frees DMA

}

};

5. State machine state objects

class FlightState {

public:

virtual ~FlightState() = default;

virtual void onEnter() = 0;

virtual void onExit() = 0;

virtual void update(float dt) = 0;

virtual std::string name() const = 0;

};

class TakeoffState : public FlightState {

float target_alt_;

float current_alt_ = 0.0f;

public:

TakeoffState(float alt) : target_alt_(alt) {}

void onEnter() override { std::cout << "Taking off to " << target_alt_ << "m\n"; }

void onExit() override  { std::cout << "Takeoff complete\n"; }

void update(float dt) override { current_alt_ += 2.0f * dt; /* climb at 2 m/s */ }

std::string name() const override { return "Takeoff"; }

};

class HoverState : public FlightState {

float hold_alt_;

public:

HoverState(float alt) : hold_alt_(alt) {}

void onEnter() override { std::cout << "Holding at " << hold_alt_ << "m\n"; }

void onExit() override  { std::cout << "Leaving hover\n"; }

void update(float dt) override { /* PID hold altitude */ }

std::string name() const override { return "Hover"; }

};

class FlightStateMachine {

std::unique_ptr<FlightState> current_state_;

public:

void transitionTo(std::unique_ptr<FlightState> new_state) {

if (current_state_) current_state_->onExit();   // old state cleans up

current_state_ = std::move(new_state);           // ownership transfers

current_state_->onEnter();                        // new state initializes

// old state's destructor ran automatically when unique_ptr was reassigned

}

void update(float dt) {

if (current_state_) current_state_->update(dt);

}

};

// Usage

FlightStateMachine fsm;

fsm.transitionTo(std::make_unique<TakeoffState>(50.0f));

// ... after reaching altitude ...

fsm.transitionTo(std::make_unique<HoverState>(50.0f));

// TakeoffState is automatically destroyed — no manual delete needed

Key patterns to notice across all 5:

- make_unique for creation (never raw new)

- std::move for transferring ownership

- .reset() for explicit destruction

- Virtual destructor on base classes (examples 4 & 5) — without it, unique_ptr<Base> won't call the derived destructor

- No delete anywhere — that's the whole point

Q: What is RAII and why does it matter for a safety-critical system?

Model Answer: Resource Acquisition Is Initialization — acquire resources in constructors, release in destructors. It matters enormously for drones because: (1) deterministic cleanup — even if an exception is thrown or a function returns early, destructors run and resources are released cleanly; (2) no resource leaks under any code path — critical when you're managing hardware peripherals (serial ports, I2C buses, DMA buffers, GPIO pins); (3) it eliminates the 'forgot to close' class of bugs entirely. Example: a ScopedI2CLock class that acquires the bus mutex in its constructor and releases it in the destructor. Even if sensor reading throws, the bus is always freed.

Likely Follow-ups:

- Can you give a real example from embedded code?

- How does RAII interact with exception handling in embedded systems?

- What if exceptions are disabled (which is common in embedded)?

Q: Explain move semantics. Why are they critical for a high-throughput sensor pipeline?

Model Answer: Move semantics transfer resources (heap allocations, file handles, buffers) from one object to another without copying. The source is left in a 'valid but unspecified' state. For a drone sensor pipeline, this is critical: (1) Point clouds and images are large (hundreds of KB to MB). Copying them between pipeline stages would blow the latency budget. Moving is O(1). (2) Thread-safe queues between producer/consumer stages: sensor thread produces a read, moves it into the queue, processing thread moves it out. Zero copies. (3) Return-by-value from functions is efficient thanks to RVO + move semantics. In practice: `queue.push(std::move(sensor_frame));` transfers the entire frame buffer in ~1 ns.

Likely Follow-ups:

- What's the difference between std::move and actually moving?

- What's RVO/NRVO? When does the compiler elide copies vs use moves?

- How do you write a move constructor and move assignment operator?

## 2.2 Concurrency & Real-Time

Q: How would you design a real-time sensor processing pipeline that must run at 50Hz on embedded Linux?

Model Answer: Architecture: dedicated threads for each pipeline stage, connected by lock-free queues. (1) Sensor reading thread — highest priority, pinned to a dedicated CPU core using pthread_setaffinity. Reads data via DMA or mmap'd device memory. (2) Processing thread — runs perception (object detection, tracking). Uses a double-buffer: reads from the latest complete frame, never blocks on sensor thread. (3) Control output thread — runs at the 50Hz control loop rate. Takes latest processed state. Communication: SPSC (single-producer, single-consumer) lock-free ring buffers between stages. No mutexes in the critical path — they cause priority inversion. Timing: Use CLOCK_MONOTONIC timestamps. Track jitter. If a frame takes >18ms (out of 20ms budget), log a warning. Use SCHED_FIFO or SCHED_RR for deterministic scheduling. Memory: Pre-allocate all buffers at startup. No dynamic allocation in the real-time loop. Use memory pools for any per-frame allocations.

Likely Follow-ups:

- How do you avoid priority inversion with mutexes?

- What's a lock-free queue? How does it work?

- What's the difference between SCHED_FIFO and SCHED_RR?

- How do you profile real-time jitter on Linux?

Q: Explain the difference between std::mutex, std::atomic, and lock-free programming.

Model Answer: std::mutex: heavyweight synchronization. Kernel-mediated. Guarantees mutual exclusion but can cause priority inversion, unbounded blocking, and context switches. Avoid in real-time paths. std::atomic: hardware-level atomic operations (compare-and-swap, load/store with memory ordering). Lock-free but NOT wait-free — a thread can spin. Good for flags, counters, simple shared state. Lock-free data structures: built on atomics. Guarantee system-wide progress (at least one thread makes progress). Needed for real-time producer/consumer queues. For a drone: mutex is fine for configuration updates (rare, not time-critical). Atomic flags for 'arm/disarm' state. Lock-free queues between sensor and processing threads.

Likely Follow-ups:

- What's std::memory_order and when do you care?

- What's a compare-and-swap (CAS) operation?

- Have you used lock-free queues in production?

Q: What is priority inversion and how do you prevent it in a drone control system?

Model Answer: Priority inversion: a high-priority thread (control loop) blocks waiting for a mutex held by a low-priority thread (logging), while a medium-priority thread (video encoding) runs instead — starving the high-priority thread. This can cause loss of control. Classic example: Mars Pathfinder (1997). Solutions: (1) Priority inheritance — when a high-priority thread blocks on a mutex, the owner's priority is temporarily boosted. Use PTHREAD_PRIO_INHERIT. (2) Lock-free design — eliminate mutexes entirely in the critical path. (3) Priority ceiling — set a mutex's priority to the highest thread that could use it. In practice for a drone: the control loop thread should NEVER acquire a mutex. Feed it data via a lock-free triple-buffer. Only the latest state matters (overwrite is fine).

Likely Follow-ups:

- How did the Mars Pathfinder bug manifest and how was it fixed?

- What's a triple-buffer and how is it different from a double-buffer?

## 2.3 Object-Oriented Design & Patterns

Q: Design the software architecture for a modular payload system on a quadcopter.

Model Answer: The key challenge: unknown payloads attached at runtime (camera, thermal sensor, LIDAR, EW payload). Architecture:

(1) Abstract PayloadInterface base class with virtual methods: initialize(), start(), stop(), getStatus(), processData().

(2) Factory pattern: a PayloadFactory reads the payload ID from a hardware pin or I2C EEPROM, and instantiates the correct derived class.

(3) Plugin architecture: payloads implemented as shared libraries (.so files) loaded at runtime via dlopen(). This allows adding new payloads without recompiling the main firmware.

(4) MAVLink/protobuf-based protocol for payload-GCS communication, extensible per payload type.

(5) Resource manager tracks power budget, bandwidth allocation, CPU time per payload.

This maps to 24 Industries' 'Modular Rail System for Payloads' — you need hot-pluggable payload support with clean hardware abstraction.

Likely Follow-ups:

- How would you handle a payload that misbehaves (crashes, consumes too much power)?

- How do you version the plugin API?

- How does this interact with the safety certifier?

Q: When would you choose composition over inheritance in a drone codebase?

Model Answer: Almost always favor composition. Inheritance creates tight coupling — changes to the base class ripple through all derived classes. Composition allows mixing and matching behaviors. Example: instead of inheriting from both FlyableVehicle and ArmedVehicle (diamond problem), compose: class Drone { FlightController fc_; WeaponsController wc_; NavigationModule nav_; }; Use inheritance only for: (1) abstract interfaces that define plugin contracts (SensorInterface), (2) when 'is-a' is genuinely true and the hierarchy is shallow (max 2-3 levels). In practice at 24 Industries: use interface inheritance for PayloadInterface, SensorInterface, CommunicationInterface. Use composition for the vehicle class that aggregates them all.

Likely Follow-ups:

- How does the diamond problem manifest in C++?

- What are virtual base classes and when would you use them?

Q: How do you handle error propagation in a system where exceptions are disabled?

Model Answer: Many embedded/real-time codebases disable exceptions (-fno-exceptions) because they have unpredictable timing and memory overhead. Alternatives: (1) std::expected<T, E> (C++23) or tl::expected (pre-C++23 library) — return either a value or an error, forcing the caller to check. (2) Error codes with [[nodiscard]] attribute — compiler warns if return code is ignored. (3) Result<T> pattern: a custom template that wraps a value or error. (4) For fatal errors: assert() in debug, dedicated error handler in release (failsafe mode). In a drone: critical errors → immediate failsafe (land or loiter). Non-critical → log and degrade. Never silently swallow errors in flight-critical code.

Likely Follow-ups:

- What is std::expected and how does it compare to Rust's Result<T, E>?

- How do you implement a failsafe state machine?

## 2.4 Templates & Compile-Time Programming

Q: How would you use templates to build a type-safe message passing system for drone subsystems?

Model Answer: Design a Message<Tag, Payload> template where Tag is a compile-time identifier and Payload is the data type. The message bus uses a type-erased queue internally but provides type-safe publish/subscribe: bus.publish<IMUData>(imu_reading); bus.subscribe<IMUData>([](const IMUData& d) { ... }); Benefits: (1) compile-time checking — subscribing to wrong type is a compiler error. (2) Zero-overhead abstraction — templates are resolved at compile time. (3) Extensible — add new message types without modifying the bus. Implementation: type-erased callback storage internally (std::any or void*), type-safe wrapper externally. Use std::type_index as the routing key.

Likely Follow-ups:

- How does type erasure work under the hood?

- What are the compile-time costs of heavy template usage?

- How would you handle serialization for the RF link?

Q: What is CRTP (Curiously Recurring Template Pattern) and when would you use it?

Model Answer: CRTP: a class derives from a template parameterized by itself: class LidarSensor : public SensorBase<LidarSensor> { }; It enables 'static polymorphism' — the compiler resolves virtual-like dispatch at compile time, eliminating vtable overhead. Use case in drones: a high-frequency sensor pipeline where virtual function overhead is measurable. Instead of virtual readData(), the base class calls static_cast<Derived*>(this)->readDataImpl(). The compiler inlines the call entirely. Trade-off: harder to read than virtual dispatch. Only worth it when profiling shows vtable overhead matters (inner loops called >1MHz).

Likely Follow-ups:

- What's the runtime overhead of a virtual function call?

- Can you show the CRTP pattern in code?

## 2.5 Embedded Linux & Low-Level

Q: Describe how you'd bring up software on a new companion computer board for a drone.

Model Answer: Phase 1 — Board bring-up: (1) Boot Linux on the target: configure U-Boot, build kernel with device tree for our specific board (camera interfaces, SPI buses, UART to FC). Use Yocto/Buildroot for the root filesystem. (2) Verify all peripherals: i2cdetect for sensors, v4l2-ctl for cameras, test UART to flight controller with minicom. Phase 2 — Application layer: (3) Cross-compile toolchain: set up CMake cross-compilation for ARM64 target from x86 dev machine. (4) Bring up MAVLink/DDS communication with the flight controller. (5) Bring up camera pipeline: V4L2 capture → ISP processing → perception algorithms. Phase 3 — System hardening: (6) Watchdog timer: hardware WDT that reboots the companion computer if it hangs. (7) Read-only root filesystem with overlayfs for logs. (8) OTA update mechanism for field deployments. (9) Resource limits: cgroups for memory, CPU quotas per process.

Likely Follow-ups:

- What's your experience with Yocto vs Buildroot?

- How do you handle kernel driver development?

- How do you debug when you don't have JTAG access?

Q: How would you design a low-latency video pipeline for a drone streaming to a ground station?

Model Answer: Architecture: Camera → ISP → Encoder → Packetizer → RF Transmit. Key choices: (1) Capture: V4L2 with MMAP'd buffers — zero-copy from sensor to user space. DMA-BUF for sharing between ISP/encoder without memcpy. (2) Encoding: Hardware H.264/H.265 encoder. Use low-latency mode (no B-frames, CBR rate control, small GOP). Target 2-3 frame latency (60-100ms at 30fps). (3) Transport: RTP/UDP over the RF link. UDP, not TCP — retransmission delays are worse than losing a frame. Implement simple FEC (Forward Error Correction) for lossy links. (4) Adaptive bitrate: monitor RF link quality, adjust encoder bitrate dynamically. Lower quality > frozen stream. (5) Total glass-to-glass latency target: <150ms. Track each stage with timestamps. At 24 Industries with 12km range, the RF link is the bottleneck. Bandwidth-adaptive encoding is essential.

Likely Follow-ups:

- How do you handle link degradation gracefully?

- What's your experience with GStreamer vs custom pipelines?

- How would you add on-board AI inference in the video pipeline?

# Part 3: System Architecture & Design

The JD emphasizes: "Drive architectural decisions that survive real-world integration." Expect whiteboard-style design questions.

## 3.1 Full Drone Software Architecture

Be ready to whiteboard this:

Draw the complete software architecture of a semi-autonomous drone system. Label:
  • Hardware interfaces (sensors, actuators, RF)
  • Software layers (HAL, middleware, application)
  • Communication buses (internal: MAVLink/DDS, external: RF to GCS)
  • Safety boundaries (what runs on the FC vs companion computer)
  • Key processes/threads and their priorities

ARCHITECTURE LAYERS (bottom to top):

Layer 1 — Hardware Abstraction Layer (HAL)
  • Drivers for: IMU, barometer, GPS, magnetometer, camera, LIDAR, radar, servo/ESC
  • Runs on: flight controller (FC) for flight-critical sensors, companion for payload sensors
  • Protocol: SPI/I2C/UART, V4L2, custom kernel modules

Layer 2 — Flight Controller (FC) — runs on dedicated MCU (e.g., STM32)
  • Attitude estimation (EKF: IMU + GPS + Baro + Mag)
  • Attitude/Rate controller (PID or cascaded PID)
  • Position controller
  • Failsafe manager
  • Based on PX4 or ArduPilot (likely PX4 for a European defense startup)

Layer 3 — Companion Computer — runs on embedded Linux (e.g., Jetson Orin Nano)
  • Perception: object detection, tracking, obstacle avoidance
  • Mission planning: path planning, waypoint management
  • GNSS-denied navigation: visual-inertial odometry (VIO), SLAM
  • Video pipeline: capture → encode → stream
  • Payload management: modular payload driver framework

Layer 4 — Communication Layer
  • MAVLink v2 between FC ↔ Companion ↔ GCS (primary command protocol)
  • Custom low-latency protocol for video stream
  • Encrypted data link (AES-256 for defense applications)
  • Link quality monitoring and adaptive bitrate

Layer 5 — Ground Control Station (GCS)
  • Operator UI: map, video, telemetry, alerts
  • Mission planning interface
  • Multi-vehicle management (fleet operations)
  • Native app (Qt/C++) or web-based

## 3.2 Design Question: GNSS-Denied Navigation

Q: Your drone loses GPS signal in an urban canyon. How does it continue to navigate?

Model Answer: Multi-modal navigation fallback stack: (1) Primary: RTK-GPS — centimeter accuracy when available. (2) Visual-Inertial Odometry (VIO): fuse camera images with IMU data to estimate motion. Libraries: VINS-Fusion, OpenVINS, or ROVIO. Needs a stereo or mono camera + IMU at 200Hz+. Drift accumulates over time (~1% of distance traveled). (3) Visual SLAM: build a map and localize within it. Use pre-built maps for known operating areas (defense operations often involve pre-surveying). (4) Optical flow + rangefinder: for altitude hold when other systems degrade. (5) Barometer: coarse altitude estimation, fused with other altitude sources. Key design: state estimator (EKF) that fuses ALL available sources, weighted by reliability. When GPS is lost, the EKF seamlessly increases weight on VIO. Never a hard switch. Failsafe: if position uncertainty exceeds threshold, enter loiter mode and attempt to recover (climb for GPS, return to last known good position).

Likely Follow-ups:

- What's the drift rate of typical VIO systems?

- How do you handle the transition when GPS comes back?

- What hardware would you select for the VIO pipeline?

## 3.3 Design Question: OTA Updates for a Drone Fleet

Q: How would you design a secure OTA (over-the-air) update system for 500+ drones deployed in the field?

Model Answer: Architecture: (1) Dual-bank (A/B) partitioning: two copies of the root filesystem. Update writes to the inactive partition. If boot fails, fall back to the known-good partition. Never brick a drone. (2) Delta updates: only transmit changed blocks (using bsdiff or similar). A 500MB filesystem with a 10MB code change should transmit ~15MB, not 500MB. Critical for bandwidth-constrained RF links. (3) Cryptographic verification: signed update packages (RSA/ECDSA). The drone verifies the signature before applying — prevents tampered firmware in a defense context. (4) Staged rollout: update 5% of fleet → monitor telemetry for 24h → roll forward or stop. (5) Manifest system: each drone reports its current software version. Fleet management server tracks which drones need which updates. (6) Connectivity-aware: queue update for next ground connection if drone is in-flight. Never update during a mission. Tools: SWUpdate or RAUC for the embedded Linux update agent. Custom fleet server.

Likely Follow-ups:

- How do you handle partial/interrupted updates?

- What about updating the flight controller firmware (different MCU)?

- How do you manage configuration vs code updates?

## 3.4 Design Question: Safety Architecture

Q: How do you ensure the software doesn't crash a $50k drone or harm people?

Model Answer: Defense-in-depth safety architecture: (1) SEPARATION: Flight-critical software (FC) is isolated from mission software (companion). If perception crashes, the FC can still fly. The companion can be fully power-cycled without affecting flight stability. (2) WATCHDOGS: Hardware watchdog on both FC and companion. If software hangs, hardware forces a reboot. FC watchdog triggers RTL (Return to Launch). (3) GEOFENCING: Hard position/altitude limits enforced at FC level. Software cannot command the drone outside the approved operating volume. (4) STATE MACHINE: Explicit drone states (Disarmed → Armed → Takeoff → Mission → Return → Land). Every transition has preconditions. Invalid transitions are rejected. (5) FAILSAFE CASCADE: GPS lost → switch to VIO. VIO degraded → loiter. Battery low → RTL. Comms lost → RTL after timeout. Companion crash → FC-only flight (manual control). (6) WORST-CASE EXECUTION TIME: Critical loops have hard time budgets. Overrun → skip frame, not block. (7) TESTING: SITL (Software-In-The-Loop). HITL (Hardware-In-The-Loop). Automated regression tests for every commit. Fault injection testing.

Likely Follow-ups:

- What safety standards are relevant (DO-178C, STANAG)?

- How do you test failsafe cascades?

- How do you handle a motor failure in flight?

# Part 4: Leadership & Behavioral Questions

The JD says: "Senior IC first: you personally own critical technical domains before scaling through others." They want a player-coach, not a project manager. Also: "Build, scale, and harden the software organization from ~5 to >30 engineers."

## 4.1 Team Building & Scaling

Q: How would you scale a software team from 5 to 30 engineers while maintaining velocity and quality?

Model Answer: Phase 1 (5→10): Hire senior generalists who can own entire subsystems. At this stage, every hire must be able to work autonomously. Minimize process — daily standup, weekly demo, that's it. Focus on establishing: coding standards, CI/CD pipeline, testing culture. Phase 2 (10→20): Introduce team structure. Split into 3-4 focused teams: Flight Systems, Perception/Autonomy, GCS/Infrastructure, Platform (build/deploy/test). Each team has a tech lead. Introduce code review requirements (2 approvals). Weekly architecture sync across teams. Phase 3 (20→30): Add mid-level and junior engineers per team. Budget for mentorship overhead — each senior/lead should expect 20% time mentoring. Introduce sprint planning at team level. Architecture Decision Records (ADRs) for cross-team decisions. Throughout: hire people smarter than me. Define clear ownership. Measure outcomes, not activity. Never let hiring bar drop — one bad hire at 5 people is catastrophic.

Likely Follow-ups:

- What's your interview process for evaluating engineers?

- How do you handle underperformers?

- How do you maintain culture during rapid growth?

Q: Tell me about a time you made an architectural decision that you later had to reverse.

Model Answer: FRAMEWORK: Use STAR (Situation, Task, Action, Result). Prepare YOUR specific story. Here's the structure:

Situation: Describe the project context and constraints.
Task: What decision did you face?
Action: What did you decide and why? What made you realize it was wrong?
Result: How did you course-correct? What did you learn?

KEY MESSAGES they want to hear:
  • You can make decisions under uncertainty (not analysis paralysis)
  • You recognize when a decision is wrong (ego doesn't block you)
  • You can reverse course pragmatically (not sunk-cost fallacy)
  • You extract lessons and share them (ADRs, postmortems)
  • You own the mistake — no blame-shifting

Likely Follow-ups:

- Walk me through the technical trade-offs you considered

- How did the team react when you reversed course?

- What process did you put in place to prevent similar mistakes?

Q: How do you balance speed vs. robustness in a startup shipping defense-grade systems?

Model Answer: The answer is NOT 'always prioritize quality' or 'move fast and break things.' It's contextual. My framework: Create two lanes. (1) FLIGHT-CRITICAL path: robustness is non-negotiable. Full testing, code review, regression testing. Never ship if it could crash the drone or harm someone. This is where we invest in process and verification. (2) MISSION/FEATURE path: iterate fast, ship MVPs, get field feedback. A slightly buggy GCS feature is acceptable — a slightly buggy attitude controller is not. The key insight: clearly define the boundary between these lanes. Document it. Everyone on the team must know which lane they're operating in. Techniques for going fast safely: feature flags, A/B partitioning, extensive SITL testing that runs in minutes (not hours), staged rollout to field units.

Likely Follow-ups:

- Give me an example of a trade-off you've made between these two

- How do you communicate this to the engineering team?

- How do you handle pressure from business to ship faster?

## 4.2 Technical Leadership

Q: You join and the existing codebase is messy with no tests. What do you do?

Model Answer: Priority 1 (Week 1-2): Understand before changing. Read the code, talk to engineers, understand WHY it's like this. Often there are good historical reasons. Priority 2 (Week 2-4): Add CI/CD pipeline. Automated build, static analysis (clang-tidy), and basic testing infrastructure. Make it easy to add tests. Priority 3 (Month 1-3): Write tests for the most critical and most fragile code first — not alphabetically. Flight controller interface tests, state machine tests, failsafe tests. Rule: all NEW code must have tests. Don't demand instant coverage of old code (demoralizing). Priority 4 (Month 3-6): Incremental refactoring. When you touch a module for a feature, improve it. Boy Scout Rule — leave it better than you found it. NEVER propose a 'big rewrite'. They always fail. Strangle pattern: build new components next to old ones, gradually migrate, then remove old code.

Likely Follow-ups:

- How do you measure code quality improvement?

- How do you get buy-in from engineers who wrote the messy code?

Q: How do you evaluate build vs. buy vs. open-source decisions?

Model Answer: Framework: (1) Is this a CORE DIFFERENTIATOR? If yes → build it. For 24 Industries: the autonomy stack, mission planning, and fleet management are strategic differentiators. Own them. (2) Is this a COMMODITY? If yes → use open source or buy. Don't build your own JSON parser, RTOS, or CI server. PX4 is battle-tested — don't rewrite an autopilot from scratch. (3) Is this STRATEGIC CONTROL? For a defense company, you may need to own components that would otherwise be commodity — supply chain security. Can't depend on a US company's SDK for a European sovereignty product. (4) Integration cost: open-source isn't free. Evaluate maintenance burden, license compatibility (GPL risks in commercial/defense), and the cost of adapting vs building. The JD mentions 'experience integrating third-party software' — this is clearly a key skill.

Likely Follow-ups:

- How do you handle GPL-licensed code in a defense product?

- How do you evaluate the health of an open-source project?

- If a critical open-source dependency dies, what's your plan?

## 4.3 Defense-Specific Questions

Q: What are the unique software challenges of building for defense/military customers?

Model Answer: Several major differences from commercial software: (1) Certification & compliance: depending on the use case, may need to follow DO-178C (airborne software), STANAG standards, or national defense procurement standards. Even if full certification isn't required, the customer expects evidence of rigorous process. (2) Export controls: ITAR (US) or EAR, European dual-use regulations. Certain software components may be export-controlled. Need careful tracking of who accesses what code. (3) Security: assume the enemy will capture the drone. Encrypt all stored data. Secure boot. Tamper detection. Ability to remotely wipe. Classified vs unclassified software separation. (4) Reliability over features: military wants systems that work in harsh conditions (heat, cold, rain, vibration, EMP), not systems with the most features. (5) Interoperability: NATO STANAG for communication, command & control interfaces. For 24 Industries: European sovereignty is their pitch. This means: no US dependencies, comply with EU export regulations, and build trust with European defense procurement.

Likely Follow-ups:

- How would you handle ITAR restrictions on a component you need?

- What's your approach to security in the software supply chain?

# Part 5: Likely Coding Challenges

As a "senior IC first" role, expect at least one hands-on coding challenge. These are the most likely problems for a drone software role:

## 5.1 Implement a Ring Buffer (Lock-Free)

VERY common for real-time systems interviews. Implement a single-producer single-consumer ring buffer that's safe to use between a sensor thread and a processing thread.

Study this implementation — be ready to write it from memory:

template <typename T, size_t Capacity>

class SPSCRingBuffer {

public:

bool push(const T& item) {

size_t curr_write = write_idx_.load(std::memory_order_relaxed);

size_t next_write = (curr_write + 1) % Capacity;

if (next_write == read_idx_.load(std::memory_order_acquire)) {

return false;  // Full

}

buffer_[curr_write] = item;

write_idx_.store(next_write, std::memory_order_release);

return true;

}

bool pop(T& item) {

size_t curr_read = read_idx_.load(std::memory_order_relaxed);

if (curr_read == write_idx_.load(std::memory_order_acquire)) {

return false;  // Empty

}

item = buffer_[curr_read];

read_idx_.store((curr_read + 1) % Capacity, std::memory_order_release);

return true;

}

private:

std::array<T, Capacity> buffer_;

std::atomic<size_t> read_idx_{0};

std::atomic<size_t> write_idx_{0};

};

Be prepared to explain: why memory_order_acquire/release (not seq_cst), why this only works for single-producer/single-consumer, and how you'd extend it to MPMC (multiple producers).

## 5.2 Implement a Simple Kalman Filter

State estimation is fundamental to drones. Be ready to implement a 1D or 2D Kalman filter. At minimum know the predict/update equations:

1D Kalman Filter:

class KalmanFilter1D {

public:

KalmanFilter1D(float initial_estimate, float initial_error,

float process_noise, float measurement_noise)

: x_(initial_estimate), p_(initial_error)

, q_(process_noise), r_(measurement_noise) {}

void predict() {

// x stays same (constant model), uncertainty grows

p_ = p_ + q_;

}

void update(float measurement) {

float k = p_ / (p_ + r_);  // Kalman gain

x_ = x_ + k * (measurement - x_);  // Update estimate

p_ = (1.0f - k) * p_;              // Update uncertainty

}

float estimate() const { return x_; }

float uncertainty() const { return p_; }

private:

float x_;  // State estimate

float p_;  // Estimate uncertainty

float q_;  // Process noise

float r_;  // Measurement noise

};

## 5.3 Implement a Finite State Machine

Drone flight management uses a state machine. Be ready to implement one:

Flight State Machine:

enum class DroneState { Disarmed, Armed, TakingOff, Hovering, InMission, Returning, Landing };

class FlightStateMachine {

public:

bool transition(DroneState target) {

// Validate transition

if (!is_valid_transition(state_, target)) return false;

on_exit(state_);

state_ = target;

on_enter(state_);

return true;

}

DroneState current() const { return state_; }

private:

DroneState state_ = DroneState::Disarmed;

bool is_valid_transition(DroneState from, DroneState to) {

// Only allow specific transitions

switch (from) {

case DroneState::Disarmed:   return to == DroneState::Armed;

case DroneState::Armed:      return to == DroneState::TakingOff

|| to == DroneState::Disarmed;

case DroneState::TakingOff:  return to == DroneState::Hovering;

case DroneState::Hovering:   return to == DroneState::InMission

|| to == DroneState::Landing

|| to == DroneState::Returning;

case DroneState::InMission:  return to == DroneState::Hovering

|| to == DroneState::Returning;

case DroneState::Returning:  return to == DroneState::Hovering

|| to == DroneState::Landing;

case DroneState::Landing:    return to == DroneState::Disarmed;

}

return false;

}

void on_exit(DroneState s) { /* cleanup actions */ }

void on_enter(DroneState s) { /* entry actions */ }

};

## 5.4 Data Structure Questions

Common C++ DS/algorithm questions relevant to drones:
• Spatial data structures: KD-tree for nearest-neighbor in point clouds
• Priority queue: for A* path planning, event scheduling
• Graph BFS/DFS: for path planning, dependency resolution
• Sliding window: sensor data windowed averages, median filter
• Implement std::vector from scratch — tests memory management understanding

# Part 6: Questions You Should Ask Them

Asking smart questions demonstrates your understanding of the domain and the role. Pick 3-4 from this list based on what feels natural in the conversation.

## Technical Questions

1. "What's the current flight controller stack — PX4, ArduPilot, or custom? And what's the companion computer hardware?"

2. "How much of the autonomy stack exists today vs. what needs to be built from scratch?"

3. "What's the real-time latency budget for the perception-to-control loop?"

4. "How do you handle software updates for drones already deployed in the field?"

5. "What's the current testing setup — SITL, HITL, flight test cadence?"

## Team / Culture Questions

6. "What does the current team of ~5 look like? What are their strengths, and where are the gaps?"

7. "How does R&D interact with manufacturing and field operations?"

8. "What does success look like for the Head of Software in the first 6 months?"

9. "How do you balance feature development for customers vs. building the core platform?"

## Strategy Questions

10. "What's the competitive landscape in European defense drones? Who are the primary competitors?"

11. "How much autonomy does the defense customer actually want vs. HITL (human-in-the-loop)?"

12. "What certification/compliance requirements apply to your current and planned products?"

13. "What's the CTO's vision for the software architecture in 3 years?"

# Part 7: Pre-Interview Study Checklist

## C++ Must-Know Topics

- Smart pointers (unique_ptr, shared_ptr) — when, why, overhead

- RAII — explain and give examples

- Move semantics — move constructor, std::move, rvalue references

- Virtual functions — vtable, virtual destructor, override keyword

- Templates — function templates, class templates, SFINAE concepts

- STL containers — vector, map, unordered_map, array — complexity guarantees

- Concurrency — std::thread, std::mutex, std::atomic, std::condition_variable

- Memory model — memory_order, happens-before, acquire/release semantics

- Lambda expressions — capture by value/reference, generic lambdas

- constexpr and compile-time computation

- Exception safety guarantees (basic, strong, nothrow)

- Rule of 0/3/5 — when to implement special member functions

## Drone / Robotics Must-Know Topics

- PX4 / ArduPilot architecture (at least high-level)

- MAVLink protocol — message structure, key message types

- Kalman Filter — predict/update cycle, state vector, covariance

- Visual-Inertial Odometry (VIO) — concept and limitations

- SLAM — concept, loop closure, key algorithms (ORB-SLAM, LSD-SLAM)

- Path planning — A*, RRT, potential fields

- Control theory basics — PID, cascaded controllers

- Coordinate frames — body frame, NED, ENU, transformations

- Sensor characteristics — IMU, GPS, cameras, LiDAR, radar

- Real-time Linux — PREEMPT_RT, scheduling policies, CPU affinity

## Leadership Talking Points to Prepare

- A story about building/growing a team (hiring, culture, process)

- A story about making a hard architectural decision under uncertainty

- A story about debugging a critical production issue

- A story about integrating a difficult third-party component

- A story about disagreeing with a stakeholder and how you resolved it

- Your approach to code review, testing standards, and engineering culture

- How you balance IC work with management as a team grows

## Day Before the Interview

- Review this document one final time

- Prepare 3 STAR stories (Situation, Task, Action, Result) for behavioral questions

- Practice whiteboarding: draw the drone architecture stack from memory

- Practice coding: implement a ring buffer and Kalman filter from scratch

- Research recent news about Twentyfour Industries and European defense drone market

- Prepare your 2-minute 'tell me about yourself' pitch — emphasize: tech leadership + real-time systems + shipping production code

- Have 4-5 thoughtful questions ready for THEM

- Get a good night's sleep

— Good luck! You've got this. —