# Autonomous Drone
Software Architecture

## System Design with Cloud Connectivity,
Modular Payload System & SLAM Navigation

Interview Preparation — Head of Software
Twentyfour Industries

Prepared: February 21, 2026

# Table of Contents

- 1. System Overview & Hardware Context

- 2. High-Level Software Architecture

- 3. Flight Controller Software Layer

- 4. Companion Computer Software Layer

- 5. Perception & Autonomy Pipeline

- 6. SLAM & GNSS-Denied Navigation (NEW)

- 7. Modular Payload System Design

- 8. Communication Architecture

- 9. Cloud Connectivity & Fleet Management

- 10. Safety & Fault Tolerance Architecture

- 11. Data Flow & Threading Model

- 12. Software Deployment & OTA Updates

- 13. Technology Stack & Trade-offs

- 14. Interview Whiteboard Walkthrough

# 1. System Overview & Hardware Context

Before designing software, you must understand the hardware constraints. For a small tactical drone like the Q-X (2.5kg payload, 20min endurance, 140km/h):

| Component | Typical Hardware | Constraints |
| --- | --- | --- |
| Flight Controller | STM32H7 / PX4 FMU v6 | 480MHz, 1MB RAM, hard real-time |
| Companion Computer | Jetson Orin Nano / RPi CM4 | 6-8 TOPS AI, 4-8GB RAM, 7-15W |
| RF Data Link | Microhard / Silvus / Custom | 1-50 Mbps, 12km range, encrypted |
| GPS/GNSS | u-blox F9P (RTK) | cm accuracy with RTK, degraded in urban |
| IMU | ICM-42688 / BMI088 | 200-1000Hz, vibration-sensitive |
| Stereo Camera | OAK-D / RealSense D455 | Stereo for depth + VIO/SLAM |
| Mission Camera | IMX477 / IMX585 | 12MP, 60fps, mission payload |
| LiDAR (optional) | Livox Mid-360 | 200m range, 200K pts/s, 10Hz |
| Power Budget | — | 50-100W total, compute ≤ 15W |

⚡ Interview Tip: Start your whiteboard answer with hardware context. It shows you understand that software architecture is driven by physical constraints — power, weight, compute, latency. This is what separates you from a web developer.

# 2. High-Level Software Architecture

The architecture follows a classic separation between safety-critical flight control and mission/payload computing, connected by a well-defined protocol boundary. SLAM runs on the companion computer and feeds corrected position estimates back to the FC.

Figure 1: Complete System Architecture — SLAM Integrated

┌─────────────────────────────────────────────────────────────────────────────┐ │ GROUND SEGMENT │ │ │ │ ┌──────────────────┐ ┌──────────────────┐ ┌─────────────────────┐ │ │ │ Ground Control │ │ Cloud Backend │ │ Fleet Management │ │ │ │ Station (GCS) │ │ (AWS/Azure/K8s) │ │ Dashboard │ │ │ │ │ │ │ │ │ │ │ │ • Operator UI │ │ • Telemetry DB │ │ • Multi-vehicle │ │ │ │ • Video display │ │ • Mission store │ │ • OTA updates │ │ │ │ • Mission plan │ │ • AI/ML training │ │ • Analytics │ │ │ │ • SLAM map view │ │ • SLAM map store │ │ • SLAM map mgmt │ │ │ │ • Manual control │ │ • Map tiles │ │ • Alerts │ │ │ └────────┬─────────┘ └────────┬─────────┘ └──────────┬──────────┘ │ │ │ │ │ │ └───────────┼───────────────────────┼──────────────────────────┼──────────────┘ │ MAVLink │ MQTT/gRPC │ REST/WS │ (low-latency) │ (telemetry) │ (mgmt) ▼ ▼ ▼ ════════════════════════════════════════════════════════════════ ENCRYPTED RF DATA LINK (Microhard / Silvus / Custom) 1-50 Mbps • 12km range • AES-256 ════════════════════════════════════════════════════════════════ │ │ │ ▼ ▼ ▼ ┌─────────────────────────────────────────────────────────────────────────────┐ │ AIR SEGMENT (DRONE) │ │ │ │ ┌─────────────────────────────────────────────────────────────────────┐ │ │ │ COMPANION COMPUTER (Jetson Orin Nano) │ │ │ │ Linux • Multi-process │ │ │ │ │ │ │ │ ┌────────────────────────────────────────────────────────────┐ │ │ │ │ │ VIDEO / CAPTURE (Process 1) │ │ │ │ │ │ V4L2 → ISP → Raw Frames │ H.265 Encode → RTP → GCS │ │ │ │ │ │ (Mission Cam + Stereo Cam) │ Recording to SSD │ │ │ │ │ └──────────────┬───────────────┴─────────────────────────────┘ │ │ │ │ │ Shared Memory (zero-copy frames) │ │ │ │ ┌──────┴──────┐ │ │ │ │ ▼ ▼ │ │ │ │ ┌────────────┐ ┌────────────┐ ┌────────────┐ ┌────────────────┐ │ │ │ │ │ Perception │ │ SLAM / │ │ Mission │ │ Payload │ │ │ │ │ │ Pipeline │ │ VIO Nav │ │ Planner │ │ Manager │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ │ • Detect │ │ • SLAM │ │ • Plan │ │ • Discovery │ │ │ │ │ │ • Track │ │ • VIO │ │ • Avoid │ │ • Drivers │ │ │ │ │ │ • Classify │ │ • Map mgmt │ │ • Replan │ │ • Data pipes │ │ │ │ │ │ • Fuse │ │ • Loop cl. │ │ • Waypoint │ │ • Hot-plug │ │ │ │ │ └─────┬──────┘ └─────┬──────┘ └─────┬──────┘ └───────┬────────┘ │ │ │ │ │ │ │ │ │ │ │ │ └──────────────┴───────────────┴────────────────┘ │ │ │ │ Internal Bus (DDS / Lock-free queues) │ │ │ └─────────────────────────────┬───────────────────────────────────────┘ │ │ │ MAVLink v2 (UART 921600 baud) │ │ │ SLAM pose → VISION_POSITION_ESTIMATE │ │ ┌─────────────────────────────┴───────────────────────────────────────┐ │ │ │ FLIGHT CONTROLLER (STM32 / PX4) │ │ │ │ RTOS • Hard Real-Time │ │ │ │ │ │ │ │ ┌──────────┐ ┌──────────┐ ┌──────────┐ ┌──────────┐ ┌──────────┐ │ │ │ │ │ EKF │ │ Attitude │ │ Position │ │ Failsafe │ │ Motor │ │ │ │ │ │ State │ │ Control │ │ Control │ │ Manager │ │ Mixer │ │ │ │ │ │ Estimator│ │ (Rate+ │ │ (Pos+Vel)│ │ │ │ │ │ │ │ │ │ │ │ Angle) │ │ │ │ • Geofnc │ │ • PWM │ │ │ │ │ │ • IMU │ │ • PID │ │ • PID │ │ • Battery│ │ • DShot │ │ │ │ │ │ • GPS │ │ • 400Hz │ │ • 50Hz │ │ • Comms │ │ • 4-8 ch │ │ │ │ │ │ • Baro │ │ │ │ │ │ • Sensor │ │ │ │ │ │ │ │ • Mag │ │ │ │ │ │ │ │ │ │ │ │ │ │ • SLAM │ │ │ │ │ │ │ │ │ │ │ │ │ │ (ext.) │ │ │ │ │ │ │ │ │ │ │ │ │ └──────────┘ └──────────┘ └──────────┘ └──────────┘ └──────────┘ │ │ │ └──────────────────────────────────────────────────────────────────────┘ │ │ │ │ ┌────────────────────────────────────────────────────────────────────┐ │ │ │ HARDWARE / SENSORS │ │ │ │ IMU GPS Baro Mag StereoCamera MissionCam LiDAR Radar ESC×4 Payload│ │ │ └────────────────────────────────────────────────────────────────────┘ │ └─────────────────────────────────────────────────────────────────────────────┘

## Key Architectural Decisions

- 1. SEPARATION OF CONCERNS — Flight Controller vs Companion Computer

The FC runs an RTOS (NuttX for PX4) and handles only flight-critical functions: state estimation, attitude/position control, motor mixing, and failsafes. It runs at up to 400Hz with hard real-time guarantees. If the companion computer crashes, the FC continues flying safely.

- 2. SLAM RUNS ON COMPANION, FEEDS INTO FC's EKF

SLAM is computationally expensive — it cannot run on the FC's MCU. It runs on the companion computer and sends corrected pose estimates to the FC via MAVLink (VISION_POSITION_ESTIMATE message). The FC's EKF treats SLAM as just another sensor source — fusing it with IMU, GPS (when available), and barometer. This creates a clean boundary: SLAM provides position, the FC decides what to do with it. ( sends position + orientation + covariance at 30Hz)

- 3. CLEAN PROTOCOL BOUNDARY — MAVLink v2

The FC and companion communicate via MAVLink v2 over UART or Ethernet. This creates a well-defined interface: the companion sends position estimates and high-level commands, the FC executes them. You can swap PX4 for ArduPilot without changing the companion software.

- 4. LINUX ON THE COMPANION — Full OS Capabilities

The companion runs embedded Linux, giving us: GPU acceleration (CUDA for inference and SLAM), mature networking stack, V4L2 camera drivers, containerization options, and the full C++ standard library.

About UKF vs EKF

The key insight: on an STM32 at 200Hz, compute budget matters enormously. The EKF2 in PX4 has 24 states (position, velocity, orientation, gyro bias, accel bias, mag bias, wind). A UKF would need to propagate 49 sigma points through the full nonlinear dynamics model every 5ms. On a 480MHz MCU with no FPU pipeline, that's tight.

When UKF Actually Matters

The EKF's linearization error is significant when:

The system is highly nonlinear (aggressive aerobatic maneuvers, >60° roll)

The time step is large (linearization error grows with Δt)

The uncertainty is large (the distribution is far from Gaussian)

For a drone in normal flight:

Attitude changes between EKF updates (5ms) are tiny — maybe 0.5°

The system is approximately linear at this timescale

EKF linearization error is negligible

The Practical Answer for the Interview

"The UKF captures second-order nonlinear effects better than the EKF, so it's theoretically more accurate. But on an STM32 flight controller running at 200Hz with 24 states, the UKF's compute cost — propagating 49 sigma points through the full dynamics — would consume most of the CPU budget. The EKF's linearization error is negligible at 5ms timesteps during normal flight. Both PX4 and ArduPilot use EKF, which means it's battle-tested across millions of flights. I'd only consider UKF for a companion-computer-side estimator where we have more compute — for example, a UKF for SLAM's IMU preintegration could be beneficial. But on the FC, the EKF is the right call."

Where You COULD Use UKF in This Architecture

On the companion computer (Jetson Orin, plenty of compute):

- SLAM's IMU preintegration — UKF could give better rotation estimates during fast motion

- Object tracking — if tracked objects move nonlinearly (e.g., a vehicle making a sharp turn), a UKF tracker handles the nonlinearity better than an EKF tracker

- Sensor fusion in the perception pipeline — fusing camera + LiDAR + radar with nonlinear observation models

So the architecture could actually use both: EKF on the FC (proven, fast, sufficient), UKF on the companion for perception tasks where accuracy matters more than microseconds.

# 3. Flight Controller Software Layer

Figure 2: Flight Controller — SLAM as EKF Sensor Source

Sensor Inputs Motor Outputs ┌─────┬─────┬─────┐ ┌──────────────────┐ │ IMU │ GPS │Baro │ Mag │ Motor Mixer │ │200Hz│10Hz │50Hz │ 50Hz │ │ └──┬──┴──┬──┴──┬──┘ │ roll/pitch/yaw/ │ │ │ │ │ thrust → PWM×4 │ │ │ │ ┌───────────────────┐ └────────┬─────────┘ │ │ │ │ SLAM/VIO Pose │ │ │ │ │ │ (from companion) │ │ │ │ │ │ 30Hz via MAVLink │ │ │ │ │ │VISION_POSITION_ │ │ │ │ │ │ESTIMATE msg │ │ │ │ │ └───────┬───────────┘ │ │ │ │ │ │ ▼ ▼ ▼ ▼ │ ┌──────────────────────────────┐ ┌──────────────────┐ │ │ EKF2 / EKF3 │ │ Position │ │ │ State Estimator │───▶│ Controller │────┐ │ │ │ (50Hz) │ │ │ Fuses ALL sources: │ │ │ ▼ │ • IMU (always) │ │ Pos error → Vel │ ┌──────────┐ │ • GPS (when available) │ │ Vel error → Accel │ │ Attitude │ │ • Barometer (always) │ └──────────────────┘ │ Controller│ │ • Magnetometer (always) │ │ (400Hz) │ │ • SLAM pose (when available) │ ┌──────────────────┐ │ │ │ │ │ Failsafe │ │Angle→Rate│ │ GPS + SLAM available: │ │ Manager │►│Rate→Torq │ │ → weight both, cross-check │ │ │ └──────────┘ │ GPS lost, SLAM available: │ │ • GPS lost → SLAM │ │ → seamlessly rely on SLAM │ │ • SLAM drift high │ │ Both lost: │ │ → altitude hold │ │ → IMU dead-reckoning only │ │ • Comms lost → RTL│ │ → uncertainty grows fast │ │ • Batt low → Land │ └──────────────────────────────┘ └──────────────────┘

## How SLAM Enters the FC's State Estimator

The FC's EKF doesn't know or care that the position came from SLAM. It receives a MAVLink VISION_POSITION_ESTIMATE message containing x, y, z, roll, pitch, yaw in the local NED frame. The EKF fuses this just like a GPS measurement — with a configurable noise covariance. When SLAM confidence is high, the EKF weights it heavily. When SLAM reports high uncertainty (featureless environment, motion blur), the EKF reduces its weight.

## Control Loop Timing

| Loop | Rate | Period | Max Jitter |
| --- | --- | --- | --- |
| IMU Sampling | 1000 Hz | 1 ms | < 50 µs |
| Attitude/Rate Control | 400 Hz | 2.5 ms | < 100 µs |
| Position/Velocity Control | 50 Hz | 20 ms | < 1 ms |
| EKF State Estimation | 200 Hz | 5 ms | < 500 µs |
| SLAM Pose Ingestion | 30 Hz | 33 ms | < 5 ms |
| Failsafe Check | 10 Hz | 100 ms | < 10 ms |
| MAVLink TX/RX | 50 Hz | 20 ms | < 5 ms |

⚡ Interview Tip: Know these numbers. If the interviewer asks 'what rate does the attitude controller run at?' and you say '400Hz' confidently, it signals real domain knowledge. Also note SLAM at 30Hz — this is much slower than the IMU but the EKF interpolates between SLAM updates.

┌────────────────────────────────────────────────────────────────┐ │ COMPANION COMPUTER (Linux) │ │ │ │ Process 1: VIDEO / CAPTURE ┌───── To GCS (RF Link) ──► │ │ ┌──────────────────────────────────┐ │ │ │ │ Thread: Mission Cam (V4L2) │ │ Encodes H.265 → RTP │ │ │ Thread: Stereo Cam (V4L2) │ │ Adaptive bitrate │ │ │ Thread: ISP / Preprocessing │ │ FEC encoding │ │ │ Thread: H.265 Encode → Stream │ │ Recording to SSD │ │ │ Thread: SSD Recording │ │ │ │ └───────────┬──────────────────────┘ │ │ │ │ Raw frames via shared memory (zero-copy) │ │ │ │ │ ┌──────┴──────┐ │ │ ▼ ▼ │ │ Process 2: PERCEPTION Process 3: SLAM / VIO NAV │ │ ┌────────────────────────┐ ┌──────────────────────────┐ │ │ │ Thread: LiDAR Capture │ │ Thread: Visual Frontend │ │ │ │ Thread: Radar Capture │ │ Thread: IMU Reader │ │ │ │ Thread: Inference │ │ Thread: Backend Optim. │ │ │ │ Thread: Tracker │ │ Thread: Loop Closure │ │ │ │ Thread: Sensor Fusion │ │ Thread: Map Management │ │ │ └───────────┬────────────┘ └────────────┬─────────────┘ │ │ │ │ │ │ │ │ SLAM pose → FC │ │ ▼ ▼ │ │ ┌───────────────────────────────────────────────────────────────────┐ │ │ │ SHARED DATA BUS / MESSAGE BROKER │ │ │ │ (Lock-free queues / DDS / Custom pub-sub) │ │ │ └───────────────────────────────────────────────────────────────────┘ │ │ │ │ │ │ ▼ ▼ │ │ Process 4: MISSION PLANNER Process 5: COMMS │ │ ┌──────────────────────────┐ ┌──────────────────────────────┐ │ │ │ Thread: Path Planner │ │ Thread: MAVLink RX/TX │ │ │ │ A*/D* search on │ │ Serial UART to FC │ │ │ │ SLAM occupancy grid │ │ Parse/build MAVLink msgs │ │ │ │ │ │ │ │ │ │ Thread: Obstacle Avoid │ │ Thread: GCS Link │ │ │ │ Real-time replan at │ │ UDP to/from GCS │ │ │ │ 10Hz from perception │ │ Command relay, telemetry │ │ │ │ object list │ │ Video stream control │ │ │ │ │ │ │ │ │ │ Thread: Mission Exec │ │ Thread: Cloud Uplink │ │ │ │ Waypoint sequencing │ │ MQTT publish telemetry │ │ │ │ Geofence enforcement │ │ Receive OTA commands │ │ │ │ Takeoff/land/RTL │ │ Heartbeat / keep-alive │ │ │ │ State machine │ │ │ │ │ └──────────────────────────┘ │ Thread: SLAM Pose Fwd │ │ │ │ Receive SLAM pose from │ │ │ │ data bus, package into │ │ │ │ VISION_POSITION_ESTIMATE │ │ │ │ Send to FC at 30Hz │ │ │ └──────────────────────────────┘ │ │ │ │ Process 6: PAYLOAD MGR Process 7: SYS MONITOR │ │ ┌──────────────────────────┐ ┌───────────────────────────────┐ │ │ │ Thread: Discovery │ │ Thread: Resource Monitor │ │ │ │ USB/SPI/I2C scan for │ │ Poll CPU/GPU/RAM/temp │ │ │ │ new payload devices │ │ per-process via cgroups │ │ │ │ Hot-plug events │ │ Publish metrics to bus │ │ │ │ │ │ │ │ │ │ Thread: Plugin Runtime │ │ Thread: Watchdog │ │ │ │ Load .so plugins for │ │ Heartbeat check all 7 │ │ │ │ each payload type │ │ processes every 1s │ │ │ │ Camera, multispectral │ │ sd_notify() integration │ │ │ │ gas sensor, dropper │ │ Trigger systemd restart │ │ │ │ │ │ on missed heartbeat │ │ │ │ Thread: Data Pipe │ │ │ │ │ │ Route payload data │ │ Thread: Logger / OTA │ │ │ │ to recording + cloud │ │ Structured logging (JSON) │ │ │ │ Format conversion │ │ Log rotation + compression │ │ │ │ Bandwidth throttle │ │ OTA update agent (RAUC) │ │ │ └──────────────────────────┘ │ A/B partition switching │ │ │ └───────────────────────────────┘ │ └───────────────────────────────────────────────────────────────────────────┘

systemd starts each as an independent service:

- perception.service

- slam.service

- mission-planner.service

- video-pipeline.service

- payload-manager.service

- comms.service

- system-monitor.service

Each is a native C++ binary. If one crashes, systemd restarts it. The others continue running.

Example of cgroups – resource limits with out containers

# Limit SLAM to 2 CPU cores and 1.5GB RAM — no Docker needed

# /etc/systemd/system/slam.service

[Service]

ExecStart=/opt/drone/bin/slam

CPUAffinity=0 1            # Pin to cores 0-1

MemoryMax=1536M             # Hard memory limit

MemoryHigh=1024M            # Start reclaiming at 1GB

CPUQuota=200%               # Max 2 cores worth of CPU

Restart=always              # Auto-restart on crash

RestartSec=100ms            # Restart within 100ms

WatchdogSec=5               # Killed if no heartbeat for 5s

Example of name spaces – filesystem isolation

# If you really want filesystem isolation:

ProtectSystem=strict        # Read-only root filesystem

PrivateTmp=true             # Isolated /tmp

ReadWritePaths=/data/slam   # Only this path is writable

Answer: On the drone itself, we don't containerize — Docker adds memory overhead, startup latency, and complicates GPU access and shared-memory IPC, which we rely on for zero-copy image transfer between SLAM and perception. Instead, we use native Linux process isolation: separate systemd services with cgroup resource limits, CPU affinity pinning, watchdog timers, and auto-restart policies. We get the same isolation guarantees Docker provides, without the overhead. Docker is great for our cloud backend and CI/CD pipeline — just not on a 15-watt embedded computer where every millisecond and megabyte counts."

## Why SLAM Gets Its Own Process

SLAM is separated from the Perception pipeline into its own process for three reasons:

1. FAULT ISOLATION: If SLAM crashes (out-of-memory during map growth, assertion in optimization), the perception pipeline continues providing object detections. The FC falls back to GPS or IMU dead-reckoning.

2. INDEPENDENT SENSORS: The Video/Capture process owns all cameras and distributes raw frames via shared memory. SLAM consumes stereo frames + IMU. Perception consumes mission camera frames + LiDAR + radar. Different consumer pipelines, different timing, different failure modes.

3. RESOURCE ISOLATION: SLAM's backend optimization is CPU/GPU-intensive and bursty (loop closure can spike to 100% for ~200ms). Running it in a separate process with cgroup limits prevents it from starving the perception pipeline or the video capture process.

## Inter-Process Communication Choices

| Method | Latency | Throughput | Use Case |
| --- | --- | --- | --- |
| Lock-free shared memory (iceoryx / custom) | < 1 µs | Highest | Image/point cloud transfer between perception stages |
| DDS (Micro-XRCE-DDS) | ~100 µs | High | SLAM pose → Mission Planner Typed messages between subsystems |
| UNIX domain sockets | ~10 µs | Medium | Command/control messages (mission commands) |
| MAVLink (serial/UDP) | ~1 ms | Low | FC ↔ Companion SLAM pose → FC EKF |

Integration test example for using docker :

# .gitlab-ci.yml — test SLAM with simulated flight

slam-integration-test:

services:

- name: px4-sitl:v1.15.2        # Simulated flight controller

- name: gazebo:11.14             # 3D world with visual features

script:

- ./build.sh                     # Build your companion code

- ./bin/slam &                   # Start SLAM process

- sleep 2                        # Wait for initialization

- python3 test_slam_accuracy.py  # Fly simulated mission, check SLAM drift

artifacts:

paths:

- slam_trajectory.csv          # Save for analysis if test fails

# 5. Perception & Autonomy Pipeline

Figure 4: Perception Pipeline + SLAM Integration

┌──────────┐ ┌──────────┐ ┌──────────┐ ┌───────────────┐ │ Mission │ │ LiDAR │ │ Radar │ │ Stereo Camera │ │ Camera │ │ 10 Hz │ │ 20 Hz │ │ (for SLAM) │ │ 30 fps │ │ 200K pts │ │ Targets │ │ 30 fps │ └────┬─────┘ └────┬─────┘ └────┬─────┘ └──────┬────────┘ │ │ │ │ ▼ ▼ ▼ │ ┌──────────┐ ┌──────────┐ ┌──────────┐ │ │Undistort │ │ Filter │ │ Parse │ │ │ Rectify │ │ Voxelize │ │ Cluster │ │ └────┬─────┘ └────┬─────┘ └────┬─────┘ │ │ │ │ │ ▼ │ │ ▼ ┌──────────┐ │ │ ┌────────────────┐ │ Object │ │ │ │ SLAM / VIO │ │Detection │ │ │ │ Process │ │(YOLOv8) │ │ │ │ │ │ GPU/DLA │ │ │ │ (See Section 6 │ └────┬─────┘ │ │ │ for details) │ │ │ │ └───────┬────────┘ │ │ │ │ ▼ ▼ ▼ │ ┌──────────────────────────────────────┐ │ │ SENSOR FUSION (EKF) │ │ │ │ │ │ Camera detections (2D bbox + class) │ │ │ + LiDAR clusters (3D position) │ SLAM provides:│ │ + Radar targets (range + velocity) │ • Drone pose │ │ = Fused object list │ • 3D map │ │ (3D pos, velocity, class, ID) │ • Occupancy │ └──────────────────┬───────────────────┘ │ │ │ │ │ ▼ │ │ ┌──────────────────────────────────────┐ │ │ │ MULTI-OBJECT TRACKER │ │ │ │ │ │ │ │ • Hungarian algorithm (assignment) │ │ │ │ • Kalman filter (per-object state) │ │ │ │ • Track lifecycle (init/confirm/ │ │ │ │ coast/delete) │ │ │ │ • Track ID persistence │ │ │ └──────────────────┬───────────────────┘ │ │ │ │ │ ▼ ▼ │ ┌──────────────────────────────────────────────────┐ │ │ WORLD MODEL (OUTPUT) │ │ │ │ │ │ From Perception: From SLAM: │ │ │ • Object list with IDs • 3D occupancy map │ │ │ • 3D position + velocity • Drone pose (6DOF) │ │ │ • Object class + conf. • Traversability map │ │ │ • Free-space estimation • Visual landmarks │ │ └──────────────────┬──────────────┬────────────────┘ │ │ │ │ ┌────────┴──────┐ ┌────┴───────┐ ┌──────────┴──┐ ▼ ▼ ▼ ▼ ▼ │ ┌──────────────┐ ┌──────────────┐ ┌──────────────┐ │ │ Obstacle │ │ Mission │ │ FC EKF │ │ │ Avoidance │ │ Planner │ │ (via │ │ │ (reactive) │ │ (global) │ │ MAVLink) │ │ │ │ │ │ │ │ │ │ Uses SLAM │ │ Uses SLAM │ │ VISION_POS │ │ │ occupancy │ │ 3D map for │ │ _ESTIMATE │ │ │ for local │ │ path plan │ │ message │ │ │ avoidance │ │ in known env │ │ @30Hz │ │ └──────────────┘ └──────────────┘ └──────────────┘

## How Perception and SLAM Interact

Perception and SLAM are complementary but independent:

• PERCEPTION answers: "What objects are around me?" (dynamic: people, vehicles, obstacles)

• SLAM answers: "Where am I?" and "What does the static environment look like?" (walls, buildings, terrain)

• Both feed into the WORLD MODEL, which the mission planner and obstacle avoidance consume

• SLAM also sends its pose estimate to the FC for flight control — this is the critical path for GNSS-denied flight

## Timing Budget (30 fps camera)

| Stage | Target Latency | Notes |
| --- | --- | --- |
| Image capture + undistort | 2 ms | DMA + GPU ISP pipeline |
| Object detection (inference) | 15 ms | TensorRT FP16, Orin DLA |
| LiDAR preprocessing | 3 ms | Voxel grid filter, ground removal |
| Sensor fusion | 2 ms | EKF predict + update per object |
| Multi-object tracking | 3 ms | Hungarian + Kalman update |
| Obstacle avoidance | 2 ms | Potential field or VFH+ |
| SLAM frontend (parallel) | 10 ms | Feature extraction + tracking (own thread) |
| SLAM backend (async) | 50-200 ms | Bundle adjustment, loop closure (bursty) |
| Perception TOTAL | ~27 ms | 6 ms headroom (18%) |

Note: SLAM runs on its own process/threads and does NOT block the perception pipeline. The SLAM frontend (feature tracking) runs at camera rate (30Hz, ~10ms). The SLAM backend (optimization, loop closure) runs asynchronously and may take 50-200ms — that's fine because it doesn't need to run every frame.

# 6. SLAM & GNSS-Denied Navigation

This section covers the SLAM system in detail — architecture, algorithms, how it integrates with the rest of the drone software, and the key design decisions.

## 6.1 Why SLAM Matters for 24 Industries

The job posting lists "GNSS-denied navigation" as a nice-to-have. For a tactical drone operating in defense scenarios, GPS can be:

• JAMMED — adversary uses a GPS jammer (common in military scenarios)

• SPOOFED — adversary sends fake GPS signals to misdirect the drone

• BLOCKED — urban canyons, indoor operations, under dense canopy

• UNAVAILABLE — operations in areas with poor satellite coverage

Without an alternative navigation solution, GPS denial = drone is blind. SLAM provides that alternative: a self-contained navigation system that needs only a camera and IMU — sensors the drone already has.

## 6.2 VIO vs SLAM — When to Use Which

|  | VIO (Visual-Inertial Odometry) | Visual SLAM |
| --- | --- | --- |
| What it does | Estimates relative motion (how much I moved) | Estimates position + builds a map of the environment |
| Builds a map? | No | Yes — sparse or dense 3D map |
| Loop closure? | No — drift accumulates forever | Yes — recognizes revisited places, corrects all drift at once |
| Drift | ~0.5-2% of distance traveled (100m flight → 0.5-2m error) | Bounded by loop closures (can be < 10cm over km) |
| Compute cost | Low (runs on RPi4) | Medium-High (needs Jetson-class) |
| Use case | Short flights, one-way missions, GPS intermittent | Long flights, revisiting areas, indoor mapping, full GPS denial |
| Can pre-load maps? | No | Yes — localize in a previously built map |
| Example algorithms | MSCKF, ROVIO, OpenVINS VINS-Mono | ORB-SLAM3, VINS-Fusion LSD-SLAM, Kimera |

For Twentyfour Industries: start with VIO (simpler, more robust), then add full SLAM for missions that require it. The architecture should support both from day one — same interface to the FC, just swap the navigation module.

## 6.3 SLAM System Architecture

Figure 5: SLAM System Internal Architecture — Frontend/Backend Split

┌──────────────────────────────────────────────────────────────────────────┐ │ SLAM PROCESS (Companion Computer) │ │ │ │ ┌────────────────┐ ┌────────────────┐ │ │ │ Stereo Camera │ │ IMU │ │ │ │ 30 fps │ │ 200-1000 Hz │ │ │ │ 640×480 stereo │ │ accel + gyro │ │ │ └───────┬────────┘ └───────┬────────┘ │ │ │ │ │ │ ▼ ▼ │ │ ┌──────────────────────────────────────────┐ │ │ │ FRONTEND (real-time, 30Hz) │ │ │ │ │ │ │ │ 1. Feature Extraction │ │ │ │ • ORB features (fast, rotation- │ │ │ │ invariant) │ │ │ │ • Detect ~1000 features per frame │ │ │ │ • ~3ms on GPU │ │ │ │ │ │ │ │ 2. Feature Matching / Tracking │ │ │ │ • Track features across frames │ │ │ │ • Lucas-Kanade optical flow OR │ │ │ │ descriptor matching │ │ │ │ • Reject outliers (RANSAC) │ │ │ │ • ~4ms │ │ │ │ │ │ │ │ 3. IMU Preintegration │ │ │ │ • Integrate IMU between keyframes │ │ │ │ • Provides rotation + velocity prior │ │ │ │ • Handles fast motion between frames │ │ │ │ • ~1ms │ │ │ │ │ │ │ │ 4. Pose Estimation │ │ │ │ • PnP (Perspective-n-Point) solve │ │ │ │ • Fuse visual + IMU constraints │ │ │ │ • Output: camera pose (6DOF) │ │ │ │ • ~2ms │ │ │ │ │ │ │ │ OUTPUT: Pose estimate @ 30Hz (~10ms) │ │ │ └──────────────┬───────────────────────────┘ │ │ │ │ │ │ Keyframe? (every ~5-10 frames) │ │ │ │ │ ▼ │ │ ┌──────────────────────────────────────────┐ │ │ │ BACKEND (async, runs in parallel) │ │ │ │ │ │ │ │ 5. Keyframe Selection │ │ │ │ • New keyframe if: enough parallax, │ │ │ │ enough new features, or time limit │ │ │ │ │ │ │ │ 6. Local Bundle Adjustment │ │ │ │ • Optimize last N keyframe poses │ │ │ │ + 3D point positions jointly │ │ │ │ • Minimizes reprojection error │ │ │ │ • ~50ms for N=10 keyframes │ │ │ │ │ │ │ │ 7. Loop Closure Detection │ │ │ │ • DBoW2 or NetVLAD: "Have I been │ │ │ │ here before?" │ │ │ │ • Compare current visual descriptor │ │ │ │ against all previous keyframes │ │ │ │ • ~20ms per check │ │ │ │ │ │ │ │ 8. Pose Graph Optimization │ │ │ │ (triggered by loop closure) │ │ │ │ • Correct ALL keyframe poses to be │ │ │ │ globally consistent │ │ │ │ • Uses g2o or GTSAM optimizer │ │ │ │ • ~100-200ms (can be expensive) │ │ │ │ │ │ │ └──────────────┬───────────────────────────┘ │ │ │ │ │ ▼ │ │ ┌──────────────────────────────────────────┐ │ │ │ MAP MANAGER │ │ │ │ │ │ │ │ • Stores keyframes + 3D points (sparse) │ │ │ │ • Optional: dense/occupancy map for │ │ │ │ obstacle avoidance │ │ │ │ • Map serialization (save/load for │ │ │ │ revisiting known environments) │ │ │ │ • Memory management (cull old keyframes │ │ │ │ to bound memory usage) │ │ │ │ • Map upload to cloud for fleet sharing │ │ │ └──────────────────────────────────────────┘ │ │ │ │ OUTPUTS: │ │ ┌────────────────────────────────────────────────────────────────┐ │ │ │ → FC: Pose (x,y,z,qw,qx,qy,qz) via MAVLink @ 30Hz │ │ │ │ → Mission Planner: 3D map + pose for path planning │ │ │ │ → Obstacle Avoidance: occupancy grid for local avoidance │ │ │ │ → Cloud: map upload for persistence / fleet sharing │ │ │ │ → GCS: live SLAM map visualization for operator │ │ │ └────────────────────────────────────────────────────────────────┘ │ └──────────────────────────────────────────────────────────────────────────┘

## 6.4 The Frontend/Backend Split — Why It Matters

This is the most important architectural decision in SLAM. The system is split into:

FRONTEND (real-time, every frame): Feature extraction, tracking, IMU integration, and pose estimation. Must complete within 10ms at 30fps. If the frontend is slow, frames are dropped and tracking quality degrades.

BACKEND (asynchronous, periodic): Bundle adjustment and loop closure. These are computationally expensive but don't need to run every frame. The backend runs in a separate thread and updates the map asynchronously. When a loop closure happens, the backend corrects all past poses — and the frontend seamlessly uses the corrected map.

This split means the drone always has a real-time pose estimate (from the frontend) even while the backend is doing heavy optimization. No jitter, no dropped frames.

## 6.5 Loop Closure — What It Is and Why It's Critical

Figure 6: Loop Closure Corrects Accumulated Drift

WITHOUT LOOP CLOSURE (VIO only): Start ●──────────────────────────────────┐ │ Actual path is a square: │ 100m × 100m │ │ ●───────────────────● End │ ↑ │ │ 2m drift accumulated │ │ (not back at start!) │ │ │ ●─────────────────────────────────────────● WITH LOOP CLOSURE (SLAM): Start ●──────────────────────────────────┐ │ │ SLAM recognizes: "I've been here before!" │ Triggers pose graph optimization │ │ │ End ● ←── corrected back to Start │ │ │ ●─────────────────────────────────────────● All intermediate poses corrected retroactively. Final error: < 10cm (instead of 2m)

Loop closure works by maintaining a visual vocabulary (bag-of-words, e.g., DBoW2). Each keyframe is described by its visual features compressed into a compact descriptor. When a new keyframe's descriptor is similar to an old keyframe, a loop closure candidate is detected. The system then verifies geometrically (feature matching + PnP) and, if confirmed, adds a constraint to the pose graph. The optimizer (g2o/GTSAM) then adjusts all poses to satisfy this new constraint — correcting the entire trajectory.

## 6.6 Pre-Built Maps — Localize in Known Environments

For defense operations, the operating area is often surveyed in advance. SLAM enables:

1. MAPPING FLIGHT: Fly the area once (manually or autonomously), build a SLAM map, save it.

2. UPLOAD TO CLOUD: Map is stored in the fleet management cloud.

3. DEPLOY TO DRONES: Before a mission in that area, download the map to the drone.

4. LOCALIZATION-ONLY MODE: Drone doesn't build a new map — it localizes within the existing one. Faster, more accurate, lower compute.

This is extremely valuable for repeated operations in the same area (e.g., perimeter patrol, building inspection, urban surveillance). The drone has centimeter-level positioning from the first second, with no drift accumulation.

## 6.7 Algorithm Choices for a Drone

| Algorithm | Type | Pros | Cons |
| --- | --- | --- | --- |
| ORB-SLAM3 | Visual-Inertial SLAM (sparse) | • Most complete open-source SLAM • Mono/stereo/RGB-D + IMU • Loop closure + map reuse • Well documented | • CPU-heavy backend • Struggles with low texture • GPL license (problem for commercial/defense) |
| VINS-Fusion | Visual-Inertial Odometry/SLAM | • Excellent VIO accuracy • Stereo + IMU • Loop closure support • Widely used in drones | • Less robust map reuse than ORB-SLAM3 • Can struggle in pure rotation |
| Kimera | Metric-Semantic SLAM | • Dense metric mesh output • Semantic labels on map • Good for planning | • Higher compute cost • Academic, less production- ready |
| OpenVINS | Visual-Inertial Odometry | • Very modular, configurable • Great for research/tuning • Well maintained | • VIO only — no loop closure • No map reuse • Would need SLAM added on top |
| Custom / Proprietary | Depends on implementation | • Tailored to your hardware • No license issues • Optimized for your use case | • High development cost • Long timeline to match open-source quality |

⚡ Interview Tip: For the interview: recommend VINS-Fusion as the starting point (battle-tested in drones, good VIO + loop closure). Mention ORB-SLAM3 as the most feature-complete but flag the GPL license issue for a defense product. Show awareness that you may need to build custom or use a commercially licensed solution long-term.

## 6.8 SLAM Failure Modes & Mitigations

| Failure Mode | Cause | Mitigation |
| --- | --- | --- |
| Tracking lost | Fast rotation, motion blur, featureless scene (white wall, sky, snow) | IMU preintegration bridges gaps. Relocalization from map. Fall back to VIO / IMU-only. |
| Scale drift (mono camera) | Monocular camera cannot observe metric scale | Use STEREO camera (solves this) or fuse with IMU (also solves it). Never use mono-only for drones. |
| Loop closure false positive | Perceptual aliasing — two different places look similar | Geometric verification after visual match. Require minimum inlier count before accepting. |
| Map grows too large | Long mission, many keyframes, OOM on embedded device | Keyframe culling: remove redundant keyframes. Bound max map size. Sliding window. |
| Vibration | Drone propellers cause image blur + IMU noise | Vibration-dampened camera mount. IMU low-pass filter. Use global shutter camera (not rolling). |
| Dynamic objects | Moving people/vehicles create false features that confuse SLAM | Mask detected objects (from perception pipeline) before feature extraction. Use only static feature points. |

## 6.9 SLAM → FC Integration (MAVLink)

SLAM → FC Bridge (MAVLink VISION_POSITION_ESTIMATE):

// slam_bridge.cpp — sends SLAM pose to FC via MAVLink

// This is the critical interface between SLAM and flight control

#include <mavsdk/mavsdk.h>

#include <mavsdk/plugins/mocap/mocap.h>  // "motion capture" — used for external pose

class SLAMToFCBridge {

mavsdk::Mocap& mocap_;

public:

// Called by SLAM frontend at 30Hz with each new pose estimate

void onSLAMPose(const SLAMPose& pose) {

mavsdk::Mocap::VisionPositionEstimate vpe;

// Convert SLAM frame (camera) → NED (flight controller)

// This rotation depends on how the camera is mounted

vpe.position_body.x_m = pose.z;    // SLAM z → NED x (forward)

vpe.position_body.y_m = -pose.x;   // SLAM x → NED -y (right)

vpe.position_body.z_m = -pose.y;   // SLAM y → NED -z (down)

// Quaternion rotation (camera frame → NED)

vpe.angle_body.roll_rad  = pose.roll;

vpe.angle_body.pitch_rad = pose.pitch;

vpe.angle_body.yaw_rad   = pose.yaw;

// Covariance — tells EKF how much to trust this estimate

// Lower values = EKF trusts SLAM more

// When SLAM tracking is degraded, increase these values

vpe.pose_covariance.covariance_matrix = {

pose.cov_x,  0, 0, 0, 0, 0,   // position uncertainty

0, pose.cov_y,  0, 0, 0, 0,

0, 0, pose.cov_z,  0, 0, 0,

0, 0, 0, pose.cov_roll,  0, 0,  // orientation uncertainty

0, 0, 0, 0, pose.cov_pitch, 0,

0, 0, 0, 0, 0, pose.cov_yaw

};

vpe.time_usec = pose.timestamp_us;

mocap_.set_vision_position_estimate(vpe);

// FC receives this and fuses it into EKF2 alongside GPS, IMU, baro

}

// Called when SLAM tracking is lost

void onTrackingLost() {

// Stop sending poses — FC will gradually increase position uncertainty

// and may enter failsafe if no other position source is available

log_warning("SLAM tracking lost — FC falling back to other sources");

}

};

Key detail: the covariance values are critical. When SLAM is tracking well (lots of features, no motion blur), send low covariance → FC trusts SLAM heavily. When SLAM is degraded (few features, fast motion), send high covariance → FC relies more on IMU and other sensors. This adaptive weighting is handled automatically by the FC's EKF — you just need to report honest uncertainty.

## 6.10 Navigation Fallback Chain

Figure 7: Navigation Fallback Chain — GPS → SLAM → VIO → IMU → Land

POSITION SOURCE PRIORITY (best → worst) ┌─────────────────────────────────────────────────────────┐ │ Level 1: RTK-GPS + SLAM (full redundancy) │ │ Accuracy: 2cm Drift: none │ │ GPS cross-checks SLAM, SLAM cross-checks GPS │ │ If they disagree → flag anomaly (spoofing?) │ └───────────────────────┬─────────────────────────────────┘ │ GPS lost (jammed / indoor) ▼ ┌─────────────────────────────────────────────────────────┐ │ Level 2: SLAM only (full navigation, no GPS) │ │ Accuracy: ~10cm with loop closure │ │ Drift: bounded by loop closures │ │ Can navigate indefinitely in mapped areas │ └───────────────────────┬─────────────────────────────────┘ │ SLAM tracking lost (featureless / dark) ▼ ┌─────────────────────────────────────────────────────────┐ │ Level 3: VIO only (short-term relative navigation) │ │ Accuracy: ~1-2% of distance traveled │ │ Drift: accumulates continuously │ │ Usable for: return-to-last-known-good │ └───────────────────────┬─────────────────────────────────┘ │ VIO also lost (camera obscured / broken) ▼ ┌─────────────────────────────────────────────────────────┐ │ Level 4: IMU dead-reckoning + Barometer │ │ Accuracy: degrades rapidly (meters per second)│ │ Usable for: ~10 seconds of controlled flight │ │ Action: IMMEDIATE AUTO-LAND │ └───────────────────────┬─────────────────────────────────┘ │ IMU fails (should not happen — dual redundant) ▼ ┌─────────────────────────────────────────────────────────┐ │ Level 5: Motor kill (parachute deploy if equipped) │ │ Last resort — uncontrolled descent │ └─────────────────────────────────────────────────────────┘ KEY: The EKF handles all transitions SEAMLESSLY. No hard switches. As a source degrades, its weight decreases gradually in the EKF. The drone doesn't "notice" the switch — the math handles it smoothly.

⚡ Interview Tip: This fallback chain is a GREAT thing to draw in the interview. It shows you think in terms of graceful degradation, not just happy-path design. Every level has a defined accuracy, a defined trigger for fallback, and a defined action.

# 7. Modular Payload System Design

This is a key differentiator for 24 Industries — their "Modular Rail System for Payloads." The software must support hot-pluggable, mission-swappable payloads without recompiling.

Figure 8: Modular Payload System Architecture

┌───────────────────────────────────────────────────────────────────┐ │ PAYLOAD MANAGER (Process) │ │ │ │ ┌─────────────────┐ ┌──────────────────────────────────┐ │ │ │ Discovery │ │ Plugin Loader │ │ │ │ Service │ │ │ │ │ │ │ │ dlopen("payload_thermal.so") │ │ │ │ • I2C EEPROM │ │ → creates ThermalPayload* │ │ │ │ scan on boot │ │ │ │ │ │ • USB hotplug │ │ dlopen("payload_ew.so") │ │ │ │ (udev events) │ │ → creates EWPayload* │ │ │ │ • GPIO interrupt │ │ │ │ │ │ on rail detect │ │ Factory pattern: │ │ │ └────────┬──────────┘ │ PayloadID → .so path → instance │ │ │ │ └──────────────┬───────────────────┘ │ │ ▼ ▼ │ │ ┌──────────────────────────────────────────────────────────┐ │ │ │ PAYLOAD INTERFACE (Abstract Base) │ │ │ │ │ │ │ │ virtual Status initialize() = 0; │ │ │ │ virtual Status start() = 0; │ │ │ │ virtual Status stop() = 0; │ │ │ │ virtual PayloadInfo getInfo() = 0; │ │ │ │ virtual DataStream getDataStream() = 0; │ │ │ │ virtual Status handleCommand(const Command&) = 0; │ │ │ │ virtual HealthStatus checkHealth() = 0; │ │ │ │ virtual ~PayloadInterface() = default; │ │ │ └───────┬──────────┬──────────┬──────────┬─────────────────┘ │ │ │ │ │ │ │ │ ▼ ▼ ▼ ▼ │ │ ┌──────────┐ ┌──────────┐ ┌──────────┐ ┌──────────┐ │ │ │ RGB │ │ Thermal │ │ EW │ │ Delivery │ │ │ │ Camera │ │ Camera │ │ Payload │ │ Mech. │ │ │ │ .so │ │ .so │ │ .so │ │ .so │ │ │ └──────────┘ └──────────┘ └──────────┘ └──────────┘ │ │ │ │ ┌──────────────────────────────────────────────────────────┐ │ │ │ RESOURCE MANAGER │ │ │ │ │ │ │ │ • Power budget: 15W total → allocate per payload │ │ │ │ • Bandwidth: 10 Mbps RF → allocate per stream │ │ │ │ • CPU/GPU: limit via cgroups per payload process │ │ │ │ • Priority: mission-critical payload gets more resources │ │ │ └──────────────────────────────────────────────────────────┘ │ └───────────────────────────────────────────────────────────────────┘

## Payload Plugin Interface (C++)

Payload Interface Header:

// payload_interface.h — the contract all payloads must implement

#pragma once

#include <string>

#include <memory>

#include <cstdint>

#include <functional>

struct PayloadInfo {

std::string name;

std::string version;

uint16_t payload_id;

float max_power_watts;

float max_bandwidth_mbps;

};

enum class PayloadStatus { OK, Error, Degraded, NotReady };

struct PayloadHealth {

PayloadStatus status;

float temperature_c;

float power_draw_w;

std::string message;

};

class PayloadInterface {

public:

virtual ~PayloadInterface() = default;

// Lifecycle

virtual bool initialize() = 0;

virtual bool start() = 0;

virtual bool stop() = 0;

// Identity

virtual PayloadInfo getInfo() const = 0;

// Health monitoring

virtual PayloadHealth checkHealth() const = 0;

// Command handling (payload-specific commands from GCS)

virtual bool handleCommand(uint16_t cmd_id, const uint8_t* data, size_t len) = 0;

// Data output — payload pushes data to registered callbacks

using DataCallback = std::function<void(const uint8_t* data, size_t len, uint64_t ts)>;

virtual void registerDataCallback(DataCallback cb) = 0;

};

// Every .so plugin must export this C function (no name mangling)

extern "C" {

PayloadInterface* create_payload();

void destroy_payload(PayloadInterface* p);

}

Payload Manager — Dynamic Loading:

// payload_manager.cpp — dynamic loading

#include <dlfcn.h>

class PayloadManager {

struct LoadedPayload {

void* lib_handle;

std::unique_ptr<PayloadInterface, void(*)(PayloadInterface*)> instance;

};

std::vector<LoadedPayload> payloads_;

public:

bool loadPayload(const std::string& so_path) {

void* handle = dlopen(so_path.c_str(), RTLD_NOW);

if (!handle) { log_error(dlerror()); return false; }

auto create = (PayloadInterface*(*)())dlsym(handle, "create_payload");

auto destroy = (void(*)(PayloadInterface*))dlsym(handle, "destroy_payload");

if (!create || !destroy) { dlclose(handle); return false; }

PayloadInterface* raw = create();

if (!raw->initialize()) { destroy(raw); dlclose(handle); return false; }

payloads_.push_back({handle, {raw, destroy}});

return true;

}

~PayloadManager() {

for (auto& p : payloads_) {

p.instance->stop();

p.instance.reset();

dlclose(p.lib_handle);

}

}

};

# 8. Communication Architecture

Figure 9: Communication Stack

┌─────────────────────────────┐ │ COMMUNICATION LAYERS │ └─────────────────────────────┘ Layer 5: APPLICATION ┌────────────────────────────────────────────────────────────┐ │ GCS Commands Telemetry Video Cloud Sync │ │ (MAVLink) (MAVLink) (RTP) (MQTT/gRPC) │ └──────┬─────────────┬────────────┬─────────────┬───────────┘ │ │ │ │ Layer 4: MULTIPLEXING ┌──────┴─────────────┴────────────┴─────────────┴───────────┐ │ CHANNEL MULTIPLEXER │ │ │ │ Channel 0: Command/Control (MAVLink) Priority: HIGH │ │ Channel 1: Telemetry + SLAM pose Priority: MEDIUM │ │ Channel 2: Video (RTP/H.265) Priority: LOW │ │ Channel 3: Cloud sync / Map upload Priority: LOWEST │ └──────────────────────────┬─────────────────────────────────┘ │ Layer 3: LINK MANAGEMENT ┌──────────────────────────┴─────────────────────────────────┐ │ • Link quality monitoring (RSSI, SNR, loss %) │ │ • Adaptive bitrate for video │ │ • Link failover (primary RF → backup LTE) │ │ • Heartbeat timeout detection │ └──────────────────────────┬─────────────────────────────────┘ │ Layer 2: SECURITY ┌──────────────────────────┴─────────────────────────────────┐ │ • AES-256-GCM encryption (all channels) │ │ • HMAC authentication (anti-spoofing) │ │ • Frequency hopping (anti-jamming) │ │ • Key exchange: pre-shared or ECDH │ └──────────────────────────┬─────────────────────────────────┘ │ Layer 1: PHYSICAL ┌──────────────────────────┴─────────────────────────────────┐ │ Primary: Microhard pDDL (1.3-2.5 GHz, up to 50 Mbps) │ │ Backup: LTE modem (urban ops, cloud-only mode) │ │ Emergency: Iridium SBD (satellite, text-only, 340 bytes) │ └───────────────────────────────────────────────────────────┘

# 9. Cloud Connectivity & Fleet Management

Figure 10: Cloud Backend with SLAM Map Management

┌──────────────────────────────────────────────────────────────────────────────┐ │ CLOUD BACKEND │ │ │ │ ┌────────────────┐ ┌────────────────┐ ┌────────────────┐ │ │ │ API Gateway │ │ Auth Service │ │ MQTT Broker │ │ │ │ (REST + WS) │ │ (OAuth2/mTLS) │ │ (EMQX/HiveMQ)│ │ │ └───────┬────────┘ └───────┬────────┘ └───────┬────────┘ │ │ ▼ ▼ ▼ │ │ ┌──────────────────────────────────────────────────────────────┐ │ │ │ MICROSERVICES LAYER │ │ │ │ │ │ │ │ ┌──────────────┐ ┌──────────────┐ ┌──────────────┐ │ │ │ │ │ Telemetry │ │ Mission │ │ Fleet │ │ │ │ │ │ Ingestion │ │ Service │ │ Management │ │ │ │ │ └──────────────┘ └──────────────┘ └──────────────┘ │ │ │ │ │ │ │ │ ┌──────────────┐ ┌──────────────┐ ┌──────────────┐ │ │ │ │ │ OTA │ │ ML Model │ │ SLAM Map │ │ │ │ │ │ Update │ │ Registry │ │ Storage & │ │ │ │ │ │ Service │ │ │ │ Distribution│ │ │ │ │ │ │ │ • Store │ │ │ │ │ │ │ │ • Package │ │ • Version │ │ • Store maps│ │ │ │ │ │ • Stage │ │ • Deploy │ │ • Share maps│ │ │ │ │ │ • Rollout │ │ • A/B test │ │ • Version │ │ │ │ │ └──────────────┘ └──────────────┘ └──────────────┘ │ │ │ └──────────────────────────────────────────────────────────────┘ │ │ │ │ ┌──────────────────────────────────────────────────────────────┐ │ │ │ DATA LAYER │ │ │ │ TimescaleDB S3/MinIO PostgreSQL │ │ │ │ (time-series (video, logs, (missions, │ │ │ │ telemetry) ML models, fleet config, │ │ │ │ SLAM maps) map metadata) │ │ │ └──────────────────────────────────────────────────────────────┘ │ └──────────────────────────────────────────────────────────────────────────────┘ MQTT Topics: drone/{id}/telemetry ← position, battery, SLAM status @ 1Hz drone/{id}/slam/status ← SLAM tracking quality, loop closures drone/{id}/slam/map ← compressed map upload (on ground) fleet/{group}/slam/map → download shared map before mission

## SLAM Map Sharing Across Fleet

A powerful capability: drone A maps a building. The map is uploaded to the cloud. Before drone B's mission in the same area, the map is downloaded. Drone B has centimeter-level positioning from launch — no mapping phase needed. This is especially valuable for defense operations where multiple drones operate in the same area of interest.

# 10. Safety & Fault Tolerance Architecture

Figure 11: Fault Matrix — With SLAM Failover & Spoofing Detection

FAULT DETECTION & RESPONSE MATRIX ┌──────────────────────┬─────────────────────┬──────────────────┐ │ FAULT │ DETECTION │ RESPONSE │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ GPS lost │ EKF innovation check │ Switch to SLAM │ │ GPS + SLAM degraded │ Position uncertainty │ Switch to VIO │ │ GPS + SLAM + VIO lost │ Pos uncertainty high │ Alt hold + land │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ SLAM tracking lost │ Feature count < min │ Fall back to VIO │ │ │ Innovation too large │ Attempt reloc. │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ Single motor fail │ RPM/current feedback │ Reduce payload, │ │ │ │ emergency land │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ RF link lost < 30s │ Heartbeat timeout │ Continue mission │ │ RF link lost > 30s │ │ RTL (SLAM-aided) │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ Battery critical │ Voltage + current │ Immediate land │ │ Battery low │ model prediction │ RTL │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ Companion crash │ FC watchdog │ FC continues, │ │ │ │ GPS-only flight │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ SLAM process crash │ System monitor │ Restart SLAM, │ │ │ │ VIO bridge ~1s │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ Geofence breach │ Position vs polygon │ Hard stop at │ │ │ (FC-enforced) │ boundary │ ├──────────────────────┼─────────────────────┼──────────────────┤ │ GPS spoofing detected │ SLAM vs GPS disagree │ Trust SLAM, │ │ │ by > threshold │ alert operator │ └──────────────────────┴─────────────────────┴──────────────────┘

## GPS Spoofing Detection via SLAM

A unique advantage of running SLAM alongside GPS: spoofing detection. If an adversary sends fake GPS signals, SLAM will disagree with GPS (because SLAM uses cameras, not RF). If the two position sources diverge beyond a threshold, the system flags a spoofing attack and switches to SLAM-only navigation. This is a significant capability for defense applications.

Figure 12: Safety Priority Hierarchy

SAFETY HIERARCHY (what can override what) Priority 1 (highest): HARDWARE LIMITS ┌──────────────────────────────────────────┐ │ • Physical motor current limits (ESC) │ │ • Hardware watchdog timer (reboot) │ │ • Battery management system (cutoff) │ │ → Cannot be overridden by any software │ └─────────────────────┬────────────────────┘ │ Priority 2: FLIGHT CONTROLLER FAILSAFES ┌─────────────────────┴────────────────────┐ │ • Geofence (hard boundary in FC) │ │ • Maximum altitude / speed limits │ │ • Attitude limits (max tilt angle) │ │ • Auto-disarm on ground │ │ → Only configurable via secure config │ └─────────────────────┬────────────────────┘ │ Priority 3: AUTONOMY DECISIONS ┌─────────────────────┴────────────────────┐ │ • SLAM-aided obstacle avoidance │ │ • Return-to-launch decisions │ │ • GPS spoofing response │ │ • Low battery land site selection │ │ → Override by operator with confirmation│ └─────────────────────┬────────────────────┘ │ Priority 4 (lowest): OPERATOR COMMANDS ┌─────────────────────┴────────────────────┐ │ • Mission waypoints │ │ • Speed / altitude targets │ │ • Payload commands │ │ → Rejected if violates higher priority │ └──────────────────────────────────────────┘

# 11. Data Flow & Threading Model

Figure 13: Thread Architecture — SLAM + Perception Core Assignment

COMPANION COMPUTER — THREAD ARCHITECTURE (ALL PROCESSES) ═══════════════════════════════════════════════════════════════ PROCESS: SLAM / VIO Navigation ═══════════════════════════════════════════════════════════════ CPU Core 0: SLAM FRONTEND (pinned, SCHED_FIFO priority 85) ┌───────────────────────────────────────────────┐ │ while(running): │ │ stereo_pair = stereo_camera.capture() │ │ imu_samples = imu_buffer.drain() │ │ features = extract_ORB(stereo_pair) // 3ms │ │ matches = track_features(features) // 4ms │ │ imu_preint = preintegrate(imu_samples) │ │ pose = solve_PnP(matches, imu_preint) // 2ms│ │ publish_pose_to_FC(pose) // MAVLink │ │ if is_keyframe(pose): │ │ backend_queue.push(keyframe) │ └──────────────────────┬────────────────────────┘ │ Queue (when keyframe) CPU Core 0: SLAM BACKEND (same core, lower prio, SCHED_RR 60) ┌──────────────────────┴────────────────────────┐ │ while(running): │ │ kf = backend_queue.pop() │ │ local_BA(last_10_keyframes) // 50ms │ │ loop = check_loop_closure(kf) // 20ms │ │ if loop: │ │ pose_graph_optimize() // 200ms │ │ notify_frontend(corrected_poses) │ └───────────────────────────────────────────────┘ ═══════════════════════════════════════════════════════════════ PROCESS: PERCEPTION ═══════════════════════════════════════════════════════════════ CPU Core 1: CAMERA CAPTURE (pinned, SCHED_FIFO priority 90) ┌───────────────────────────────────────────────┐ │ frame = v4l2_dequeue_buffer() // ~1ms │ │ queue_to_inference.push(move(frame)) │ └──────────────────────┬────────────────────────┘ │ SPSC Queue CPU Core 2: GPU INFERENCE (pinned, SCHED_FIFO priority 80) ┌──────────────────────┴────────────────────────┐ │ frame = queue_from_camera.pop() │ │ detections = tensorrt_infer(frame) // ~15ms │ │ queue_to_tracker.push(move(detections)) │ └──────────────────────┬────────────────────────┘ │ SPSC Queue CPU Core 3: TRACKER + FUSION (pinned, SCHED_FIFO priority 70) ┌──────────────────────┴────────────────────────┐ │ dets = queue_from_inference.pop() │ │ lidar = lidar_queue.try_pop() │ │ radar = radar_queue.try_pop() │ │ fused = sensor_fusion(dets, lidar, radar) │ │ tracked = tracker.update(fused) // ~5ms │ │ world_model.update(tracked) │ └───────────────────────────────────────────────┘ CPU Core 3: SENSOR READERS (shared core, lower priority) ┌───────────────────────────────────────────────┐ │ Thread A: LiDAR reader (SCHED_FIFO 75) │ │ Thread B: Radar reader (SCHED_FIFO 75) │ │ Thread C: MAVLink router (SCHED_RR 60) │ └───────────────────────────────────────────────┘ KEY RULES: ✓ SLAM and Perception on SEPARATE cores — no interference ✓ SLAM backend is lower priority — won't starve frontend ✓ Pre-allocate ALL memory at startup ✓ Lock-free SPSC queues between pipeline stages ✓ CPU affinity pinning — no context-switch jitter

# 12. Software Deployment & OTA Updates

Figure 14: OTA Update Flow with SLAM Map Distribution

OTA UPDATE FLOW (with SLAM map distribution) ┌──────────────┐ ┌──────────────┐ ┌──────────────┐ │ Developer │ │ CI/CD │ │ Cloud OTA │ │ pushes code │ ───▶ │ Pipeline │ ───▶ │ Service │ └──────────────┘ │ │ │ │ │ • Build │ │ • Stage │ │ • SITL test │ │ • Sign │ │ • SLAM tests │ │ • Distribute │ │ • Package │ │ • Map deploy │ └──────────────┘ └──────┬───────┘ │ MQTT: ota/manifest │ ┌────────────────────────┘ ▼ ┌─────────────────────────────────────────────────────────────┐ │ DRONE (on ground, idle) │ │ │ │ OTA Agent: │ │ 1. Receive manifest (version, hash, size, signature) │ │ 2. Download to INACTIVE partition (A/B scheme) │ │ 3. Verify ECDSA signature + SHA-256 hash │ │ 4. Set bootloader flag → reboot │ │ 5. Self-test → mark good OR auto-revert │ │ │ │ SLAM Map Updates (separate from firmware): │ │ • Download pre-built maps for next mission area │ │ • Upload newly built maps after mapping flights │ │ • Maps stored on persistent /data partition │ │ • Maps versioned + checksummed │ │ │ │ DISK LAYOUT: │ │ ┌──────────┬──────────┬──────────────────────────┐ │ │ │ Part. A │ Part. B │ /data (persistent) │ │ │ │ (ACTIVE) │ (UPDATE) │ • SLAM maps │ │ │ │ │ │ • Config │ │ │ │ │ │ • Flight logs │ │ │ │ │ │ • ML models │ │ │ └──────────┴──────────┴──────────────────────────┘ │ └─────────────────────────────────────────────────────────────┘

## Fleet Rollout Strategy

Stage 1: Deploy to 1 test drone (internal). Fly full test mission. Verify.

Stage 2: Deploy to 5% of fleet (canary). Monitor telemetry for 24 hours.

Stage 3: Deploy to 25% of fleet. Monitor for 48 hours.

Stage 4: Deploy to 100%. Full rollout.

At ANY stage: if anomalies detected → halt rollout, auto-revert affected drones.

# 13. Technology Stack & Trade-offs

| Component | Recommended | Alternative | Rationale |
| --- | --- | --- | --- |
| Flight Controller | PX4 on NuttX | ArduPilot on ChibiOS | PX4 more modular, native SLAM pose input via EKF2 |
| Companion OS | Yocto Linux | Ubuntu/Buildroot | Yocto: reproducible, stripped down, long-term support |
| SLAM Algorithm | VINS-Fusion (start) Custom (long-term) | ORB-SLAM3 (GPL license risk) | VINS-Fusion: proven on drones, good VIO+SLAM. ORB-SLAM3 best features but GPL is a problem |
| SLAM Optimizer | GTSAM / g2o | Ceres Solver | GTSAM: iSAM2 incremental, good for real-time. g2o lighter weight, widely used |
| IPC Middleware | Custom lock-free + DDS | ROS2 / Apex.OS | Lighter than ROS2 for small drone |
| Inference Engine | TensorRT (NVIDIA) | ONNX Runtime | TensorRT for Jetson. ONNX if platform-agnostic |
| Video Pipeline | GStreamer | Custom V4L2+FFmpeg | GStreamer: plugin architecture, HW encoder support |
| Serialization | Protobuf / FlatBuffers | MessagePack | FlatBuffers for zero-copy. Protobuf for cloud APIs |
| Cloud Transport | MQTT v5 | gRPC | MQTT designed for unreliable links, tiny overhead |
| Cloud Backend | Kubernetes + Go/Rust | Serverless | K8s for control, TimescaleDB for telemetry |
| OTA System | RAUC / SWUpdate | Custom | Proven, A/B aware, signed updates |
| Build System | CMake + Yocto | Bazel | CMake is PX4 standard. Yocto for system image |
| CI/CD | GitLab CI + SITL | GitHub Actions | GitLab: self-hosted (defense requirement) |
| Language | C++17 (core) Python (tools) | Rust | C++17 for PX4 compat + mature SLAM libraries |

⚡ Interview Tip: In the interview, don't just name technologies — explain WHY. For SLAM specifically, mention the GPL license risk of ORB-SLAM3 for a defense product. This shows commercial awareness.

# 14. Interview Whiteboard Walkthrough

When asked to "design the software architecture for an autonomous drone," follow this structure. Practice drawing this in under 15 minutes.

## Step 1: Clarify Requirements (2 min)

Ask the interviewer:

• "What level of autonomy? Fully autonomous, semi-autonomous, or remote piloted?"

• "GPS-denied capability required? Indoor operations?"

• "What are the key payload types — ISR, delivery, mapping?"

• "What scale — single drone or fleet of 100+?"

## Step 2: Draw Hardware Context (2 min)

Draw the physical drone with labeled components: FC, companion computer, stereo camera (for SLAM), mission camera, sensors, RF link, payload rail.

## Step 3: Two-Computer Architecture (3 min)

Draw the FC ↔ Companion split. Explain WHY: fault isolation, real-time vs Linux. Label MAVLink. Show SLAM pose flowing from companion → FC EKF via VISION_POSITION_ESTIMATE.

## Step 4: Companion Software Stack (3 min)

Draw the 7 processes: Perception, SLAM/VIO, Mission Planner, Video, Payload Manager, Comms, System Monitor. Explain multi-process for fault isolation. Show SLAM feeding into both the FC and the Mission Planner.

## Step 5: Navigation Fallback Chain (2 min)

Draw: GPS → SLAM → VIO → IMU → Land. Explain graceful degradation. Mention GPS spoofing detection via SLAM/GPS disagreement. This is a HIGH-IMPACT diagram for a defense drone company.

## Step 6: Payload Plugin Architecture (1 min)

Draw abstract PayloadInterface → concrete plugins. Mention dlopen, factory pattern.

## Step 7: Cloud & Comms (1 min)

Draw GCS + Cloud. MQTT for drone↔cloud. Mention offline-first, A/B OTA, SLAM map sharing.

## Step 8: Safety Story (1 min)

End with the failsafe cascade. Strong final impression — you think about failure modes.

— Practice this walkthrough 3 times before the interview. —

You should be able to explain and draw the full architecture in 15 minutes.
The SLAM section + navigation fallback chain will set you apart.

15. Clarification Questions — Study Session Q&A

This section captures all the follow-up questions asked during the study session, with detailed answers. These represent likely interview follow-up questions — understanding these demonstrates depth beyond surface-level architecture knowledge.

Q1: What is MAVLink v2?

MAVLink v2 is the communication protocol that drones use to talk between their flight controller and everything else — the companion computer, the ground station, and cloud systems. It is a lightweight binary messaging format designed specifically for drones and robots operating over slow, unreliable links.

Key characteristics:

• ~300+ predefined message types (GPS position, attitude, battery status, motor commands, etc.)

• Each message is a small binary packet (typically 8–280 bytes) with a header, payload, and checksum

• Runs over raw UART serial, UDP, or radio modems — not TCP/IP

MAVLink v1 vs v2

| Feature | MAVLink v1 | MAVLink v2 |
| --- | --- | --- |
| Header size | 6 bytes | 10 bytes |
| Message ID | 8-bit (256 messages max) | 24-bit (16M messages) |
| Signing | None | HMAC-SHA256 (anti-spoofing) |
| Packet trimming | No | Yes — trailing zeros removed |
| Extensible messages | No | Yes — add fields without breaking old receivers |

The message signing in v2 is critical for defense — it prevents an adversary from injecting fake MAVLink commands into the drone's data link.

Key messages used in the architecture:

• VISION_POSITION_ESTIMATE (msg ID 102): SLAM pose from companion → FC EKF at 30Hz

• HEARTBEAT: system alive status at 1Hz

• COMMAND_LONG: high-level commands (go to waypoint, arm, takeoff)

• ATTITUDE: current orientation at 50Hz

Why not TCP/JSON or Protobuf? UART serial links have no concept of TCP. JSON would be 10-50x larger per message — unacceptable at 50Hz over a 115200 baud serial link. MAVLink is the PX4/ArduPilot standard. It's designed for lossy links: no connection state, each packet is self-contained.

Q2: Isn't the Unscented Kalman Filter (UKF) better than EKF in the architecture?

The UKF is theoretically more accurate than the EKF. But in practice, the EKF wins for drone flight controllers.

Core Difference

EKF: Linearizes the nonlinear system using Jacobians (first-order Taylor expansion). Fast — compute a Jacobian matrix and do standard Kalman math.

UKF: No Jacobians — picks 2n+1 sigma points (where n = state dimension), passes each through the actual nonlinear function, recovers mean and covariance from transformed points. Captures second-order nonlinear effects that EKF misses.

Why EKF Wins on a Flight Controller

| Factor | EKF | UKF |
| --- | --- | --- |
| Accuracy | Good (1st order) | Better (2nd order) |
| Compute cost | O(n²) per update | O(n³) — propagate 2n+1 sigma points |
| PX4 EKF2 (n=24) | 24 states | 49 sigma points through full dynamics |
| RAM usage | ~2-3 KB | ~10-15 KB |
| Runs on STM32H7? | Yes, comfortably at 200Hz | Barely — most of CPU budget |
| PX4/ArduPilot support | Native (EKF2/EKF3) | Not implemented |

The EKF's linearization error is significant only when the system is highly nonlinear (aggressive aerobatic maneuvers, >60° roll) or the time step is large. For a drone in normal flight with 5ms update intervals, attitude changes between updates are tiny (~0.5°) — EKF linearization error is negligible.

Where UKF COULD be used: on the companion computer (plenty of compute) for SLAM's IMU preintegration, object tracking with nonlinear motion models, or sensor fusion in the perception pipeline. The architecture could use both: EKF on the FC, UKF on the companion for perception tasks.

Interview Answer: "The UKF captures second-order nonlinear effects better, so it's theoretically more accurate. But on an STM32 at 200Hz with 24 states, the UKF's compute cost would consume most of the CPU budget. The EKF's linearization error is negligible at 5ms timesteps. Both PX4 and ArduPilot use EKF — it's battle-tested across millions of flights. I'd only consider UKF on the companion computer for perception tasks where we have more compute."

Q3: Do drones use LiDARs or cameras for SLAM?

Both — but for small tactical drones like the Q-X, cameras dominate due to weight, power, and cost constraints.

Weight/Power/Cost Comparison

| Factor | Stereo Camera | LiDAR (Livox Mid-360) |
| --- | --- | --- |
| Weight | ~50g | ~265g |
| Power | ~1.5W | ~9W |
| Cost | ~$200 | ~$1,500 |
| Range | ~10m (depth), 100m+ (features) | ~200m |
| Output | Dense pixels, texture, color | Sparse 3D points, no texture |

What Platforms Use What

| Platform | SLAM Sensor | Why |
| --- | --- | --- |
| Small drones (<5kg) | Visual SLAM (cameras) | Weight/power constraints |
| Medium drones (5-25kg) | Camera primary + optional LiDAR | LiDAR for mapping missions |
| Large drones (>25kg) | LiDAR SLAM (often primary) | No weight constraints |
| Ground robots | LiDAR SLAM (dominant) | No weight constraints, structured environments |
| Self-driving cars | LiDAR + Camera + Radar fusion | No constraints, maximum accuracy |

For 24 Industries Q-X: Visual-Inertial SLAM as baseline (stereo camera + IMU, ~50g, 1.5W). If a LiDAR payload is attached via the modular rail system for a mapping mission, fuse it in for better accuracy. LiDAR is additive, not required. The architecture supports both.

Interview Answer: "We use visual-inertial SLAM as the baseline because the stereo camera is always on the drone at minimal weight/power cost. For mapping-specific missions, we attach a LiDAR payload and fuse it into the SLAM pipeline. The architecture supports both — LiDAR is additive, not required."

Q4: Why not use the stereo camera as the mission camera itself?

The fundamental incompatibility is the GIMBAL. For ISR (Intelligence, Surveillance, Reconnaissance) missions, the operator needs to look in a different direction than the drone is flying. A gimballed camera pans/tilts independently — but SLAM cannot tolerate arbitrary gimbal movements because all tracked features would disappear in one frame.

SLAM Needs vs Mission Needs

| Requirement | SLAM Camera | Mission Camera |
| --- | --- | --- |
| Field of view | Wide (90-120°) | Narrow (30-60°) for detail |
| Resolution | Low OK (640×480) | High (4K/12MP) |
| Frame rate | Consistent 30fps, never skip | Variable (60fps burst, photos) |
| Orientation | Fixed, forward-facing | Gimballed — operator pans/tilts |
| Lens | Global shutter (no motion blur) | Rolling shutter OK |
| Exposure | Optimized for feature tracking | Optimized for image quality / HDR |

If the operator pans the gimbal 90° to look at a target, every tracked feature disappears in one frame. SLAM loses tracking instantly.

Weight impact: Stereo camera (~50g) + mission camera with gimbal (~110g) = 160g total. On a 2.5kg payload budget, that's 6.4%. Not worth the capability loss of a single camera.

When One Camera DOES Work

For drones that don't need a gimbal (autonomous delivery, racing, mapping with downward camera), a single fixed stereo camera with downsampling for SLAM is the lighter, simpler, cheaper design. The sensor architecture is driven by the mission profile, not the algorithm.

Q5: Would fixing the stereo camera and downsampling the image for SLAM work?

Yes — this approach is technically feasible and some drones do exactly this. The GPU/ISP can downsample in <1ms. SLAM gets 640×480, perception gets full 4K. One capture pipeline, two consumers.

What Works

• Downsampling quality: SLAM features (ORB, FAST corners) survive downsampling well

• Processing cost: GPU resize is nearly free

• Synchronization: Better than two cameras — same timestamp, no sync issues

• Weight/power: Win — one sensor instead of two

What's Still a Problem

1. THE GIMBAL: For ISR missions, the operator needs to look in a different direction than the drone flies. Fixed camera = must turn the entire drone. Unacceptable for surveillance.

2. EXPOSURE CONFLICT: SLAM wants short, consistent exposure for features. Mission wants HDR/longer exposure for target visibility. Manageable compromise but noticeable quality loss.

3. FOV TRADE-OFF: Wide FOV (good for SLAM, targets are tiny) vs narrow FOV (good for detection range, fewer SLAM features). A middle ground (~70-80°) is okay for both but optimal for neither.

Interview Answer: "If the mission profile allows a fixed camera — like delivery or mapping — I'd absolutely use one high-res stereo camera and downsample for SLAM. Simpler, lighter, fewer failure points. But for ISR missions where the operator needs to independently pan and tilt to track targets, we need a separate gimballed mission camera. The sensor architecture is driven by the mission profile."

Q6: What are ISR missions?

ISR = Intelligence, Surveillance, Reconnaissance. The military term for "fly over an area and gather information."

| Component | Meaning | Drone Example |
| --- | --- | --- |
| Reconnaissance | "What's out there?" — broad area survey | Fly over 5km², photograph everything, build a map |
| Surveillance | "Watch this specific thing" — persistent monitoring | Orbit a building for 20 min, stream live video |
| Intelligence | Analyze collected data for actionable conclusions | "3 vehicles arrived at 14:00, moved to building B" |

The drone does the R and S. Humans (or AI) do the I. ISR demands a gimbal because the drone flies straight/orbits for efficiency, but the operator needs the camera pointing at the target — which is often sideways or below, not forward. For the Q-X at 24 Industries, ISR is the primary mission — it's a tactical surveillance drone.

Q7: Should the companion computer processes be containerized with Docker?

No — not Docker containers on a small drone. Docker solves a problem drones don't have: "I need to deploy the same software on 1000 different server configurations." On a drone, you control the exact hardware and OS image.

Why NOT Docker on a Drone

| Factor | Docker Impact | Drone Constraint |
| --- | --- | --- |
| Startup time | 1-5s per container | All systems must run within ~2s of boot |
| Memory overhead | ~50-100MB per container | Jetson has 4-8GB total |
| Networking | Virtual bridges, NAT | +100µs per IPC call — breaks 30Hz SLAM |
| GPU access | nvidia-container-runtime | Extra failure mode for TensorRT |
| Real-time | cgroup interaction tricky | Risk of priority inversion |

What to Use Instead: systemd + cgroups

You get all Docker's isolation using native Linux features — Docker is just a wrapper around these:

• Separate processes: systemd starts each subsystem as an independent service with auto-restart

• cgroups: CPU affinity pinning, memory limits, CPU quotas — no Docker needed

• Namespaces: filesystem isolation (ProtectSystem=strict, PrivateTmp=true)

• Capabilities: security restrictions (CapabilityBoundingSet, NoNewPrivileges)

• Watchdog: WatchdogSec=5 — killed if no heartbeat for 5 seconds

Example systemd service with resource isolation:

# /etc/systemd/system/slam.service

[Service]

ExecStart=/opt/drone/bin/slam

CPUAffinity=0 1            # Pin to cores 0-1

MemoryMax=1536M             # Hard memory limit

CPUQuota=200%               # Max 2 cores

Restart=always              # Auto-restart on crash

RestartSec=100ms            # Restart within 100ms

WatchdogSec=5               # Killed if no heartbeat

ProtectSystem=strict        # Read-only root filesystem

Interview Answer: "On the drone, we don't containerize — Docker adds memory overhead, startup latency, and complicates GPU access and shared-memory IPC. Instead we use native systemd services with cgroup resource limits, CPU affinity pinning, and watchdog timers. Same isolation, zero overhead. Docker is great for our cloud backend and CI/CD — just not on a 15-watt embedded computer."

Q8: Why does Docker make sense for the cloud backend and CI/CD pipeline?

Because the cloud and CI/CD have the exact problems Docker was invented to solve — problems that don't exist on the drone.

Cloud Backend — The Problem Docker Solves

Multiple microservices in different languages (Go, Python, Rust) with different dependencies, different release schedules, different scaling needs. Docker lets you deploy/update one service without touching the others. Kubernetes auto-scales: more load → more container replicas, no code changes.

CI/CD — Reproducible Test Environments

Testing drone software requires PX4 SITL, Gazebo simulator, ROS2, SLAM libraries, MAVSDK — all at specific versions. Without Docker: 2-4 hours to set up a CI runner, breaks every time Ubuntu updates a package. With Docker: docker-compose up (30 seconds, images cached), 100% reproducible.

Why Docker's Overhead Doesn't Matter Here

| Factor | Drone | Cloud |
| --- | --- | --- |
| RAM | 4-8 GB total | 64-256 GB per server |
| Startup time | Must boot in <2s | 3s startup? Fine |
| CPU overhead | 15W, ~4 cores, real-time | 64 cores, no RT requirement |
| Networking | Shared memory <1µs needed | Already 1-10ms over network |
| Deployment target | One specific hardware | 5 devs, 3 staging, 20 prod nodes |
| Scaling | One computer, one instance | Scale to 10 replicas for 500 drones |

Q9: What does the SLAM frontend and backend do exactly?

Frontend — "Where am I RIGHT NOW?" (real-time, every frame)

The frontend estimates how the drone moved since the last frame. Runs at 30Hz, must finish in ~10ms.

Step 1: Feature Extraction (~3ms)

Find distinctive points (corners, edges) in the image — ~1000 features per frame. Uses ORB (Oriented FAST and Rotated BRIEF): FAST finds corners quickly, BRIEF creates a 256-bit binary descriptor (compact fingerprint), ORB adds rotation invariance.

Step 2: Feature Matching / Tracking (~4ms)

Find the same features in the new frame. Either optical flow (Lucas-Kanade: "where did each point move?") or descriptor matching (compare binary fingerprints, Hamming distance). RANSAC rejects outliers: randomly pick matches, compute motion model, keep the model with most agreement. Result: ~800 good matches out of 1000 features.

Step 3: IMU Preintegration (~1ms)

Between camera frames (33ms gap), the IMU measured acceleration and rotation at 200-1000Hz. Integrate these measurements to get a motion prior (expected rotation/velocity/position change). IMU bridges gaps during fast motion; camera corrects IMU drift. Together they're robust.

Step 4: Pose Estimation (~2ms)

Combine visual matches with IMU prior: PnP (Perspective-n-Point) solves for camera pose that minimizes reprojection error. Output: 6DOF pose (x, y, z, roll, pitch, yaw) sent to FC via MAVLink at 30Hz.

Backend — "Make the map globally consistent" (async, periodic)

The backend does expensive optimization that doesn't need to happen every frame. Runs in a separate thread at lower priority, processes KEYFRAMES only (1 every 5-10 frames, ~3-6/sec).

Step 5: Local Bundle Adjustment (~50ms)

Take the last N keyframes and all shared 3D points, optimize ALL positions jointly. Minimizes total reprojection error. For N=10 keyframes, ~1560 variables. Solver: Levenberg-Marquardt via g2o or GTSAM. Result: more accurate poses and map points.

Step 6: Loop Closure Detection (~20ms)

"Have I been here before?" Each keyframe gets a visual fingerprint (Bag of Words via DBoW2). Compare current fingerprint against all previous keyframes. High similarity → candidate. Then geometrically verify (feature matching + PnP, require 50+ inliers) to reject false positives.

Step 7: Pose Graph Optimization (~100-200ms)

Triggered by confirmed loop closure. Adds a constraint (these two keyframes are the same place) and optimizes ALL past poses to satisfy it. Distributes the accumulated drift error evenly across all keyframes. Entire trajectory becomes globally consistent.

Step 8: Map Management (ongoing)

Stores keyframes + 3D points (sparse map). Keyframe culling removes redundant keyframes to bound memory. Map serialization enables save/load for revisiting known environments. Map upload to cloud for fleet sharing.

Why the Split Matters

The frontend gives real-time pose estimates every frame — if it's slow, frames drop and tracking degrades (CRITICAL). The backend improves accuracy but is not time-critical — if it's slow, map accuracy degrades slowly but the drone still navigates (ACCEPTABLE). This split means no jitter, no dropped frames during heavy optimization.

Q10: What does "bursty" mean?

Bursty means the workload isn't constant — it spikes unpredictably, then goes quiet. The SLAM backend is bursty: most of the time it does small local bundle adjustments (~50ms, ~30% CPU). But when a loop closure is detected, it must re-optimize the entire pose graph — spiking to 100% CPU for 100-200ms, then dropping back down.

This matters for system design: you can't assume SLAM backend always uses 30% CPU. If you don't isolate it (separate process, cgroup limits), the burst can starve the perception pipeline or video encoder of CPU time. This is why SLAM runs as a separate process with resource limits.

Q11: Which SLAM algorithm is the best?

There's no single best — it depends on compute budget, sensor setup, and mission requirements. Here's the ranking for a small tactical drone:

| Algorithm | Type | Pros | Cons | License |
| --- | --- | --- | --- | --- |
| ORB-SLAM3 | Visual-Inertial SLAM (sparse) | Most complete. Mono/stereo/ RGB-D+IMU. Map reuse. Best accuracy. | CPU-heavy backend. Struggles low texture. | GPL v3 |
| VINS-Fusion | Visual-Inertial SLAM | Battle-tested on drones. Excellent VIO accuracy. Loop closure. | Less robust map reuse than ORB-SLAM3. | GPL v3 |
| OpenVINS | Visual-Inertial Odometry only | Most robust VIO. Very modular. Lowest compute. | No loop closure. No map. Drift accumulates. | GPL v3 |
| Kimera | Metric-Semantic SLAM | Dense mesh output. Semantic labels. | Higher compute. Less production-ready. | BSD |

The License Problem

ALL top SLAM systems are GPL v3: if you use the code in your product, you must release your entire product's source code under GPL. For a defense drone company: absolutely not.

Options:

1. Use GPL for research/prototyping only, then rewrite for production (clean-room)

2. License commercially (some authors offer this)

3. Build custom SLAM from BSD-licensed components (OpenCV, GTSAM, DBoW2, Eigen)

4. Buy a commercial SLAM SDK (SLAMcore, Kudan)

Recommended Approach for 24 Industries

Phase 1 (0-6 months): Prototype with VINS-Fusion. Prove the concept. Research only — never ships.

Phase 2 (6-18 months): Build custom SLAM from BSD components (OpenCV ORB, GTSAM iSAM2, DBoW2). Own the IP.

Phase 3 (18+ months): Add differentiating features (deep learning loop closure, LiDAR fusion, collaborative SLAM).

Interview Answer: "I'd prototype with VINS-Fusion — battle-tested on drones. But for production, we build custom from BSD components because all top open-source SLAM is GPL, incompatible with commercial defense. The architecture should be modular — swap SLAM implementations without changing FC or planner interfaces."

Q12: What is loop closure?

Loop closure is when the drone recognizes it's returned to a place it's been before — and uses that recognition to fix all the navigation drift that accumulated along the way.

The Problem

Every SLAM system drifts. Each frame-to-frame estimate has ~0.1% error. Over a 400m square path: 0.5% × 400m = 2m error at the end. The drone thinks it's 2m from where it actually is. After a 5km mission: 25m error.

How It Works

Step 1 — Recognition: Each keyframe is compressed into a visual fingerprint using Bag of Words (DBoW2). Compare current keyframe's fingerprint against ALL stored keyframes. High similarity score → candidate.

Step 2 — Verification: Match individual features between the two keyframes, compute relative pose via PnP + RANSAC. Require 50+ geometric inliers. This rejects false positives (perceptual aliasing — two different places that look similar).

Step 3 — Correction: Add a constraint to the pose graph ("these two keyframes are the same place"). The optimizer (g2o/GTSAM) distributes the error evenly across all intermediate keyframes. Every pose gets a small nudge — no single pose jumps discontinuously. The entire trajectory becomes globally consistent.

Impact

• Without loop closure (VIO): 0.5-2% drift. 5km flight → 25-100m error.

• With loop closure (SLAM): error bounded to <1m regardless of flight duration.

• Return-to-launch accuracy: from "25m away" to "within 1m of the pad."

• This is the difference between "close enough" and "crashes into the building next door."

Q13: What is REST/WS for the fleet management dashboard?

REST = RESTful HTTP API, WS = WebSocket. Two web communication patterns for the browser-based fleet management UI.

REST — Request/Response (Pull-Based)

"I ask, you answer." Standard HTTP. Dashboard sends a request, backend responds. Each request is independent, no persistent connection. Good for: actions (start mission, upload map, create user), fetching data on demand, CRUD operations.

WebSocket — Persistent Connection (Push-Based)

"Keep the line open, push me updates." Connection stays open for hours. Backend pushes data the instant it arrives — no polling, no wasted requests. Good for: live telemetry, real-time drone positions on map, alerts, SLAM status changes.

Why Both?

REST-only (polling): 30 drones × 1 req/s = 30 req/s of mostly useless traffic, up to 1 second delay. WebSocket-only: no HTTP semantics (status codes, caching), harder to debug. Both together: REST for actions/data fetch, WebSocket for live streaming. Standard web pattern.

Compared to Other Links

• MQTT: drone ↔ cloud (unreliable RF link, tiny packets, pub/sub)

• REST/WS: dashboard ↔ cloud (browser on fast network, JSON, HTTP semantics)

Different tools for different problems. MQTT handles the hard part (radio link). REST/WS handles the easy part (browser to server).

Q14: What kind of analytics would be in the fleet management dashboard?

Operational Analytics (Real-Time & Post-Mission)

• Mission success rate (completed vs aborted vs failed — by drone, area, time)

• Mission duration vs planned (estimation accuracy)

• Abort reasons breakdown (battery, weather, comms loss, SLAM lost, operator)

• Area coverage — how much of assigned zone was surveyed

• Total fleet flight hours (daily/weekly/monthly)

• Flight hours per drone (identify overused/underused airframes)

Maintenance & Health Analytics (Predictive)

• Battery capacity degradation over charge cycles (predict replacement date)

• Motor current draw trends (increasing = bearing wear → predict failure)

• Vibration levels from IMU (propeller imbalance, loose mount, frame crack)

• SLAM drift rate per mission (increasing → camera calibration drifting?)

• Loop closure success rate (declining → vocabulary needs retraining?)

• Tracking loss events — where and why

Strategic Analytics (Long-Term)

• Software version performance comparison (v2.3.1 vs v2.2.0 detection accuracy, drift, latency)

• Environmental insights (which areas cause SLAM failures, RF dead zones)

• Fleet utilization — drone availability rate, peak demand times

• Cost per flight hour, cost per km² surveyed

• MTBF (Mean Time Between Failures) per drone model/batch

Build Priority

Phase 1 (MVP, 5 drones): Live positions on map, battery levels, mission status, flight hours

Phase 2 (10+ drones): Health tracking, SLAM metrics, OTA tracking, per-drone reports

Phase 3 (50+ drones, months of data): Predictive maintenance (ML), A/B software comparison, cost modeling

Interview Answer: "Analytics starts simple — live positions, battery, mission status. As the fleet scales to 30+, we add component health tracking and predictive maintenance. The key is building the telemetry ingestion pipeline correctly from day one so the data is there when we need it."

Q15: Isn't telemetry and fleet data sensitive in a defense context?

Absolutely — ALL drone data in a defense context is sensitive, not just commands. Even "harmless" telemetry reveals: WHERE drones operate (areas of interest), HOW MANY drones you have (fleet size), WHEN you operate (patterns), FLIGHT PATTERNS (coverage gaps), BATTERY LEVELS (windows of no coverage), SLAM MAPS (environment layout), MISSION PLANS (what you're looking for). Even metadata is dangerous: "a drone flew from base A to location B at 02:00" tells the enemy something is happening.

Two Separate Communication Paths

| Path | Protocols | Encryption | Goes Through RF? |
| --- | --- | --- | --- |
| Drone ↔ GCS | MAVLink, RTP video | AES-256 (RF hardware) + MAVLink HMAC signing | YES — direct radio |
| Drone ↔ Cloud | MQTT | TLS 1.3 + mTLS certificates | NO — LTE/WiFi |
| Cloud ↔ Dashboard | REST, WebSocket | HTTPS / WSS (TLS 1.3) | NO — internet |

Why MAVLink Gets Signed Even Though RF Is Encrypted

Defense-in-depth. If an enemy captures one of your radios, they have the RF encryption keys. Without MAVLink signing: they can inject fake commands ("disarm motors mid-flight"). With signing: FC checks HMAC → INVALID → message rejected. The signing key is in the FC's secure element, not the radio. Two independent layers — even if one is compromised.

GPS Spoofing Detection

Running SLAM alongside GPS enables spoofing detection. If an adversary sends fake GPS signals, SLAM (which uses cameras) will disagree with GPS. Divergence beyond threshold → flag spoofing attack → switch to SLAM-only navigation.

Cloud Backend Deployment — Not Public Cloud

Option A: On-premises (customer's data center, air-gapped) — most secure

Option B: Government cloud (AWS GovCloud / Azure Government, FedRAMP certified)

Option C: Tactical edge server (rugged mini-server in a Pelican case, deploys with the unit, no internet needed)

Data at Rest & Lifecycle

• Drone storage: encrypted (dm-crypt/LUKS), keys in hardware secure element (TPM)

• After mission: offload data to secure server, verify, then WIPE drone storage

• If drone captured/lost: auto-wipe after comms loss timeout, secure element self-destructs

• Secure boot chain: signed bootloader → signed kernel → signed application

Interview Answer: "In a defense context, telemetry is just as sensitive as commands. The architecture needs encryption at every layer: AES-256 on RF, MAVLink signing for authenticity, TLS with mTLS for cloud, encrypted storage with remote wipe. The backend runs on-premises or government cloud. I'd add a data lifecycle policy: auto-wipe from drone after offload, capture response with secure erase on comms loss."

Q16: The perception timing budget adds up to more than 27ms — how does it work?

The table was misleading because it lists stages vertically, implying sequential execution. In reality, stages run in PARALLEL across CPU cores and are PIPELINED across frames.

Parallel Execution Across Cores

Parallel Execution — 4 Cores

Time (ms): 0 2 5 17 19 22 24 33 │ │ │ │ │ │ │ │ Core 0: ████████████░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░ (SLAM) │ Frontend │ Backend (async, won't block perception) │ │ 10ms │ │ │ │ │ │ │ │ │ │ Core 1: █████░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░░ (Capture)│cam│ idle — waiting for next frame │ │2ms│ │ │ │ │ │ │ │ │ │ │ │ │ │ Core 2: ░░░░░████████████████████████████████░░░░░░░░░░░░░░░░░░░░ (GPU) wait │◄── Object Detection (15ms) ──►│ idle │ │ │ │ │ │ │ │ Core 3: ░░░░░██████░░░░░░░░░░░░░░░░░░░░░░░░████─████─████░░░░░░░ (Fusion) wait │LiDAR│ wait for detect │Fus│Trck│Avd│ SPARE │ │Radar│ (data ready in queue) │2ms│ 3ms│2ms│ 9ms │ │ 3ms │ │ │ │ │ │ 0 2 5 17 19 22 24 33 CRITICAL PATH: Capture(2) + Detection(15) + Fusion(2) + Tracking(3) + Avoid(2) = 24ms LiDAR: runs PARALLEL with detection — finishes 12ms early, NOT on critical path SLAM: SEPARATE process, separate core, completely independent Headroom: 33ms - 24ms = 9ms spare (27%)

The actual critical path is 24ms, not 27ms. LiDAR preprocessing runs in parallel with object detection — by the time detection finishes (15ms), LiDAR data has been ready for 12ms, sitting in a queue. SLAM runs on its own process/core and is entirely independent.